3GPP TSG-SA WG1 Meeting #92e	S1-204038

Electronic Meeting, 11-20 November 2020	(revision of S1-20xxxx)

Title:	FS_PIN: New use case: UE accessing PIN applications hosted by 5G enabled gateways

Agenda Item:	7.12.1

Source:	InterDigital

Contact:	Atle Monrad <Atle.Monrad@InterDigital.com>

Abstract

This document proposes a new use case for FS_PIN, where Service Hosting Environments can be hosted by 5G enabled gateways in PINs.

Proposal

It is proposed to include the below new use case for UE accessing PIN applications hosted by 5G enabled gateways in 3GPP TR 22.859 version 0.1.0.

* * * *   First change   * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 22.101: "Service principles".

[4]	Christiane Attig, Nadine Rauh, Thomas Franke, & Josef F. Krems (May 2017) "System Latency Guidelines Then and Now – is Zero Latency Really Considered Necessary?
https://www.researchgate.net/publication/317801643_System_Latency_Guidelines_Then_and_Now_-_Is_Zero_Latency_Really_Considered_Necessary/link/5ae18fc4458515c60f662370/download

[5]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS); Stage 1".

[6]	3GPP TR 22.858: " Study of Enhancements for Residential 5G; Stage 1".

* * * *   Second change   * * * *

## 5.x	UE Accessing PIN Applications Hosted by 5G Enabled Gateways

### 5.x.1	Description

PIN devices (e.g. media server, printer, smart thermostat/sprinkler/blinds, smart lightning system, NAS server, etc.) are located behind a residential gateway. This gateway can host, or directly be connected to, significant compute and storage resources. These resources can therefore be virtual on the gateway, be locally connected to the gateway, or a mix of both.

On-site computing can be useful to process sensitive data locally. Data generated by some PIN devices can be sensitive, it is therefore important to limit the scope of its dissemination (e.g. temperature and light readings can be used to know if there is someone at home).On-site computing can also be useful to host latency sensitive processing. Games and office applications are examples of applications where it can be beneficial to render a latency sensitive application locally.

Today Service Hosting Environments can be located in core networks and base station sites, while in the context of a PIN such an environment could be extended over compute and storage resources provided by a homeowner, or by the gateway itself. Whether the compute/storage resources are located on the gateway, PIN devices or a mix of those, the Service Hosting Environment deployed over those resources is referred to as gateway-hosted (or 5G RG-hosted) Service Hosting Environment.

Figure 5.x.1-1. Service Hosting Environment hosted by 5G enabled gateways

Case (a): the user/UE is out of home and uses a client application that connects to an on-site application server deployed in a Service Hosting Environment at home (on the gateway or on other hardware at home).

Case (b): the user/UE is at home and uses a client application that connects to an on-site application server deployed in a Service Hosting Environment at home.

Service aspects:

- Deployment of a gateway-hosted Service Hosting Environment and hosted on-site applications by service provider.

- Access to on-site application server in gateway-hosted Service Hosting Environment by UE outside of home.

- Additional aspects are already covered in 3GPP TR 22.858 [6], sections 5.4 and 5.5 when the on-site application is on a server (no specific new routing support is needed when the on-site application is on the 5G RG):

- Access to on-site application server in gateway-hosted Service Hosting Environment by UE or PIN device from inside home.

- Access to services offered by PIN devices by server applications in gateway-hosted Service Hosting Environment.

### 5.x.2	Pre-conditions

User deploys PIN devices behind a 5G enabled gateway and subscribes with a service provider for on-site application services. This subscription can be directly with the service provider(s), or through a 5G network operator.

### 5.x.3	Service Flows

1)	Setup:

a)	User requests some compute/storage resources (in gateway or PIN servers) to be reserved for service provider’s Service Hosting Environment.

b)	Software is deployed on Service Hosting Environment hosted by gateway (by service provider, 5G network operator or User).

2)	Using an on-site application server application:

a)	User installs client applications on UE, e.g. a client application that configures temperature at home based on time or day or other triggers.

b)	Service provider provisions corresponding application server on gateway-hosted Service Hosting Environment.

c)	Client application on UE connects through 5G network to application server on gateway-hosted Service Hosting Environment. User configures application. The application server can communicate with PIN devices, e.g. to collect and process data locally.

### 5.x.4	Post-conditions

The user can install and use a different UE application supported by the service provider. The service provider deploys corresponding application servers in the gateway-hosted Service Hosting Environment. Suitable applications include all kind of applications acting on privacy sensitive data, and, when the client UE is at home or close to home, latency sensitive rendering or interactive application components for games or office applications.

### 5.x.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] clause 6.5.2: The 5G network shall enable a Service Hosting Environment provided by operator.

3GPP TS 22.261 [2] clause 6.5.2: Based on operator policy, the 5G network shall be able to support routing of data traffic between a UE attached to the network and an application in a Service Hosting Environment for specific services, modifying the path as needed when the UE moves during an active communication.

3GPP TS 22.261 [2] clause 6.7.2: The 5G system shall be able to support QoS for applications in a Service Hosting Environment.

3GPP TS 22.261 [2] clause 8.2: The 5G system shall support a secure mechanism to access a service or an application in an operator's Service Hosting Environment.

Additionally, the following potential requirements defined in 3GPP TR 22.858 [6] partly cover the use case functionality:

[PR. 5.4.6-001] The 5G system shall be able to support efficient routing, without going through the 5GC, for the communication between a UE and a non-3GPP device, via the residential gateway and an indoor small base station connected to that residential gateway.

[PR. 5.5.6-001] The 5G system shall be able to provide E2E QoS control for the communication path between the UE and a residential gateway via an indoor small base station.

[PR. 5.5.6-002] The 5G system shall be able to support efficient routing for communication between UEs via a residential gateway without going through the 5GC.

[PR. 5.5.6-003] The 5G system shall ensure the use of a residential gateway does not compromise the security of any PLMN or broadband access network.

[PR. 5.5.6-004] The 5G system shall ensure the use of a residential gateway does not compromise the security of the UE.

[PR. 5.5.6-005] The 5G system shall enable the network operator associated with a residential gateway to control the security policy of the residential gateway.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 5G network shall enable a 5G RG-hosted Service Hosting Environment provided by operator.

[PR 5.x.6-2] Based on operator policy, the 5G network shall be able to support routing of data traffic between a UE attached to the network and an application in a 5G RG-hosted Service Hosting Environment for specific services.

[PR 5.x.6-3] The 5G system shall be able to support QoS for applications in a 5G RG-hosted Service Hosting Environment.

[PR 5.x.6-4] The 5G system shall support a secure mechanism to access a service or an application in an operator's 5G RG-hosted Service Hosting Environment.

* * * *   End of changes   * * * *