3GPP TSG-SA1 Meeting #92e	S1-204041

Electronic Meeting, 10 September - 19 September 2020	(revision of S1-XXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.101 | CR | XXX | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage and Accounting Information requirements for Shared E-UTRAN/NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | xxx |  |  |  |  |  | Date: |  |  | 2020-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN sharing enhancements allow multiple Participating Operators to share the resources. Hence, a Hosting E-UTRAN/NG-RAN Operator shall be able to collect resource usage and accounting information separately for each Participating Operator. In addition, network management system information exchanged between Hosting Operator and Other Participating Operators should be credible (tamper-proof and traceable) to avoid artificially modification on collected information at NMS of operator. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add radio resource usage and accounting information for each Participating Operator in 28.2.3. To add requirements on credible network management system information exchanged between Hosting Operator and Other Participating Operators in 28.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Radio resource statistics for each Participating Operators in RAN sharing cannot be accurately reflect the actual radio usage status. Information exchanged between Hosting Operator and Other Participating Operators may be distorted without traceable records and cannot be credible for billing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 28.2.2, 28.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the Change---

28.2.2	OA&M Access to the Shared E-UTRAN/NG-RAN

Each Participating Operator can have their own OA&M capabilities, which are used for monitoring and for selected operations in a shared E-UTRAN/NG-RAN. Information exchange involved in those operations need to be controlled by the Hosting E-UTRAN/NG-RAN Operator as to prevent disclosing them to other Participating Operators, be it for business, operational, or technical reasons. Network management system information exchanged between Hosting Operator and Other Paticipating Operators should be credible, including tamper-proof and traceable, so that Hosting Operator network management system cannot distort the information.

The following requirements apply:

Selected OA&M capabilities for the Shared E-UTRAN/NG-RAN, under the control of the Hosting E- UTRAN Operator, shall be accessible by the Participating Operator's OA&M functions

This would allow, for example, the Participating Operator to do the following:

-	test of communication path between the Participating Operator's network elements and the Shared E-UTRAN/NG-RAN,

-	obtain fault reports,

-	retrieve RAN resource usage information.

28.2.3	Generation and retrieval of usage and accounting information

To facilitate interoperator accounting between Hosting E-UTRAN/NG-RAN Operator and Participating Operator the Hosting E-UTRAN/NG-RAN Operator needs to record E-UTRAN/NG-RAN resource usage by UEs of the Participating Operator.

The following requirements apply:

A Hosting E-UTRAN/NG-RAN Operator shall be able to collect events supporting the accounting of network resource usage separately for each Participating Operator. Collected events may be delivered to the subscriber's Participating Operator. This includes:

-	start of service in the Shared E-UTRAN/NG-RAN for a UE of the Participating Operator,

-	end of service in the Shared E-UTRAN/NG-RAN for a UE of the Participating Operator.

-    Radio resource usage and accounting information per cell in a time period for each Participating Operator (identified by PLMN ID) in the Shared E-UTRAN/NG-RAN.

-    Radio resource usage and accounting information per cell in a time period for each Participating Operator who shares E-UTRAN/NG-RAN in Non-Public scenarios (identified by PLMN ID and ID of NPN).

## ---End of the Change---