3GPP TSG-SA WG1 Meeting #92e	S1-204264r01

Electronic Meeting, 11-20 November 2020	(revision of S1-20xxxx)

Title:	FS_Resident: Use case on seamless switching from a UE-to-UE direct communication to an indirect communication via a residential gateway

Agenda Item:	7.11.1 FS_Resident

Source:	InterDigital

Contact:	Atle Monrad <atle dot monrad at interdigital dot com>

Abstract

This document proposes a use case on “seamless switching from a UE-to-UE direct communication to an indirect communication via a residential gateway”.

Proposal

It is proposed to include the below new use case on seamless switching from a UE-to-UE direct communication to an indirect communication via a residential gateway in 3GPP TR 22.858 version 0.1.0.

* * * *   First Change   * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

5G-RG	5G Residential Gateway

<ACRONYM>	<Explanation>

* * * *   Second Change   * * * *

## 5.x	Use case: seamless switching from a UE-to-UE direct communication to an indirect communication via a residential gateway

### 5.x.1	Description

This use case is depicted in Figure 5.x.1-1. It shows the scenario where two UEs who have been in direct communication using 3GPP technology switch seamlessly to an indirect communication using 3GPP technology via a 5G-RG as the two UEs move apart from each other.

Figure 5.x.1-1. Seamless switching from residential direct UE-to-UE to a residential network path via a residential gateway

### 5.x.2	Pre-conditions

Multiple 5G indoor base stations are deployed in the home (building) for serving different floors or rooms and interconnected via the 5G-RG. The 5G-RG is connected to the 5G-CN and the base stations are connected to the 5G-CN via the 5G-RG.

### 5.x.3	Service Flows

We consider a scenario where two UEs, UE1 and UE2, are connected via a direct device communication. Both UEs are assumed to be in proximity of one another (e.g. on the same floor) while enjoying their service via the direct link. An event occurs that triggers the move of UE1 away from UE2. While the UE1 moves away from UE2, it is the expectation of both UEs to continue their service seamlessly with the same QoS albeit through an indirect communication via the network. Below we present the service flows:

1.	UE1 (e.g. smartphone or tablet) and UE2 (e.g. laptop) are both on the same floor (Floor 2 in this example) in the home and connected to each other via 3GPP direct communication. UE1 is communicating with  UE2 using direct device communications. This service has a specific QoS (e.g. latency sensitive).

2.	UE1 moves to another floor (Floor 1 in this example) whilst maintaining its service with UE2 with the same specific QoS.

3.	As the UE1-UE2 move apart the direct device communication link starts to fail, UE1 connects automatically to indoor BS1 while UE2 connects automatically to indoor BS2.

NOTE:	UEs connect to indoor base station that provides the necessary connectivity to the other and necessary QoS.

4.	The UE1-UE2 service is maintained with the same QoS through the 5G-RG connecting BS1 and BS2.

### 5.x.4	Post-conditions

The UE1-UE2 service is continued seamlessly through the residential gateway with exactly the same QoS as through the direct device communication.

### 5.x.5	Existing features partly or fully covering the use case functionality

[3GPP TS 22.261 [2] clause 6.2.3 Service continuity: Requirements]

The 5G system shall support service continuity for a remote UE, when the remote UE changes from a direct network connection to an indirect network connection and vice-versa.

[3GPP TS 22.261 [2] clause 6.7.2: Priority, QoS, and policy control: Requirements]

The 5G system shall be able to support E2E (e.g. UE to UE) QoS for a service.

NOTE:	E2E QoS needs to consider QoS in the access networks, backhaul, core network, and network to network interconnect.

The 5G system shall be able to support QoS for applications in a Service Hosting Environment.

[3GPP TS 22.261 [2] clause 7.6.1 AR/VR: Gaming and Training Data Exchanging]

This use case is characterized by the exchange of the gaming or training service data between two 5G AR/VR devices.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 5G system shall be able to support seamless path switching for communication between UEs via the residential gateway.

[PR 5.x.6-1] The 5G system shall be able to provide E2E QoS control for the communication path between the UE and the 5G residential gateway via indoor 5G base station.

* * * *   End of Changes   * * *