3GPP TSG-SA WG1 Meeting #94bis	    																S1-212034r6
Electronic Meeting, July 5-12, 2021                                       					 	(Revision of S1-21xxxx)

Title:	Updated proposal on consolidated requirements for FS_VMR

Agenda Item:	2.5.1

Source:	Qualcomm

Contact:	Francesco Pica
	fpica <at> qti.qualcomm.com

# Abstract: This document includes an updated proposal on consolidated potential requirements for FS_VMR, based current TR 22.839 (v1.0.0).

# Introduction

This document includes:

- A text proposal for the consolidated requirements (CPRs), to be included in Section 7 of TR 22.839, for SA1 discussion and approval.

- Two Annexes:

- Annex A, including a summary of revisions and changes made during S1-94bis (vs the initial proposal in 2034)

- Annex B, capturing a table with a consolidation summary of all PRs, and mapping to the final CPRs.

# Proposal

A text proposal is provided below, to be included in Section 7 of TR 22.839.

------------------ START of TR text proposal --------------------

# 7	Consolidated Potential Requirements

Introduction

The requirements below refer to a “mobile base station relay” (or mobile BS relay), which is a mobile base station acting as a relay between a UE and the 5G network, i.e. providing a NR access link to UEs and connected wirelessly (using NR) through a donor NG-RAN to the 5G Core. Such mobile BS relay is assumed to be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle).

NOTE: the radio link used between a mobile BS relay and served UEs, as well as between mobile BS relay and donor RAN,  is assumed to be NR-Uu; in that regard, it should be clear that a mobile BS relay is different than a UE relay (which uses instead a PC5-based link to provide indirect connection to remote UEs).

Few main underlying assumptions and notes (from the TR overview and gap analysis) are:

- use cases and requirements cover single-hop relay scenarios as baseline (multi-hop is not precluded);

- legacy UEs are supported;

- current stage-1 requirements (e.g. on wireless self-backhaul, in TS 22.261), as well as existing stage-2/3 functionalities and architecture options (e.g. IAB) do not assume or address full relay mobility (e.g. BS relays on board of moving vehicles), thus may not cover the identified new potential requirements, which are intended to be specific to mobile BS relays;

- The identified requirements do not intend to imply or exclude specific network/relay architectures and topology solutions (e.g. could be IAB based, or others).

General requirements

CPR 001: The 5G system shall support efficient operation of mobile base station relays.

CPR 002: The 5G system shall be able to support means, for a mobile network operator, to configure, provision and control the operation of a mobile base station relay, [e.g. authorization, activation/deactivation, frequency bands, permitted geographic areas/locations or time periods].

CPR 003: The 5G system shall be able to support provisioning and configuration mechanisms to control UEs’ selection and access to a mobile base station relay, [e.g. based on UE authorization, geographic or temporary restrictions, user preference (for manual selection)]

[CPR 004: The 5G System shall be able to support mobile base station relays using a NR satellite access link to connect to RAN.]

CPR 005: The 5G system shall be able to support RAN sharing between multiple PLMNs for UEs connected to the 5G network via mobile base station relays.

NOTE 1: the above requirement assumes both relay and (donor) RAN resources, including UE access link and relay backhaul link, are shared among operators.

[CPR 006: The 5G system shall be able to configure and provision specific required QoS for traffic relayed via a mobile base station relay.

NOTE 2: QoS is intended as end-to-end QoS, i.e. covering both UE access and relay backhaul QoS.]

[CPR 007: The 5G system shall support means to expose monitoring information of a mobile base station relay (e.g. number of UEs connected, QoS) to an MNO’s authorized third-party, based on monitoring configuration requested by the third-party.]

[CPR 008: The 5G System shall be able to support configuration and connectivity of mobile base station relays for providing ad-hoc coverage and capacity extension in specific locations, e,g, for public safety scenarios. ]

CPR 009: The 5G system shall be able to provide means to optimize network behaviour to efficiently deliver data based on the mobility information (e.g., itinerary), known or predicted, of mobile base station relays.

CPR 010: The 5G system shall be able to support communication from/to users of one MNO (MNO-A) via mobile base station relays, where the traffic between the relay and the MNO-A network is transported using 5G connectivity (RAN and 5GC) provided by a different MNO (MNO-B).

NOTE 3: The 5G connectivity provided to the MNO-A relays by the different MNO (MNO-B) assumes a generic wireless backhaul transport, independent from the mobile BS relay functionalities.

Multi-link connectivity

[CPR 011: The 5G system shall be able to support UEs connectivity to RAN using simultaneously, a link without mobile BS relay and a link via a mobile BS relay, or simultaneous links via different mobile BS relays.

NOTE 4: the above requirements cover scenarios were the two links (to the RAN) could be connected to the same or different RAN node(s), and assuming both relay and RAN belong to the same PLMN. ]

[CPR 012: The 5G System shall be able to support mobile base station relays capable of using both NR satellite access and NR terrestrial access to connect to RAN, can be one access link at a time or simultaneously. ]

Mobility and Service Continuity

[CPR 013: The 5G system shall be able to provide means to support efficient UE cell selection and cell reselection (between mobile BS relays or between relays and RAN) in the presence of mobile base station relays.]

CPR 014: The 5G system shall be able to ensure end-to-end service continuity, in the presence of mobile base station relays

NOTE 5: the above requirement intends to cover different scenarios of UE mobility (e.g. UE moving between two mobile BS relays, or between macro RAN and BS relay) and BS relay mobility (e.g. BS relay moving between different donor RAN nodes).

Charging

CPR 015: The 5G system shall be able to identify and differentiate UEs’ traffic carried via a mobile base station relay and collect charging information, including specific relay information (e.g. geographic location served by the relay).

Other aspects

CPR 016: The 5G System shall support means for a mobile base station relay to have a certain subscription with a HPLMN, used to get access and connectivity to the HPLMN network (via a donor RAN).

CPR 017: The 5G System shall support the ability of a base station relay to roam from its HPLMN into a VPLMN.

CPR 018: The 5G system shall support mechanisms, for the HPLMN controlling a mobile base station relay, to enable/disable mobile relay operation if the relay (e.g. on a mobile vehicle) is roaming in a VPLMN.

CPR 019: The 5G system shall support mechanisms to disable mobile relay operation by a VPLMN where a mobile base station relay is roaming to.

CPR 020: The 5G system shall be able to fulfil necessary regulatory requirements (e.g. for support of emergency services) when UEs access the 3GPP network via a mobile base station relay.

CPR 021: The 5G system shall be able to support priority services (e.g. MPS) when UEs access the 3GPP network via a mobile base station relay.

[CPR 022: The 5G system shall be able to support location services for the UEs accessing 5GS via a mobile base station relay.]

CPR 023: The 5G system shall ensure that existing end-to-end 5G security between the UE and 3GPP network is unaffected when the UE accesses the 3GPP network via a mobile base station relay.

CPR 024: The 5G system shall be able to minimize radio interference possibly caused by mobile base station relays.

CPR 025: The 5G system shall minimize the impact of the presence of mobile base station relays on radio network management (e.g. through automatic neighbor cell list configuration).

-------------------------------- END of text proposal ------------------------------------

# Annex A – Revisions & Changes history

This annex captures the revision history of 2034, with updates/changes proposed/discussed during S1-94bis, for information/reference.

------------

Rev0 (submitted to S-94bis):

1) - as baseline text, a copy of the proposed consolidated requirements from S1-211502 sec. 7;

2) - new text (see rev1 for track changes/highlights):

a - some of the requirements left TBDs in S1-211502 (Annex), proposed to be moved/added to the consolidated part; others are merged or left out (see annex below for details).

b - suggested clarifications/changes to the baseline requirements

c - minor edits (e.g. to the introduction part)

------------ Revisions during S1-94bis ----------------

Rev1 changes (see rev1 for track changes/highlights) - based on comments before/during call#1

- Add descriptive text in the introduction to clarify assumptions and aspects related to spectrum, single-hop, legacy UEs, and general gaps vs IAB

- Add one new requirement on network management (in the “other aspects”, see PR 6-6)

- Small update to PR.5.15.6-1

------------
Rev2&3 updates (with track changes) – based on comments after call#1

- r2: proposed removals and simplification from Huawei (see comments in 2034r1_Huawei)

- r3: add edits proposed by Ericsson (in 2034r2_Ericsson)

-----------

Rev4 updates (see green highlights) – based on comments during call#2

- way forward proposal on some controversial changes/removals in r3, i.e. proposing text/requirements to be kept/reworded.

-----------

Rev5 changes: (see highlights, plus QC_r5 marked changes)

- addressing (most) comments from 2034r4_Ericsson, and other comments on the S1 reflector.

------------------ Proposed consolidation text (ALL NEW TEXT) ------------------
NOTE, all content represents NEW TEXT; revision marks show only incremental changes compared to earlier versions of the doc (i.e. on top of rev1), to facilitate readability and review.

Introduction

The requirements below refer to a “mobile base station relay” (or mobile BS relay), which is a mobile base station acting as a relay between a UE and the 5G network, i.e. providing a NR access link to UEs and connected wirelessly (using NR) through a donor NG-RAN to the 5G Core. Such mobile BS relay is assumed to be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle).

NOTE: the radio link used between a mobile BS relay and served UEs, as well as between mobile BS relay and donor RAN,  is assumed to be NR-Uu; in that regard, it should be clear that a mobile BS relay is different than a UE relay (which uses instead a PC5-based link to provide indirect connection to remote UEs).

Few main underlying assumptions and notes (from the TR overview and gap analysis) are:

- use cases and requirements cover single-hop relay scenarios as baseline (multi-hop is not precluded);

- legacy UEs are supported;

- current stage-1 requirements (e.g. on wireless self-backhaul, in TS 22.261), as well as existing stage-2/3 functionalities and architecture options (e.g. IAB) do not assume or address full relay mobility (e.g. BS relays on board of moving vehicles), thus may not cover the identified new potential requirements, which are intended to be specific to mobile BS relays;

- The identified requirements do not intend to imply or exclude specific network/relay architectures and topology solutions (e.g. could be IAB based, or others).

General requirements

[PR 5.1-1] The 5G system shall support efficient operation of mobile base station relays.

[PR 5.1-2] The 5G system shall be able to support means, for a mobile network operator, to configure, provision and control the operation of a mobile base station relay, e.g. authorization, activation/deactivation, frequency bands, permitted geographic areas/locations or time periods.

[PR 5.4-1] The 5G system shall be able to support provisioning and configuration mechanisms to control UEs’ selection and access to a mobile base station relay, e.g. based on UE authorization, geographic or temporary restrictions, user preference (for manual selection)

[PR5.16.6-1] The 5G System shall be able to support mobile base station relays using a NR satellite access link to connect to RAN.

[PR-5.17-1] The 5G system shall be able to support RAN sharing between multiple PLMNs for UEs connected to the 5G network via mobile base station relays.

NOTE 1: the above requirement assumes both relay and (donor) RAN resources, including UE access link and relay backhaul link, are shared among operators.

[PR 5.10-1] The 5G system shall be able to configure and provision specific required QoS for traffic relayed via a mobile base station relay.

NOTE 2: QoS is intended as end-to-end QoS, i.e. covering both UE access and relay backhaul QoS.

[PR 5.18-1] The 5G system shall support means to expose monitoring information of a mobile base station relay (e.g. number of UEs connected, QoS) to an MNO’s authorized third-party, based on monitoring configuration requested by the third-party.

[PR.5.15.6-1] The 5G System shall be able to support dynamic configuration and connectivity of one or more mobile base station relaysfor use only in specific locations where UEs would receive no other telecommunications coverage, e,g, for public safety scenarios.

[PR 5.21-1] The 5G system shall be able to provide means to optimize network behaviour to efficiently deliver data based on the mobility information (e.g., itinerary), known or predicted, of mobile base station relays.

[PR 5.20-1] The 5G system shall be able to support communication from/to users of one MNO (MNO-A) via mobile base station relays, where the traffic between the relay and the MNO-A network is transported using 5G connectivity (RAN and 5GC) provided by a different MNO (MNO-B).

NOTE 4: The 5G connectivity provided to the MNO-A relays by the different MNO (MNO-B) assumes a generic wireless backhaul transport, independent from the mobile BS relay functionalities.

Multi-link connectivity

[PR.5.13.6-2] The 5G system shall be able to support UEs connectivity to RAN using simultaneously, a link without mobile BS relay and a link via a mobile BS relay, or simultaneous links via different mobile BS relays.

NOTE 5: the above requirements cover scenarios were the two links (to the RAN) could be connected to the same or different RAN node(s), and assuming both relay and RAN belong to the same PLMN.

[PR 5.22.6-1] The 5G System shall be able to support mobile base station relays capable of using both NR satellite access and NR terrestrial access to connect to RAN, can be one access link at a time or simultaneously.

Mobility and Service Continuity

[PR 5.7-1] The 5G system shall be able to provide means to support efficient UE cell selection and cell reselection (between mobile BS relays or between relays and RAN) in the presence of mobile base station relays.

[PR 5.7-2] The 5G system shall be able to ensure end-to-end service continuity, in the presence of mobile base station relays

NOTE 9: the above requirement intends to cover different scenarios of UE mobility (e.g. UE moving between two mobile BS relays, or between macro RAN and BS relay) and BS relay mobility (e.g. BS relay moving between different donor RAN nodes).

Charging

[PR 5.10-2] The 5G system shall be able to identify and differentiate UEs’ traffic carried via a mobile base station relay  and collect charging information, including specific relay information (e.g. geographic location served by the relay).

Other aspects

[PR 6-1a] The 5G System shall support means for a mobile base station relay to have a certain subscription with a HPLMN, used to get access and connectivity to the HPLMN network (via a donor RAN).

[PR 6-1b] The 5G System shall support the ability of a base station relay to roam from its HPLMN into a VPLMN.

[PR 6-1c] The 5G system shall support mechanisms, for the HPLMN controlling a mobile base station relay, to enable/disable mobile relay operation if the relay (e.g. on a mobile vehicle) is roaming in a VPLMN.

[PR 6-1d] The 5G system shall support mechanisms to disable mobile relay operation by a VPLMN where a mobile base station relay is roaming to.

[PR 6-2] The 5G system shall be able to fulfil necessary regulatory requirements (e.g. for support of emergency services) when UEs access the 3GPP network via a mobile base station relay.

[PR 6-3] The 5G system shall be able to support priority services (e.g. MPS) when UEs access the 3GPP network via a mobile base station relay.

[PR 5.9-1]: The 5G system shall be able to support location services for the UEs accessing 5GS via a mobile base station relay.

[PR 6-4] The 5G system shall ensure that existing end-to-end 5G security between the UE and 3GPP network is unaffected when the UE accesses the 3GPP network via a mobile base station relay.

[PR 6-5] The 5G system shall be able to minimize radio interference possibly caused by mobile base station relays.

[PR 6-x] The 5G system shall minimize the impact of the presence of mobile base station relays on radio network management (e.g. through automatic neighbor cell list configuration)

-------------------------------- END of text proposal ------------------------------------

# Annex B – Consolidation history and mapping

| Potential Requirement, in current TR 22.839 (v1.0.0) | Initial Consolidation proposals (S1-212034, input to S1-94bis) | Final Consolidation proposal (S1-212034r6) [and CPR#] |
| --- | --- | --- |
| [PR 5.1-1] The 5G system shall support efficient operation of mobile base station relays, e.g. on board of mobile vehicles, connected wirelessly to the NG-RAN and serving nearby 5G NR UEs (inside and/or outside the vehicle). | Included, with rewording | Included [CPR 001] |
| [PR 5.1-2] The 5G system shall support means, for a mobile network operator, to configure, provision and control the operation of a mobile base station relay, including - activation and/or deactivation of mobile relay operation; - configuration of 5G spectrum (licensed or unlicensed) used by the mobile relay, over the radio links toward UE and RAN; - configuration of relay operating conditions e.g. based on geographic areas or locations, specific time period(s), vehicle’s speed, itinerary, etc. Note: besides the MNO, a 3rd party entity could also be enabled to provision and control some of the mobile base station relay operation, e.g. (de)activation, location restrictions, or other operating conditions. | Included, with rewording | Included, further reworded/simplified [CPR 002] |
| [PR 5.2-1] The 5G system shall support the use of mobile base station relays (e.g. mounted on vehicles), which can provide 5G access to its authorized UEs in the vicinity. | Merged into 5.4-1 |  |
| [PR 5.2-2] The 5G system shall enable the authorization of the mobile base station relays (e.g. mounted on vehicles) to work in a permitted area. When the mobile base station relay moves out of this area, it is not allowed to work as a base station. | Merged into 5.1-2 |  |
| [PR 5.2-3] The 5G network can dynamically configure the operation parameters of a mobile base station relay, such as frequency band, permitted geographical area, etc. | Merged with 5.1-2 |  |
| [PR 5.3-1] The 5G system shall support the use of vehicle mounted mobile base station relays, which provide 5G access to UEs in the vicinity | Covered by 5.1-1 |  |
| [PR 5.3-2] Subject to operator policy, the 5G system shall support means to configure and enable a vehicle mounted mobile base station relays to allow access to all UEs, or to only allow access to certain authorized UEs e.g., vehicle owner’s family members, friends, etc | Merged into 5.4-1 |  |
| [PR 5.4-1] The 5G system shall support provisioning and configuration mechanisms to control UEs’ access to the 5G network via a mobile base station relay, based on - User/UE subscription and/or permission (can be specific to each preferred relay); - User/UE or relay geographical location, time of the day, load, speed | Included, with rewording | Included, further reworded/simplified [CPR 003] |
| [PR 5.5-1] The 5G system shall support mechanisms to control UEs selection of mobile base station relays and UEs access to the 5G network via a mobile base station relay, based on - user/UE subscription and/or permission (can be specific to each preferred relay); - user preference, for manual selection of a specific relay. | Merged with 5.4-1 |  |
| [PR 5.6-1] The 5G system shall support the use of mobile base station relay, which provides 5G access to UEs in the vehicle along the vehicle itinerary | Covered by 5.1-1 |  |
| [PR 5.6-2] The 5G system shall provide means to ensure that UEs inside a vehicle, once provided with 5G access and connectivity via a mobile base station relay (e.g. mounted on the vehicle), remain connected via the mobile base station relay (e.g. mounted on the vehicle). | Merged into 5.7-2 |  |
| [PR 5.7-1] The 5G system shall be able to provide a means to optimize cell selection and minimize unnecessary cell reselection (between mobile base station relays or between mobile base station relays and fixed gNB,) in the presence of mobile base station relays. NOTE: This requirement is intended to provide the capability for the 5G system (UEs/ mobile base station relays) to be able to optimize selection of a mobile base station relay, e.g., in a vehicle where the UE is on board (or that moved together so far or that is expected to move together). | Included, with rewording | Included, further reworded/simplified [CPR 013] |
| [PR 5.7-2] The 5G system shall be able to provide a means to minimize unnecessary handover (between mobile base station relays, or between mobile base station relays and a fixed gNB and between) for a UE while served via an mobile base station relay, e.g., based on UE and relay relative mobility or speed. | Included, with rewording | Included, further reworded/simplified [CPR 014] |
| [PR 5.8-1] The 5G system shall be able to provide a means to perform load balancing among mobile base station relays. NOTE: This requirement is intended to provide the capability for the 5G system (UEs/ mobile base station relays) to be able to optimize the load of network resources whenever possible | Included (w/o note) | Covered by 5.1-1 |
| [PR 5.9-1]: The 5G system shall support providing location service for the UEs accessing to the 5GS network via a mobile base station relay (e.g. mounted on a vehicle). | Included, in the other aspects | Included [CPR 022] |
| [PR 5.9-2]: The 5G system shall support providing location information to a requesting UE or other location entity, for UEs accessing the 5GS network via a mobile base station relay (e.g. mounted on a vehicle), considering e.g. specific location granularity, and efficient UE power consumption. | covered by (5.9-1) |  |
| [PR 5.10-1] The 5G system shall be able to configure and provision specific required QoS for traffic relayed via a mobile base station relay, e.g. may be based on user’s subscription, geographical area, day/time of operation. relay type, itinerary/mobility, etc. NOTE: QoS is intended as end-to-end QoS, i.e. covering both UE access and relay backhaul QoS. | Included | Included, further reworded/simplified [CPR 006] |
| [PR 5.10-2] The 5G system shall be able to identify and differentiate the traffic relayed via a mobile base station relay, e.g. to apply specific charging policies. | Included, with rewording | Included [CPR 015] |
| [PR 5.10-3] Online and offline charging shall be supported for UEs connected via a mobile base station relay | Covered by 5.10-2 |  |
| [PR 5.10-4] The 5G system shall be able to provide and collect charging information for UEs using a mobile base station relay, including e.g.: - Identification of UEs/users involved; - Initiation/termination time of relay communication; - Duration and amount of data transmitted and received; - Type of service, QoS, other allocated resources (e.g. spectrum); - Geographic location(s) served by the relay - Other relay mobility information (e.g. itinerary, speed) | Merged with 5.10-2 |  |
| [PR 5.11.1-1] The 5G system shall be able to support efficient handover when a UE active communication changes from the macro network to a mobile base station relay (e.g. mounted on a vehicle) and vice versa, ensuring end-to-end service continuity during mobility of the UE (e.g. entering or leaving the vehicle) and/or the relay | Merge with #2 |  |
| [PR 5.11.1-2] The 5G system shall be able to support efficient handover when a UE active communication changes from the macro network to a mobile base station relay (e.g. mounted on a vehicle) and vice versa, ensuring end-to-end service continuity during mobility of the UE (e.g. moving outside the vehicle) and/or the relay. | Merge with 5.7-2 |  |
| [PR 5.11.2-1] The 5G system shall be able to support efficient handover of a UE active communication when a mobile base station relay (e.g. serving a UE inside a vehicle) changes between macro network nodes, ensuring end-to-end service continuity during mobility of the relay. | Merge with #2 |  |
| [PR 5.11.2-2] The 5G system shall be able to support efficient handover of a UE active communication when a mobile base station relay (e.g. serving a UE outside a vehicle) changes between macro network nodes, ensuring end-to-end service continuity during mobility of the relay. | Merge with 5.7-2 |  |
| [PR 5.11.3-1] The 5G system shall be able to support efficient handover when a UE active communication changes between mobile base stations relays (e.g. mounted on a vehicle), ensuring end-to-end service continuity during mobility of the UE (e.g. moving inside the vehicle). | Merge with #2 |  |
| [PR 5.11.3-2] The 5G system shall be able to support efficient handover when a UE active communication changes between mobile base stations relays (e.g. mounted on a vehicle), ensuring end-to-end service continuity during mobility of the UE (e.g. moving outside the vehicle) and/or the relays | Merge with 5.7-2 |  |
| [PR 5.11.4-1] The 5G system shall be able to support efficient handover of a group of UEs’ active communications when a mobile base station relay serving a group of in vehicle UEs moves between donor gNBs, ensuring end-to-end service continuity during mobility of the relay | Merge with 5.7-2 |  |
| [PR.5.12.6-1] The 5G system shall provide means for a UE to select a suitable mobile base station relay (e.g., a mobile base station relay that has a similar trajectory as the UE) | Merged with 5.7-1 |  |
| [PR.5.13.6-1] The 5G system shall be able to minimize potential efficiently control UE idle mode mobility ping-pongs when mobile base station relays are used, e.g. to facilitate UEs connecting to the macro RAN (as opposed to a vehicle relay). | Covered by 5.7-1 and 5.7-2 |  |
| [PR.5.13.6-2] The 5G system shall be able to support simultaneous UE connectivity to RAN, using a direct UE access link to the macro RAN together with an access link via a mobile base station relay (e.g. mounted on a vehicle). NOTE 1: the above requirement covers the scenarios were the UE access links (to the macro RAN and via the mobile BS relay) could be connected to the same or different RAN node(s). NOTE 2: the above requirement includes supporting efficient addition and removal of a simultaneous access link, e.g. adding and removing a UE access link via a mobile base station relay while the UE is connected to the macro RAN. | Included, with rewording/simplification | Included, further reworded [CPR 011] |
| [PR.5.14.6-1] The 5G system shall be able to support mechanisms to optimize mobility (e.g. re-selection, handover) and energy efficiency for a UE camped or connected via a vehicle mobile BS relay (e.g. for UEs located inside a vehicle, equipped with a BS relay) | Covered by 5.7-1/2 |  |
| [PR.5.15.6-1] The 5G System shall be able to support dynamic deployment and configuration of one or more mobile base station relays (e.g. mounted on vehicles) for providing ad-hoc indoor coverage extension in specific locations. | Included | Included, further reworded/simplified [CPR 008] |
| [PR5.16.6-1] The 5G System shall be able to support mobile vehicle relays using NR satellite access to connect to a remote donor RAN node via a satellite link. | Included, with rewording | Included, with edits [CPR 004] |
| [PR5.16.6-2] The 5G System shall be able to support a mobile vehicle relay using NR satellite access with service continuity in the scenario where there is a transition from one serving satellite to another serving satellite | Included, with rewording | Covered by 5.7-2 |
| [PR-5.17-1] The 5G system shall be able to support RAN sharing between multiple PLMNs for UEs connected to 5G network via mobile base station relays (e.g. mounted on vehicles). NOTE 1: the above requirement assumes that the mobile base station relays broadcast all PLMN-IDs of the sharing PLMN operators. NOTE 2: the above requirement assumes both relay and donor RAN resources, including UE access link and relay backhaul link, are shared among operators. | Included, with rewording/simplification | Included, with edits [CPR 005] |
| [PR-5.17-2] The 3GPP System shall support end-to-end service continuity for a UE having active connection with a RAN via a mobile base station relay (e.g. mounted on a vehicle) when there is a change between a shared RAN (e.g. inside the vehicle) and a non-shared RAN (e.g. outside a vehicle), or when RAN sharing changes (for the same mobile relay) between different sharing PLMNs. | covered by requirement on service continuity (5.7-2) |  |
| [PR 5.18-1] The 5G system shall support means to expose monitoring information of a mobile base station relay (e.g. number of UEs connected, QoS, geographical location, time of data monitoring/collection, etc.) to an authorized third-party based on monitoring parameters configured by the authorized third-party (e.g. data to monitor/collect, periodicity/schedule of monitoring, etc.). | Included | Included, further reworded/simplified [CPR 007] |
| [PR 5.19-1] The 5G system shall be able to support simultaneous UE connectivity to RAN using multiple access links provided via different mobile base station relays (e.g. mounted on different vehicles). NOTE 1: the above requirement covers scenarios were the UE access links (via the mobile BS relays) could be connected to the same or different RAN node(s), assuming both relay and donor RAN node(s) belong to the same PLMN, NOTE 2: the above requirement includes supporting efficient addition and removal of a simultaneous access link via a mobile base station relay, while the UE is connected to the RAN via another mobile base station relay. | Included (with updates) | Merged into 5.13.6-2 |
| [PR 5.19-2] The 5G system shall be able to support, for a UE simultaneously connected to RAN via different mobile base station relays (e.g. mounted on vehicles), service continuity during various mobility events, e.g. when the connected mobile BS relays and/or the macro donor node(s) change. | covered by requirement on service continuity (5.7-2) |  |
| [PR 5.20-1] The 5G system shall be able to support communication from/to users of one MNO (MNO-A) via mobile vehicle base station relays, where the relayed traffic is transported to/from the MNO-A network using 5G connectivity (RAN and 5GC) provided by a different MNO (MNO-B). NOTE 1: the above requirement includes support of necessary policies and provisioning for end-to-end user’s QoS, e.g. based on MNO-A user/UE’s subscription. NOTE 2: The 5G transport and connectivity provided by the different MNO (MNO-B) assumes a generic wireless BS backhaul, independent from the BS relay functionalities. | Included | Included, further reworded/simplified [CPR 010] |
| [PR 5.21-1] The 5G system shall be able to provide a means to optimize network behaviour to efficiently deliver data based on the mobility information (e.g., itinerary), known or predicted, of the vehicle-mounted relays. | Included, with edits | Included [CPR 009] |
| [PR 5.22.6-1] The 5G System shall be able to support mobile base station relays using NR satellite access and using NR terrestrial access to connect to 5GC. Note 1: The simultaneous satellite and terrestrial access links (from the relay) are assumed to carry traffic pertaining to different UEs. | Included, with rewording | Included, further reworded/simplified [CPR 012] |
| [PR 5.22.6-2] The 5G System shall be able to support a mobile base station relay capable using both NR satellite access and NR terrestrial access with service continuity in the scenario when this mobile base station relay switch from using NR satellite access to using NR terrestrial access, and vice versa. | Included, with rewording | Covered by 5.7-2 |
| [PR 5.23.6-1] The 5G System shall be able to support mobile base station relays capable of connecting via both NR satellite access and NR terrestrial access, to select one access as a relay link between the mobile base station relays and the 5GC. | merged with 5.22.6-1 |  |
| [PR 5.23.6-2] The 5G System shall be able to support mobile base station relays capable of connecting simultaneously via NR satellite access and NR terrestrial access as relay link to connect via both to 5GC at the same time. | merged with 5.22.6-1 |  |
| [PR 5.23.6-3] The 5G System shall be able to support mobile base station relays capable of connecting in parallel via NR satellite and NR terrestrial access to add a second relay link to allow more UEs to access the network and e.g. to adapt to increased bandwidths demand for access via this mobile base station relay. | merged with 5.22.6-1 |  |
| [PR 5.23.6-4] The 5G System shall be able to support mobile base station relays connected via both NR satellite and NR terrestrial access to remove one of two relay links to adapt to decreased bandwidths demand of UEs accessing via this mobile base station relay. | merged with 5.22.6-1 |  |
| [PR 5.23.6-5] The 5G System shall be able to support mobile relay traffic load balancing across satellite and terrestrial access links, for mobile base station relays that use both NR satellite access and NR terrestrial access as relay link at the same time. | merged with 5.22.6-1 |  |
| [PR 6-1a] The 5G System shall support means for a mobile base station relay to have a subscription with a HPLMN, used to get access and connectivity to the HPLMN 5GC (via a donor RAN). [PR 6-1b] The 5G System shall support the ability of a base station relay to roam from its HPLMN into a VPLMN. [PR 6-1c] The 5G system shall support mechanisms, for the HPLMN controlling a mobile base station relay, to enable/disable mobile relay operation if/when the relay (e.g. on a mobile vehicle) is roaming in a VPLMN. [PR 6-1d] The 5G system shall support mechanisms to disable mobile relay operation by VPLMN where a mobile base station relay is roaming to. | Included | Included [CPR 016 – 019] |
| [PR 6-2] The 5G system shall be able to fulfil necessary regulatory requirements (e.g. for support of emergency services) when UEs access the 3GPP network via a mobile base station relay | Included | Included [CPR 020] |
| [PR 6-3] The 5G system shall be able to support priority services (e.g. MPS) when UEs access the 3GPP network via a mobile base station relay | Included | Included [CPR 021] |
| [PR 6-4] The 5G system shall ensure that end-to-end 5G security between the UE and 3GPP network is supported when the UE accesses the 3GPP network via a mobile base station relay | Included | Included [CPR 023] |
| [PR 6-5] The 5G system shall be able to minimize radio interference possibly caused by mobile base station relays (e.g. when mounted on moving vehicles) | Included | Included [CPR 024] |
| [PR 6-x] The 5G system shall minimize the impact of the presence of mobile base station relays (e.g. mounted on vehicles) on radio network management (e.g. through automatic neighbor cell list configuration) | N.A (new, proposed during consolidation) | Included, in other aspects [CPR 025] |