3GPP TSG-SA1 Meeting #94-bis-e	S1-212042

Electronic Meeting, 4 – 12 July 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.858 | CR | 0006 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consolidation of Resident requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Resident |  |  |  |  |  | Date: |  |  | 29-6-2021 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Potential requirements in use case sections need to be consolidated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Consolidation of potential requirements from use cases in a number of consolidated potential requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential requirements not consolidated before start of normative work. This would imply more normative work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 7.3, 7.4, 7.5, 7.6, 7.7, 7.8, 7.9, 7.10, 7.11, 7.12(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.2	General

Table 7.2-1 – RESIDENT General Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR.7.2-1 |  | Subject to local regulations, the 5G system shall support regulatory requirements for emergency calls, PWS and eCall for UEs connected to a CPN. |  |

## 7.3	Gateway

Table 7.3-1 – RESIDENT Gateway Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
|  |  |  | Descriptive text will cover that an eRG is evolved from a 5G-RG |
| CPR.7.3-1 | PR 5.13.6-001 | The 5G system shall be able to support IP traffic offload within the CPN. NOTE: The priority of offload can be from default configuration, network or user. |  |
|  | PR 5.19.6-001 |  | Descriptive text will cover that an eRG can be connected to a 5G Core Network over wireless access, fixed broadband access or hybrid access |
|  |  |  | move to provisioning and merge |
| CPR.7.3-3 | PR 5.17.6-001 PR. 5.17.6-002 | When the eRG has lost connection with the 5G networkthe eRG shall allow existing communication using non-3GPP access/control within the CPN to continue. NOTE: Setting up new communication sessions without eRG connection to the 5GC is only possible with non-3GPP provided credentials and over non-3GPP access. |  |
| CPR.7.3-4 | PR 5.17.6-001 PR. 5.17.6-003 | When there is no 5G network connection and the PRAS is serving UEs under its coverage using: - licensed spectrum, the PRAS may continue transmission using the licensed spectrum for direct device communication for an operator-defined maximum time period. - During this time period, if the PRAS continues transmission using licensed spectrum, existing communication using licensed spectrum for direct device communication within the CPN will continue. - At the end of this time period, the PRAS air interface is deactivated. - If the time period is undefined, the PRAS air interface is deactivated. - unlicensed spectrum, the PRAS shall allow direct device communication using unlicensed spectrum within the CPN to continue. NOTE2: Setting up new communication sessions without PRAS connection to the 5G network is only possible with non-3GPP provided credentials and using non-3GPP access. |  |

## 7.4	Service Discovery

Table 7.4-1– RESIDENT Service Discovery Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR.7.4-1 | PR 5.2.6-008 | The 5G system shall support a service discovery mechanism for a UE or device in a CPN to discover capabilities of other UEs or authenticated/authorized devices in the CPN, for example: - Capabilities and status (e.g., relay, eRG) - Whether an eRG has external data network connectivity - Expected availability of power (e.g., how long is remaining battery life) - Supported applications/services (e.g. similar to what is provided by UPNP) - Device manufacturer - Security/encryption mechanisms available - Connection types support by other UEs or devices (e.g. licensed spectrum direct connection, non-licensed spectrum direct connection) NOTE: service discovery can also use existing service discovery mechanism, e.g. UPNP. | Potential requirement text from TR22.859 |

## 7.5	Application Servers

Table 7.5-1 – RESIDENT Application Servers Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR.7.5-1 | PR 5.11.6-001 | The 5G system shall support applications on an Application Server connected to the Customer Premises Network. |  |
| CPR.7.5-2 | PR 5.11.6-002 | Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path, modifying the path as needed when the UE moves or application changes location, between a UE in an active communication and: - an application in a Service Hosting Environment; or - an application server located in a customer premises network. |  |
| CPR.7.5-3 | PR 5.11.6-002 | The 5G network shall maintain user experience (e.g. QoS, QoE) when a UE in an active communication moves from a location served by a Service Hosting Environment to: - another location served by a different Service Hosting Environment; or - another location served by an application server located in a customer premises network, and vice versa. |  |
| CPR.7.5-4 | PR 5.11.6-002 | The 5G network shall maintain user experience (e.g. QoS, QoE) when an application for a UE moves as follows: - from a Service Hosting Environment to an application server located in a customer premises network, and vice versa |  |

## 7.6	Identification, Privacy, and security

Table 7.6-1 – RESIDENT Identification, Privacy, and security Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR.7.6-1 | PR 5.2.6-009 | The 5G system shall ensure that communications to/from individual UEs in a CPN can be associated to a subscription by the 5G network. | What can the 5G system do to ensure this? |
|  | PR 5.22.6-001 PR 5.22.6-002 | T | Support of UIA (as defined in 22.101.clause 26a) is treated in descriptive text. |
| CPR.7.6-2 | PR 5.5.6-006 | The 5G system shall ensure an Evolved Residential Gateway provides user privacy protection for UEs that are using the Evolved Residential Gateway, including communication confidentiality, location privacy and identity protection. NOTE: Privacy protection should not block differentiated routing and QoS at the eRG for different destinations and services for the UE(s). | What can the 5G system do to ensure this? Is this covered by CPR.7.6-4 - CPR.7.6-7 below? |
| CPR.7.6-3 | PR 5.2.6-006 | The 5G system shall ensure the Premises Radio Access Station (PRAS) does not compromise user privacy for UEs that are using the PRAS, including communication confidentiality, location privacy and identity protection. NOTE: Privacy protection should not block differentiated routing, QoS, and services for the UE(s). | What can the 5G system do to ensure this? Is this covered by CPR.7.6-4 - CPR.7.6-7 below? |
| CPR.7.6-4 | PR 5.5.6-003 PR 5.5.6-004 | The 5G system shall support a mechanism to minimize security impact of communications using an eRG. |  |
| CPR.7.6-5 | PR 5.5.6-005 | The 5G system shall enable the network operator associated with an Evolved Residential Gateway to control the security policy of an Evolved Residential Gateway. |  |
| CPR.7.6-6 | PR 5.2.6-003 PR 5.2.6-004 | The 5G system shall support a mechanism to minimize the security impact of communications via a PRAS. |  |
| CPR.7.6-7 | PR 5.2.6-005 | The 5G system shall enable the network operator associated with the Premises Radio Access Station (PRAS) to control the security policy of the PRAS. |  |
| CPR.7.10-8 | PR 5.21.6-001 | The 5G system shall provide support for a network operator to authenticate a PRAS. Editor’s Note: Authentication and authorization in these requirements need to be clarified. |  |
| CPR.7.10-9 | PR 5.21.6-002 | The 5G system shall provide support for a network operator to authorize a PRAS for its use in a CPN. Editor’s Note: Authentication and authorization in these requirements need to be clarified. |  |

## 7.7	Direct Communications

Table 7.7-1 – RESIDENT Direct Communications Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text |  |
|  | PR 5.10.6-001 |  | Support of direct communication and path switching is covered in descriptive text |

## 7.8	Connectivity - QoS - charging

Table 7.8-1 – RESIDENT Connectivity, QoS and Charging Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR.7.8-1 | PR 5.5.6-002 | The 5G system shall support routing efficiency for data traffic between two UEs through an Evolved Residential Gateway. Editor’s Note: This requirement can be merged during potential requirement consolidation. | Proposed to be deleted in S1-212021 |
| CPR.7.8-2 | PR 5.4.6-001 | The 5G system shall support routing efficiency for data traffic between a UE and a non-3GPP device through an Evolved Residential Gateway. Editor’s Note: This requirement can be merged during potential requirement consolidation. | Proposed to be deleted in S1-212032 |
| CPR.7.8-3 | PR 5.6.6-001 PR 5.1.6-002 | The 5G system shall support end-to-end QoS for UEs connected to the eRG via a PRAS. NOTE: End-to-end QoS includes the customer premises network between PRAS and eRG and the (fixed) access network between the eRG and the 5G core network. |  |
| CPR.7.8-4 | PR 5.3.6-002 PR 5.10.6-002 | The 5G system shall support end-to-end QoS for UEs directly connected to the eRG via either 3GPP RAT or non-3GPP (R)AT. NOTE: End-to-end QoS includes the customer premises network between UE and eRG and the (fixed) access network between the eRG and the 5G core network. |  |
|  |  |  | Seems to be covered by CPR.7.8-3 and CPR.7.8-4 |
| CPR.7.8-6 | PR 5.2.6-007 | The 5G system shall be able to generate charging information that can differentiate between UEs connected to the Premises Radio Access Station (PRAS) and between backhaul for the PRAS and other data traffic over the same access. |  |

## 7.9	Provisioning

Table 7.9-1 – RESIDENT Provisioning Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR.7.9-1 | PR 5.18.6-001 PR 5.19.6-002 PR 5.7.6-001 | The 5G system shall enable configuration and management of an eRG by both the network operator of the public (mobile) network the eRG is connected to and, within the boundaries defined by the operator, by an Authorised Administrator. The 5G system shall support a mechanism for the network operator to provision an eRG with : - policies on which transport (e.g. wireless, cable, etc.) is best suited for different negotiated QoS levels. - credentials for verification - initial OA&M information |  |
| CPR.7.9-2 CPR 7.9-3 | PR 5.18.6-003 PR 5.12.6-002 PR 5.21.6-004 | The 5G system shall enable the network operator to configure a PRAS with: - radio settings pertaining to licensed spectrum, - credentials for verification, - identifier, and - Initial OA&M information. The 5G system shall enable the Authorised Administrator to configure a PRAS with UE access considerations (allowing all UEs, or allowing specific UEs only) |  |
|  |  |  | Merged into CPR.7.9-1 |
|  |  |  | Merged into CPR 7.9-2/3 |
|  |  |  | Merged into CPR.7.9-1 |
|  |  |  | Merged into CPR.7.9-3 |
|  |  |  | Merged into CPR.7.9-2 |

## 7.10	Premises Radio Access Station

Table 7.10-1 – RESIDENT Premises Radio Access Station Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR.7.10-1 | PR 5.1.6-001 | The 5G system shall enable the network operator to provide any 5G services to any 5G UE via a Premises Radio Access Station (PRAS) connected via an evolved Residential Gateway (eRG). NOTE 1: The PRAS may also be co-located with the eRG |  |
|  |  |  | Covered by CPR.7.10-1 |
|  |  | NOTE 2: Whether 5G UEs from VPLMNs in the same country as the HPLMN can use the PRAS is subject to regulatory policy on national roaming. | Covered by CPR.7.10-1, keep the note |
|  |  |  | Move to provisioning |
| CPR.7.10-5 | PR 5.12.6-003 | The 5G system shall minimize service disruption when a CPN communication path changes between two PRASes. |  |
| CPR.7.10-6 | PR 5.14.6-001 | The 5G system shall be able to support PRAS sharing between multiple PLMNs. |  |
| CPR.7.10-7 | PR 5.18.6-002 | The 5G System shall support PRAS that use licensed spectrum, use unlicensed spectrum, or can use both unlicensed and licensed spectrum. |  |
|  |  |  | Move to security |
|  |  |  | Move to security |
|  |  |  | Move to provisioning |

## 7.11	5G-LAN

Table 7.11-1 – RESIDENT 5G-LAN Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR.7.11-1 | PR 5.8.6-001 | The 5G system shall be able to support large numbers of small 5G LAN-VNs. NOTE: Targeting residential deployments translate into millions of 5GLAN-VN per operator per country. These 5G LAN-VNs may contain between 10-50 devices per LAN |  |
| CPR.7.11-2 | PR 5.8.6-002 | The 5G system shall support a mechanism to enable authorized 3rd parties to authorize/deauthorize UEs to access a 5G LAN-VN. |  |
| CPR.7.11-3 | PR 5.9.6-001 | The 5G system shall support the use of an evolved Residential Gateway to connect 5G devices from a 5G LAN VN with non-3GPP devices on an in-home LAN. |  |
| CPR.7.11-4 | PR 5.16.6-001 | The 5G system shall support devices in a 5G LAN-Virtual Network (5G LAN-VN) to access a fixed IP VPN. Editor’s Note: This requirement may need to be further studied. Editor’s Note: Further potential new requirements to support the use case may be identified. | It is not clear what is new here. |

## 7.12	Broadcast Multicast

Table 7.12-1 – RESIDENT Broadcast Multicast Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR.7.12-1 | PR.5.20.6-001 PR.5.20.6-002 PR.5.20.6-003 PR. 5.15.6-001 | Under operator control, an eRG, shall be able to efficiently deliver 5G multicast/broadcast services to authorized UEs and non-3GPP devices in the CPN. |  |