3GPP TSG-SA1 Meeting #94-bis-e	S1-212042r6

Electronic Meeting, 4 – 12 July 2021	(revision of S1-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.858 | CR | 0006 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consolidation of Resident requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Resident |  |  |  |  |  | Date: |  |  | 2021-07-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Potential requirements in use case sections need to be consolidated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Consolidation of potential requirements from use cases in a number of consolidated potential requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential requirements not consolidated before start of normative work. This would imply more normative work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 7.3, 7.4, 7.5, 7.6, 7.7, 7.8, 7.9, 7.10, 7.11, 7.12(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.2	General

Table 7.2-1 – RESIDENT General Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR 7.2-1 |  | Subject to local regulations, the 5G system shall support regulatory requirements for emergency calls, PWS and eCall for UEs connected to a CPN. |  |

## 7.3	Gateway

Table 7.3-1 – RESIDENT Gateway Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
|  |  |  | Descriptive text will cover that an eRG is evolved from a 5G-RG |
| CPR 7.3-1 | PR 5.13.6-001 | The 5G system shall be able to support IP traffic offload within the CPN. NOTE: The priority of offload can be from default configuration, network or user. |  |
|  | PR 5.19.6-001 |  | Descriptive text will cover that an eRG can be connected to a 5G Core Network over wireless access, fixed broadband access or hybrid access |
| CPR 7.3-2 | PR. 5.17.6-003 | When the CPN has lost connectivity with the 5G network, the 5G system shall provide an operator-controlled mechanism to enable: - in the default configuration, or under certain conditions configured by the operator, the PRAS radio interface shall be deactivated; and - under certain other conditions configured by the operator, the CPN shall continue existing intra-CPN communication, as long as no interaction with the 5G network is needed (e.g. refreshing security keys). NOTE 1: The requirement above relates to intra-CPN operations and subject to operator policy and control, under certain situations. NOTE 2: Setting up new intra-CPN communication sessions without CPN connection to the 5G network is only possible with non-3GPP provided credentials |  |

## 7.4	Service Discovery

Table 7.4-1– RESIDENT Service Discovery Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR 7.4-1 | PR 5.2.6-008 | The 5G system shall enable a UE in a CPN to discover other UEs or non-3GPP devices within the same CPN |  |
| CPR 7.4-1 | PR 5.2.6-008 | The 5G system shall efficiently support service discovery mechanisms where UEs in a CPN can discover, subject to access rights: - status, availability and reachability of other entities (e.g. other UEs or non-3GPP devices) on the CPN - capabilities of other entities on the CPN (e.g. eRG, relay UE) and/or; - services provided by other entities on the CPN (e.g. a UE is a colour printer). | Potential requirement text from TR22.859 |

## 7.5	Application Servers

Table 7.5-1 – RESIDENT Application Servers Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR 7.5-1 | PR 5.11.6-001 | The 5G system shall support applications on an Application Server connected to the Customer Premises Network. |  |
| CPR 7.5-2 | PR 5.11.6-002 | Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path, modifying the path as needed when the UE moves or application changes location, between a UE in an active communication and: - an application in a Service Hosting Environment; or - an application server located outside the operator’s network; or - an application server located in a customer premises network. | The text in italics is part of an existing requirement in 22.261 [2], the last bullet is proposed to be added. |
| CPR 7.5-3 | PR 5.11.6-002 | The 5G network shall maintain user experience (e.g. QoS, QoE) when a UE in an active communication moves from a location served by a Service Hosting Environment to: - another location served by a different Service Hosting Environment; or - another location served by an application server located in a customer premises network, and vice versa. | The text in italics is part of an existing requirement in 22.261 [2], the last bullet is proposed to be added. |
| CPR 7.5-4 | PR 5.11.6-002 | The 5G network shall maintain user experience (e.g. QoS, QoE) when an application for a UE moves as follows: - within a Service Hosting Environment; or - from a Service Hosting Environment to another Service Hosting Environment; or - from a Service Hosting Environment to an application server located place outside the operator’s network; or - from a Service Hosting Environment to an application server located in a customer premises network, and vice versa | The text in italics is part of an existing requirement in 22.261 [2], the last bullet is proposed to be added. |

## 7.6	Identification, Privacy, and security

Table 7.6-1 – RESIDENT Identification, Privacy, and security Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
|  | PR 5.22.6-001 PR 5.22.6-002 |  | Support of UIA (as defined in 22.101.clause 26a) is treated in descriptive text. |
| CPR 7.6-2 | PR 5.5.6-006 | The 5G system shall provide user privacy protection for UEs that are using the Evolved Residential Gateway, including communication confidentiality, location privacy and identity protection. NOTE: Privacy protection should not block differentiated routing and QoS at the eRG for different destinations and services for the UE(s). |  |
| CPR 7.6-3 | PR 5.2.6-006 | The 5G system shall provide user privacy for UEs that are using the PRAS, including communication confidentiality, location privacy and identity protection. NOTE: Privacy protection should not block differentiated routing, QoS, and services for the UE(s). |  |
| CPR 7.6-4 | PR 5.5.6-003 PR 5.5.6-004 | The 5G system shall support a mechanism to minimize the security risk of communications using an eRG. |  |
| CPR 7.6-5 | PR 5.5.6-005 | The 5G system shall enable the network operator associated with an Evolved Residential Gateway to control the security policy of an Evolved Residential Gateway. |  |
| CPR 7.6-6 | PR 5.2.6-003 PR 5.2.6-004 | The 5G system shall support a mechanism to minimize the security risk of communications via a PRAS. The PRAS (and its associated backhaul connectivity) shall provide a level of security equivalent to regular 5G base stations. |  |
| CPR 7.6-7 | PR 5.2.6-005 | The 5G system shall enable the network operator associated with the Premises Radio Access Station (PRAS) to control the security policy of the PRAS. |  |
| CPR 7.6-8 | PR 5.2.6-009a | The 5G system shall support authentication of a UE with 3GPP credentials for communication with entities (UEs, devices) in a CPN. NOTE: To support this functionality the CPN needs to be connected with the 5G core network. |  |
| CPR 7.6-9 | PR 5.21.6-001 | The 5G system shall provide support for a network operator to authenticate a PRAS. |  |
| CPR 7.6-10 | PR 5.21.6-002 | The 5G system shall provide support for a network operator to authorize a PRAS for its use in a CPN. |  |

## 7.7	Direct Communications

Table 7.7-1 – RESIDENT Direct Communications Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text |  |
|  | PR 5.10.6-001 |  | Support of direct communication and path switching is covered in descriptive text |

## 7.8	Connectivity - QoS - charging

Table 7.8-1 – RESIDENT Connectivity, QoS and Charging Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR 7.8-1 | PR 5.5.6-002 PR 5.4.6-001 | Subject to regulatory requirements and operator policy, the 5G system shall support an efficient data path through an Evolved Residential Gateway for intra-CPN data traffic to or from a UE. NOTE: For services an operator deploys in the 5G network (i.e. not in the CPN), local data routed via eRG does not apply. |  |
| CPR 7.8-2 | PR 5.6.6-001 PR 5.1.6-002 PR 5.5.6-001 | The 5G system shall support end-to-end QoS for UEs connected to the eRG via a PRAS. NOTE: End-to-end QoS includes the customer premises network between PRAS and eRG and the access network between the eRG and the 5G core network. |  |
| CPR 7.8-3 | PR 5.3.6-002 PR 5.5.6-001 | The 5G system shall support end-to-end QoS for UEs directly connected to the eRG via either 3GPP RAT or non-3GPP (R)AT. NOTE: End-to-end QoS includes the customer premises network between UE and eRG and the access network between the eRG and the 5G core network. |  |
| CPR 7.8-4 | PR 5.10.6-002 | The 5G system shall support real time E2E QoS monitoring and control for any data traffic path (i.e. from/to a UE to/from the 5GC and to/from another UE) via a PRAS and an eRG when there is connectivity to the 5G system. |  |
| Alternative CPR 7.8-2 | PR 5.6.6-001 PR 5.1.6-002 PR 5.5.6-001 PR 5.3.6-002 PR 5.10.6-002 | The 5G system shall support real time E2E QoS monitoring and control for any intra-CPN data traffic to or from a UE (i.e. via eRG or via PRAS and eRG) |  |
| Alternative CPR 7.8-3 | PR 5.6.6-001 PR 5.1.6-002 PR 5.5.6-001 PR 5.3.6-002 PR 5.10.6-002 | The 5G system shall support real time E2E QoS monitoring and control for any data traffic between a UE within a CPN and the 5G network (i.e. via eRG or via PRAS and eRG) |  |
| CPR 7.8-5 | PR 5.2.6-007 PR 5.2.6-009 | The 5G system shall support charging and LI of data traffic to/from individual UEs in a CPN (i.e. UEs behind the eRG and/or PRAS). |  |
| CPR 7.8-6 | PR 5.2.6-007 | The 5G system shall be able to generate charging information that can differentiate between backhaul for the PRAS and other data traffic over the same access. |  |
| CPR 7.8-7 | PR 5.3.6-003 | The 5G system shall minimize service disruption for a UE that is moving between CPN access and operator provided mobile access. NOTE: CPN access can imply access via a PRAS or can imply access directly via an eRG. Operator provided mobile access implies access via an operator owned base station. |  |

## 7.9	Provisioning

Table 7.9-1 – RESIDENT Provisioning Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR 7.9-1 | PR 5.18.6-001 | The 5G system shall enable configuration and management of an eRG by both the network operator of the public (mobile) network the eRG is connected to and, within the boundaries defined by the operator, by an Authorised Administrator. |  |
| CPR 7.9-2 | PR 5.19.6-002 | The 5G system shall support the network operator to provide policies to the eRG on which transport (e.g. wireless, cable, etc.) is best suited for different negotiated QoS levels. NOTE: This may depend on BBF work |  |
| CPR 7.9-3 | PR 5.18.6-003 PR 5.12.6-002 PR 5.21.6-004 | The 5G system shall enable configuration and management of a PRAS by a network operator and, within the boundaries defined by the network operator, by an Authorised Administrator. Specifically the network operator shall be able to configure radio settings pertaining to licensed spectrum. Specifically the Authorised Administrator shall be able to configure visitor access (allowing all UEs, or allowing specific UEs only) |  |
| CPR 7.9-4 | PR 5.7.6-001 | The 5G system shall provide mechanisms for the network operator or an Authorised Administrator (e.g. a homeowner) to trigger remote provisioning of evolved residential gateways and for the network operator to perform remote provisioning of eRGs, which includes verification and configuration of evolved residential gateway identity and initial OA&M provisioning. |  |
| CPR 7.9-5 | PR 5.7.6-002 | The 5G system shall provide mechanisms for the network operator or an Authorised Administrator (e.g. a homeowner) to trigger remote provisioning of Premises Radio Access Stations and for the network operator to perform remote provisioning of PRASs, which includes verification and configuration of Premises Radio Access Station identity and initial OA&M provisioning. |  |
| CPR 7.9-6 | PR 5.21.6-003 | The 5G system shall support a secure mechanism for a network operator to provision a PRAS with an associated subscription, authentication credentials, and identity (e.g. for management purposes or to provide backhaul connectivity to the PRAS). |  |
| Alternative CPR 7.9-1 | PR 5.18.6-001 PR 5.19.6-002 PR 5.7.6-001 | The 5G system shall support a mechanism for the network operator to provision an eRG with: - policies on which transport (e.g. wireless, cable, etc.) is best suited for different negotiated QoS levels. - authentication credentials - identifier, - initial OA&M information, and - associated subscription |  |
| Alternative CPR 7.9-2 | PR 5.18.6-003 PR 5.12.6-002 PR 5.21.6-004 PR 5.7.6-001 | The 5G system shall enable the network operator to configure a PRAS with: - radio settings pertaining to licensed spectrum, - authentication credentials, - identifier, - initial OA&M information, and - associated subscription. |  |
| Alternative CPR 7.9-3 | PR 5.18.6-003 | The 5G system shall enable the Authorised Administrator to provision a PRAS with UE access considerations (allowing all UEs, or allowing specific UEs only) |  |
| Alternative CPR 7.9-4 | PR 5.7.6-001 | The 5G system shall provide a mechanism for the Authorised Administrator to trigger initial provisioning of an eRG (e.g. after installation or changing operator), which includes verification and configuration of an eRG identity. |  |
| Alternative CPR 7.9-5 | PR 5.7.6-002 | The 5G system shall provide a mechanism for the Authorised Administrator to trigger initial provisioning of a PRAS (e.g. after installation or changing operator), which includes verification and configuration of a PRAS identity. |  |

## 7.10	Premises Radio Access Station

Table 7.10-1 – RESIDENT Premises Radio Access Station Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR 7.10-1 | PR 5.1.6-001 PR 5.2.6-001 PR 5.2.6-001 PR 5.2.6-002 | The 5G system shall enable the network operator to provide any 5G services to any 5G UE via a Premises Radio Access Station (PRAS) connected via an evolved Residential Gateway (eRG). NOTE1: Whether the PRAS can be used by 5G UEs from other PLMNs in the same country as the PLMN associated with the PRAS is subject to regulatory policy on national roaming. |  |
| CPR 7.10-2 | PR 5.12.6-001 | The 5G system shall provide means to control which UEs can connect to a PRAS. |  |
| CPR 7.10-3 | PR 5.12.6-003 | The 5G system shall minimize service disruption when a CPN communication path changes between two PRASes. |  |
| CPR 7.10-4 | PR 5.14.6-001 | The 5G system shall be able to support PRAS sharing between multiple PLMNs. |  |
|  | PR 5.18.6-002 |  | Operation in Licensed and Unlicensed bands will be treated in descriptive text. |

## 7.11	5G-LAN

Table 7.11-1 – RESIDENT 5G-LAN Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR 7.11-1 | PR 5.8.6-001 | The 5G system shall be able to support large numbers of small 5G LAN-VNs. NOTE: Targeting residential deployments translate into millions of 5GLAN-VN per operator per country. These 5G LAN-VNs may contain between 10-50 devices per LAN |  |
| CPR 7.11-2 | PR 5.8.6-002 | The 5G system shall support a mechanism to enable authorized 3rd parties to authorize/deauthorize UEs to access a 5G LAN-VN. |  |
| CPR 7.11-3 | PR 5.9.6-001 | The 5G system shall support the use of an evolved Residential Gateway to connect 5G devices from the 5G LAN VN it belongs to with non-3GPP devices on an in-home LAN. |  |
|  | PR 5.16.6-001 |  | No consensus to include this requirement in consolidated requirements |

## 7.12	Broadcast Multicast

Table 7.12-1 – RESIDENT Broadcast Multicast Consolidated Requirements

|  | Potential Requirement |  |  |
| --- | --- | --- | --- |
| CPR No. | Original Potential Requirement No. | Potential Requirement text | Comments |
| CPR 7.12-1 | PR.5.20.6-001 PR.5.20.6-002 PR.5.20.6-003 PR. 5.15.6-001 | Under operator control, an eRG, shall be able to efficiently deliver 5G multicast/broadcast services to authorized UEs and non-3GPP devices in the CPN. NOTE: The multicast service(s) that each of the authorized UEs and/or non-3GPP devices is allowed to receive may be different. |  |