3GPP TSG-SA WG1 Meeting #94e	S1- 211111

Electronic Meeting, 10 May – 20 May 2021	(revision of S1-XXXX)

Source:	China Mobile，HUAWEI

Title:	FS_MMTELin5G Consolidated potential requirements

Spec:	3GPP TR 22.873 v0.5.0

Agenda item:	7.1.1

Document for:	Approval

Contact:	Hu Yue(huyueyjy@chinamobile.com), Alice Li (alice.li1@huawei.com)

- 1. Introduction

This document proposes text for TR 22.873 consolidated potential requirements. The text proposal need to be updated to take into account of the agreements in this meeting.

- 2. Reason for Change

Contribution with new text.

- 4. Proposal

It is proposed to agree the following changes to 3GPP TR22.873 v0.5.0.

* * * First Change * * * *

# 7	Consolidated potential requirements

- 7.1	Media handling capability

The IMS multimedia telephony service shall support synchronization between media/data streams belonging to the same MMTEL session.

The IMS multimedia telephony service shall support AR media processing.

- 7.2	Authorization and authentication

Requirements in this section are subject to regulatory requirements and operator policy. Examples of authorized/trusted 3rd party and its users are e.g. enterprise and enterprise employees.

[7.2-1] The 5G system shall provide means to allow a trusted third-party to update the MMTEL subscription and allocate a third-party specific identity to an authorized user.

NOTE 1: The third party is authorized to change user identities for those subscriptions authorized by the operator.

The following requirements apply to the originating side.

[7.2-2] The 5G network shall provide a means for third parties (e.g. enterprises) to be authorized to attest to the use of calling identity information by its authorized representatives.

[7.2-3] The 5G network shall provide a means for authorized third parties to attest that an authenticated representative is authorized to include or reference the pre-established calling identity information included in the call setup or retrieved by the called party.

[7.2-4] The 5G network shall provide a means to demonstrate the authenticity of the pre-established stored identity content that is referenced by the call setup and retrieved by the called party.

The following requirements apply to the terminating side.

[7.2-5] The 5G network shall provide a means for third parties (e.g. enterprises) to be able to securely verify the caller’s authorization to use the caller identity information either in addition to any verification by the terminating PLMN or instead of verification performed by the terminating PLMN (PLMN skips the verification).

[7.2-6] The 5G network shall provide a means to securely verify the authenticity of any stored identity content referenced by the call setup to be presented to the called party.