3GPP TSG-SA1 Meeting #95e	S1-213040r6

Electronic Meeting, 23 August - 2 September 2021	(revision of S1-213zzz)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0080 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex for smart grid |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, CEPRI, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEI |  |  |  |  |  | Date: |  |  | 2021-08-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CR 0082 Smart Grid-related communication KPIs is added into TS 22.104. This CR adds background information about the related use cases to clause A.4. Since this adds more advanced use cases to this clause, the title of clause A.4 is changed to “Electric-power distribution and Smart Grid”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Background information about the Smart Grid use cases in CR 0082 are added to clause A.4. References cited in this new addition are introduced to clause two. The title of clause A.4 is changed to “Electric-power distribution and Smart Grid”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Background information for about the use cases in CR 0082 is not included in TS 22.104. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, A.4, A.4.4, adding new clauses: A.4.6, A.4.7, A.4.8, A.4.9, A.4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 22.104 CR 0078, |  |  |
|  |  |  |  |  |  |  |  | TS 22.104 CR 0079 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************** Begin 1st Change *******************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	IEC 61784-3: "Industrial communication networks – profiles – part 3: functional fieldbuses – general rules and profile definitions".

[4]	BZKI, "Aspects of dependability assessment in ZDKI", June 2017.

[5] 	BZKI, "Requirement Profiles in ZDKI", 2017.

[6]	IEC 61158: "Industrial communication networks – fieldbus specification", 2014.

[7]	IEC 61907, "Communication network dependability engineering".

[8]	Richard C. Dorf and Robert H. Bishop, "Modern Control Systems", Pearson, Harlow, 13th Edition, 2017.

[9]	Ernie Hayden, Michael Assante, and Tim Conway, "An Abbreviated History of Automation & Industrial Controls Systems and Cybersecurity", SANS Institute, https://ics.sans.org/media/An-Abbreviated-History-of-Automation-and-ICS-Cybersecurity.pdf {accessed: 2017-05-23}, 2014.

[10]	IEC 61512 "Batch control - Part 1: Models and terminology".

[11]	RESERVE project, Deliverable D1.3, ICT Requirements, 
http://www.re-serve.eu/files/reserve/Content/Deliverables/D1.3.pdf, September 2017.

[12]	RESERVE project, Deliverable D1.2, Energy System Requirements 
http://www.re-serve.eu/files/reserve/Content/Deliverables/D1.2.pdf, September 2017.

[13]	G. Garner, "Designing Last Mile Communications Infrastructures for Intelligent Utility Networks (Smart Grids)", IBM Australia Limited, 2010.

[14]	B. Al-Omar, B., A. R. Al-Ali, R. Ahmed, and T. Landolsi, "Role of Information and Communication Technologies in the Smart Grid", Journal of Emerging Trends in Computing and Information Sciences, Vol. 3, pp. 707-716, 2015.

[15]	H. Kagermann, W. Wahlster, and J. Helbig, "Recommendations for implementing the strategic initiative INDUSTRIE 4.0", Final report of the Industrie 4.0 working group, acatech – National Academy of Science and Engineering, Munich, April 2013.

[16]	IEC 62443-3-2: "Security for industrial automation and control systems - Part 3-2: Security risk assessment and system design", in progress.

[17]	IEC 62657-2: "Industrial communication networks - Wireless communication networks - Part 2: Coexistence management", 2017.

[18]	IEC 62657-1: "Industrial communication networks – Wireless communication networks – Part 1: Wireless communication requirements and spectrum considerations".

[19]	IEEE Std 802.1Q "Media Access Control (MAC) Bridges and Virtual Bridge Local Area Networks".

NOTE: IEEE Std 802.1Qbv-2015 "Enhancements for Scheduled Traffic" has been included into IEEE Std 802.1Q-2018.

[20] 	IEEE, Use Cases IEC/IEEE 60802, 2018.

[21] 	"IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications in Bridged Local Area Networks--Corrigendum 1: Technical and Editorial Corrections," IEEE Std 802.1AS-2011/Cor 1-2013 (Corrigendum to IEEE Std 802.1AS-2011), pp. 1-128, Sept 2013.

[22] 	"IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications," IEEE Std 802.1AS-Rev/D7.3, pp. 1-502, August 2018.

[23]	3GPP TS 22.289: "Mobile Communication System for Railways".

[24]	IEEE P802.1CS: “IEEE Draft Standard for Local and metropolitan area networks - Link-local Registration Protocol”

[25]	IEEE P802.1Qdd: “IEEE Draft Standard for Local and Metropolitan Area Networks--Bridges and Bridged Networks -- Amendment: Resource Allocation Protocol (RAP)”

[26]	IEC/IEEE 60802: "Time-Sensitive Networking Profile for Industrial Automation".

[27]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)".

[28] 	IEC TR 61850-90-1:2010, Communication Networks and Systems for Power Utility automation – Part 90-1: Use of IEC61850 for the communication between substations.

[29]	5G DNA White Paper: "5GDN@Smart Grid White Paper: Requirements, Technologies, and Practices" https://www.5gdna.org/

***************** End of 1st  Change *******************

***************** Begin 2nd  Change *******************

A.4	Electric-power distribution and Smart Grid

***************** End of 2nd  Change *******************

***************** Begin 3rd  Change *******************

A.4.4	Distributed energy automation

A.4.4.1	Distributed automated switching for isolation and service restoration

A power distribution grid fault is a stressful situation. There are self-healing solutions for automated switching, fault isolation, and service restoration. Furthermore, these solutions are ideally suited to handle outages that affect critical power consumers, such as industrial plants or data centres. Supply interruptions must be fixed within less than a second for critical power consumers. Automated solutions are able to restore power supply within a few hundred milliseconds.

Figure A.4.4.1-1: Depiction of a distribution ring and a failure (flash of lighting)

The FLISR (Fault Location, Isolation & Service Restoration) solution consists of switch controller devices which are especially designed for feeder automation applications that support the self-healing of power distribution grids with overhead lines. They serve as control units for reclosers and disconnectors in overhead line distribution grids.

The system is designed for using fully distributed, independent automated devices. The logic resides in each individual feeder automation controller located at the poles in the feeder level. Each feeder section has a controller device. Using peer-to-peer communication among the controller devices, the system operates autonomously without the need of a regional controller or control centre. However, all self-healing steps carried out will be reported immediately to the control centre to keep the grid status up to date. The controllers conduct self-healing of the distribution line in typically 500 ms by isolating the faults.

Peer-to-peer communication via IEC 61850 GOOSE (Generic Object Oriented Substation Event) messages provides data as fast as possible (Layer 2 multicast message). They are sent periodically (in steady state, with changing interval time in fault case) by each controller to several or all other controllers of the same feeder and are not acknowledged.

The service bit rate per controller is low in steady state, but GOOSE bursts with high service bit rate do occur, especially during fault situations. GOOSE messages are sent by several or all controller units of the feeder nearly at the same point in time during the fault location, isolation and service restoration procedure with a low end-to-end latency.

The associated (a)periodic communication KPIs are provided in Table A4.4.1-1.

Table A.4.4.1-1: Service performance requirements for distributed automated switching for isolation and service restoration

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area (note 1) |
| 1 (note 2) | 99.999 9 | – | < 5 ms | 1 kbit/s (steady state) 1.5 Mbit/s (fault case) | < 1,500 | < 60 s (steady state) ≥ 1 ms (fault case) | transfer interval (one frame loss) | stationary | 20 | 30 km x 20 km |
| 2 | > 99.999 % | - | 20 ms (note 2) | - | < 100 | - | - | stationary | ≤ 100/km2 | several km2 |
| NOTE 1: Length x width NOTE 2: UE to UE communication (two wireless links) |  |  |  |  |  |  |  |  |  |  |

Use case one

GOOSE (a)periodic deterministic communication service supporting bursty message exchange for fault location, isolation, and service restoration.

Use case two

Typically event-driven, aperiodic deterministic communication service supporting fault detection and isolation.

A.4.4.2	Distributed automation without GOOSE

If the control of electrical power distribution components is performed from a central system entity, the controlled entities can be operated in a way that a controlled service restoration is possible without the use of GOOSE. Though this is not as effective as the communication has less strict requirements, this form of distribution automation is nevertheless effective, and it is also compliant with IEC standards and widely deployed in energy systems. The associated KPI is provided in Table A.4.4.2-1.

Table A.4.4.2-1: KPI for distributed automation without use of GOOSE

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| 1 | 99.999 | – | 100 ms to 2 s | 9.6 to100 | – | – | – | – | Concentrated rural: 70.8; Dispersed rural and semi-urban: 7.6; rural support: 0.048; urban: 11.0 | several km2 |

Use case one

Distributed automation without use of GOOSE using a centralized architecture.

A.4.4.3	Intelligent distributed feeder automation

Intelligent distributed feeder automation system which supported by 5G connections is designed to realize intelligent judgment, analysis, fault location, fault isolation and non-fault area power supply restoration operations. As illustrated in the Figure A.4.4.3-1, the distributed feeder automation system is mainly composed of a distribution master station, a distribution terminal, switch stations, and the communication system (UEs in the substations, 5G network, plus the data network). The distribution master station is mainly used for information gathering and human-computer interaction, and the distributed terminals are used for the collection of feeder status information and judgment, fault location, isolation, as well as power supply restoration based on this information. Distributed terminal actions are reported to the distribution master station. The 5G communication system enables communication among the distribution terminals. The distribution master station is usually connected to the 5G system via a data network, which is out of 3GPP scope.

Editor’s Note: 	The figure A.4.4.3-1 need to be further improved.

Figure A.4.4.3-1: Example of intelligent distributed feeder automation

The distribution master station manages multiple distributed terminals. Each distributed terminal is served by a 5G UE to exchange the collected data with other distributed terminals. From an application perspective, the communication between distributed terminals is peer-to-peer. The 5G communication service availability needs to be very high. Therefore, at least two communication links are usually deployed for hot standby or for transmitting data synchronously between two distributed terminals. The associated KPI is provided in Table A.4.4.3-1.

Table A.4.4.3-1: KPI for intelligent distributed feeder automation

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| 1 | 99.999 | – | # Normal: 1 s; Fault: 2 ms (note 2) | 2 M to10 M (note 1) | – | # Normal: 1 s; Fault: 2 ms (note 2) | – | – | 54/km2 (note 3) 78/km2 (note 4) | several km2 |
| NOTE 1: The KPI values are sourced from [29]. NOTE 2: It is the one-way delay from a distributed terminal to 5G network. NOTE 3: When the distributed terminals are deployed along overhead line, about 54 terminals will be distributed along overhead lines in one square kilometre. NOTE 4: When the distributed terminals are deployed in power distribution cabinets, there are about 78 terminals in one square kilometre. |  |  |  |  |  |  |  |  |  |  |

Use cases #1: Intelligent distributed feeder automation

A.4.4.4	High-speed current differential protection

High-speed current differential protection, which is required for sub-millisecond fault detection, is another typical use case of power distribution automation. The approach utilises differential current measurements to significantly reduce fault detection time. The protection relays exchange the current samples via the 5G system. Each relay then compares the sent and received samples to determine if a fault has occurred in a protected area. This is done in order to identify and isolate a fault in the grid. The sampling rate varies and is dependent on the algorithms designed by the manufacturers. A protection relay collects the current samples (with the typical message size of up to 245 bytes) at a frequency of 600 Hz, 1200 Hz, 1600 Hz, or 3000 Hz. The exchange of measurement samples is done in a strictly cyclic and deterministic manner. With the sampling rate of 600 Hz, the transfer interval is 1.7 ms and the required bandwidth 1.2 Mbit/s; for 1200 Hz, the transfer interval is 0.83 ms and the required bandwidth 2.4 Mbit/s. The maximum allowed end-to-end delay between two protection relays is between 5 ms and 10 ms, depending on the voltage (see  IEC 61850-90-1 for more details [28]). For some legacy systems, the latency usually is set to 15 ms. The associated KPIs are provided in Table A.4.4.4-1.

Table A.4.4.4-1: KPIs for high speed current differential protection

| Use case # | Communication service availability | End-to-end latency: maximum (note) | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | UE density [#/km2)] | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | > 99.999 % | 15 ms | 2.5 Mbit/s | < 245 | ≤ 1 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 2 | > 99.999 % | 15 ms | 1.2 Mbit/s | < 245 | ≤ 2 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 3 | > 99.999 % | 10 ms | 2.5 Mbit/s | < 245 | ≤ 1 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 4 | > 99.999 % | 10 ms | 1.2 Mbit/s | < 245 | ≤ 2 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 5 | > 99.999 % | 5 ms | 2.5 Mbit/s | < 245 | ≤ 1 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| 6 | > 99.999 % | 5 ms | 1.2 Mbit/s | < 245 | ≤ 2 ms | transfer interval (one frame loss) | stationary | ≤ 100/km2 | several km2 |
| NOTE : UE-to-UE communication. |  |  |  |  |  |  |  |  |  |

Use case #1: High-speed current differential protection with a sampling rate of 1200 Hz for legacy systems.

Use case #2: High-speed current differential protection with a sampling rate of 600 Hz for legacy systems.

Use case #3: High-speed current differential protection with a sampling rate of 1200 Hz under voltage condition 1 (see  IEC 61850-90-1[28] for more details).

Use case #4: High-speed current differential protection with a sampling rate of 600 Hz under voltage condition 1 (see  IEC 61850-90-1[28] for more details).

Use case #5: High-speed current differential protection with a sampling rate of 1200 Hz under voltage condition 2 (see  IEC 61850-90-1[28] for more details).

Use case #6: High-speed current differential protection with a sampling rate of 600 Hz under voltage condition 2 (see  IEC 61850-90-1[28] for more details).

***************** End of 3rd  Change *******************

***************** Begin 4th  Change *******************

A.4.6	Distributed energy storage

Distributed power generation includes various power sources such as solar, wind, fuel cells, and gas. Distributed power generation typically comes with a low power density and entails thus typicall a decentralised deployment. Decentralisation causes technical problems and challenges Smart Grid operators. When distributed power generation is connected to the elctrical grid, the energy flow becomes more complicated as the user often is both an electricity consumer and producer (a so-called “prosumer”). Therefore, the current in the electricity grid can change direction at different locations of the grid and at different times of the day.

The information exchange in a distributed energy grid does not only include power-generation-related data, but also control commands for the distributed energy storage equipment. An example for such a command is “change the load characteristics to realise a flexible electricity grid” etc.

Figure A.4.6-1 Example of a distributed-energy storage grid

Figure A.4.6-1 shows an example of distributed-energy storage grid. The distributed-energy storage grid needs to exchange information among the distributed-energy storage management platform (DESMP) and distributed-energy devices (DEDs).

The DED is a plug-and-play device and periodically collects its energy information, such as battery energy, charge and discharge status, energy alarm information, etc. The DED then transfers this information via 5G UE to the DESMP. The DESMP regularly manages the DEDs, e.g., the DESMP monitors the DEDs working status, controls the DEDs working modes, or configures the DEDs energy parameters etc. The associated KPIs are provided in Table A.4.6-1 and Table A.4.6-2.

Table A.4.6-1: periodic communication service performance requirements ‒ data for distributed energy storage

| Use case# | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| 1 | > 99.9 % |  | DL: < 10 ms UL: < 10 ms | UL: > 16 Mbit/s (urban); 640 Mbit/s (rural) DL: > 100 kbit/s (note 1) | UL: 800 kbyte | UL: 10 ms | – | – | > 10/km2 (urban); > 100/km2 (rural) (note 2) | several km2 |
| 2 | > 99.9 % |  | DL: < 10 ms UL: < 1 s | UL: > 128 kbit/s (urban); 10.4 Mbit/s (rural); DL: > 100 kbit/s (note 1) | UL: 1.3 Mbyte DL: > 100 kbyte | UL: 1000 ms | – | – | > 10/km2 (urban); > 100/km2 (rural) (note 2) | several km2 |
| NOTE 1: Service bit rate for one energy storage station. NOTE 2: Activity storage nodes/km2. This value is used for deducing the data volume in an area that features multiple energy storage stations. The data volume can be calculated with the following formula (current service bit rate per storage station) x (activity storage nodes/km2) + (video service bit rate per storage station) x (activity storage nodes/km2). |  |  |  |  |  |  |  |  |  |  |

Use case#1: Distributed energy storage ‒ periodic communication for monitoring

Use case#2: Distributed energy storage ‒ periodic communication for data collection

Table A.4.6-2: Aperiodic communication service performance requirements ‒ video for distributed energy storage

| Use case# | Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability | Communication service reliability: mean time between failures | Max Allowed End-to-end latency (note) | Service bit rate: user-experienced data rate | Message size [byte] | Survival time | UE speed | # of UEs | Service Area |
| 1 | > 99.9 % | – | DL: < 10 ms UL:< 1 s (rural) | DL: > 100 kbit/s UL: > 5 Gbit/s (note 1) | – | – | stationary | > 100 | several km2 |
| NOTE: The downlink user experienced data rate is calculated as follows: 12.5 Mbytes/s x 50(containers) x 8 = 5 Gbit/s |  |  |  |  |  |  |  |  |  |

Use case#1: Distributed energy storage ‒ aperiodic video communication

A.4.7	Advanced metering

Instead of recording and sending metering data from a wired electricity meter unit, electricity metering collecting can be executed by a UE-integrated smart meter unit. Smart meter units can send real-time metering data to a server in the utility through mobile networks. In this way, the power enterprise―based on the analysis of the user’s power consumption behavior―gives the user power consumption suggestions, which fosters the user’s power consumption and energy saving habits.

The electric smart meters monitor relevant user energy status and deliver the status data to a measurement data management system (MDMS). The MDMS sends control commands according to its policy and the status of the data collected from the smart meters. The MDMS commands include tripping, closing permission, alarm, alarm release, power protection, and power protection release. Accurate-fee control is one of the basic services of advanced metering. When the electric power user doesn’t pay her electric fee on time, the MDMS can cut off the power supply. And when there is a need for temporary power supply for this user, the MDMS can recover the power supply. This operation requires real-time interaction between the electric smart meter and the MDMS. Due to massive number of electricity meters, it is estimated that in the near future, the amount of this kind of interaction will increase 5 to 10 times. The associated KPIs are provided in Table A.4.7-1.

Table A.4.7-1: Communication KPI for advanced metering

| Use case# | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| 1 | > 99.99 |  | Accuracy fee control: < 100 (note 1); General information data collection: < 3000 | UL: < 2 M DL: < 1 M | – | – | – | stationary | < 10 000/km2 (note2) | – |
| NOTE 1: One-way delay from 5G IoT device to backend system. The distance between the two is below 40 km (city range). NOTE 2: It is the typical connection density in today city environment. With the evolution from centralised meters to socket meters in the home, the connection density is expected to increase 5 to 10 times. |  |  |  |  |  |  |  |  |  |  |

Use case#1: Advanced metering

A.4.8	Smart distribution transformer terminal

A smart distribution transformer terminal is usually deployed in a distribution transformer area. The terminal can support multiple energy applications simultaneously. Multiple kinds of energy data are collected by the terminal and then delivered to a energy application platform. Figure A.4.8.1-1 illustrates a workflow example for a smart distribution transformer terminal.

Figure A.4.8-1: Example of a smart distribution transformer terminal workflow

In general, the connections between the smart distribution transformer terminal and the energy application platform are provided by the 5G system. The connections between energy end equipment and smart distribution transformer terminal may be provided by 5G system. In this case, about 300 to 500 energy end devices are connected to one smart distribution transformer terminal. The average service bit rate between the smart distribution transformer terminal and an energy end device is more than 2 Mbit/s in uplink for each application. The related communication distance is between 100 m and 500 m. The associated KPI is provided in Table A.4.8-1.

Table A.4.8-1: Key Performance for Smart Distribution Transformer Terminal

| Use case# | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| 1 | >99.99% | – | 10 ms, 100 ms, 3 s (note 2) | > 2 Mbit/s (note 1) | – | – | – | – | 500 in the service area (note 3) | Communication distance is from 100 m to 500 m, (outdoor, indoor, and deep indoor) |
| NOTE 1: It is the smart metering application data rate between the Smart Distribution Transformer Terminal and energy end equipment. Once there are multiple smart grid applications, it is required more data rate. NOTE 2: It depends on different applications supported by the Smart Distribution Transformer Terminal. The less the latency is, the more applications can be supported. NOTE 3: The distribution area can be calculated as 3.14 x range2 and in general is between 0.031 km2 and 0.79 km2. |  |  |  |  |  |  |  |  |  |  |

Use case#1: smart distribution transformer terminal

A.4.9	Distributed energy resources and micro-grids

Distributed energy resources (DER) become increasingly important. The potentially large number of DERs will have an impact on security, stability, and operation efficiency of the energy grid.

The integration of DERs into the energy grid poses many challenges for the involved communication system. To incorporate more renewable and alternative energy sources, the communication infrastructure must have the ability to easily handle an increasing amount of data traffic or service requests and must provide a real-time monitoring and control operation for these distributed energy resources. A reliable communication between the DERs is crucial.

When it comes to communications architecture, IEC 61850 is a widely used standard for automation and equipment of power utilities and DER, specifically for defining protocols for IEDs (Intelligent electronic devices) at electrical substations The IEC 61850 standard specifies the timing constraints for messages typically used in substations. GOOSE (Generic Object Oriented Substation Events) and SV (Sampled Values) messages are assumed as time critical messages. They have the tightest deadlines (maximum allowed transfer time) among all IEC 61850 messages, corresponding to 3 ms. While GOOSE is typically used for transfering information related to monitoring and control functions (circuit breaker status etc.), SV is used for transfering measurement samples of current and voltage signals. The SV protocol works on a periodic information transmission model, sending messages at a fixed rate. For protection purposes, the default rate is 4000 or 4800 messages per second for 50 and 60 Hz power systems, respectively. On the other hand, the GOOSE protocol operates in a sporadic information transmission model, where a continuous flow of data is maintained to increase communication reliability. The typical sizes of GOOSE and SV messages are160 and 140 bytes, respectively. GOOSE messages are transmitted in two different modes: (1) safe operation: 1 message per second (service bit rate = 1.28 kbit/s); (2) emergency operation: 32 messages per second (service bit rate = 41.0 kbit/s). SV messages are transmitted at much higher rate, namely 4800 messages per second (service bit rate = 5.4 Mbit/s). The associated KPIs are provided in Table A.4.9-1.

Table A.4.9-1: Key Performance for Distributed energy resources (DER): using SV (Sampled Values) message

| Use case# | Characteristic parameter |  |  |  | Influence quantity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed |
| 1 | 99,9999 % | – | < 3 ms | 4.5 Mbit/s | 140 | ≤ 1 ms | transfer interval | stationary |
| 2 | 99,9999 % | – | < 3 ms | 5.4 Mbit/s | 140 | ≤ 1 ms | transfer interval | stationary |
| 3 | > 99.9999 % | – | < 3 ms | – | 160 | – | – | stationary |
| NOTE: UE to UE communication is assumed. |  |  |  |  |  |  |  |  |

Use case#1: Distributed energy resources and micro-grids: using SV(sample value) message with 50Hz

Use case#2: Distributed energy resources and micro-grids: using SV(sample value) message with 60Hz

Use case#3: Distributed energy resources and micro-grids: using GOOSE message

A.4.10   Ensuring uninterrupted communication service availability during emergencies

During emergencies, public mobile land networks (PLMNs) may restrict network access, which may lead to a prohibitevly low communication service availability for machine-type communication (MTC) for Smart Grid applications. An example is communication for microgrids. Microgrids are separate parts of a power grid that can be controlled and operated individually in a so-called island mode, or together with other parts of the power grid. The idea is to prioritise Smart Grid-related communication in order to ensure reliable and available communication for selected devices during emergency conditions. Existing features of a mobile network can be used to differentiate MTC of devices in a microgrid from other kind of MTC traffic or human-to-human communication. These features can help these microgrid devices to have communication service during emergencies. The communication among the microgrid devices enables co-ordination of DERs, which help the DERs can autarkically implement recovery of an islanded microgrid.

The associated KPI is provided in Table A.4.10-1.

Table A.4.10-1: Key Performance for uninterrupted MTC service availability

| Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value | Communication service reliability: mean time between failures | Max Allowed End-to-end latency (note 1; (note 2) | Service bit rate: user-experienced data rate (note 2) | Message size [byte] | Survival time | UE speed | # of UEs | Service Area |
| 99.999 9 % | – | 100 ms | < 1 kbit/s per DER | – | – | Stationary | – | – |
| NOTE 1: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 2: It applies to both UL and DL unless stated otherwise. |  |  |  |  |  |  |  |  |

***************** End of 4th Change *******************

***************** End of Changes *********************