3GPP TSG-SA1 Meeting #95e	S1-213073r1

Electronic Meeting, 23 August – 2 September 2021	(revision of S1-213073)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.153 | CR | 0049 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MPS when access to EPC/5GC is WLAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, T-Mobile US, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS_WLAN |  |  |  |  |  | Date: |  |  | 2021-08-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current requirements in TS.22.153 do not explicitly cover MPS when the UE or IoT device has WLAN (non-3GPP) access to the EPC/5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additions of a new clause with normative requirements to support: - MPS for MMTEL voice/video calls when the UE has WLAN access to the EPC/5GC, and - MPS for DTS sessions when the UE or IoT device has WLAN access to the EPC/5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of MPS features when the UE or IoT device has WLAN access to the EPC/5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2 and new clause 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START of 1st CHANGE***

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DTS 	Data Transport Service

MMTEL 	Multimedia Telephony

MPS 	Multimedia Priority Service

OTT 	Over-The-Top

PIN 	Personal Identification Number

VPN 				Virtual Private Network
NSWO			Non-Seamless WLAN Offload

*** END of 1st CHANGE***

*** END of 2nd CHANGE***

# 10	Non-3GPP access

## 10.1	MPS when access to the EPC/5GC is WLAN

MPS can be supported for MMTEL voice/video calls and data sessions when the access to the EPC/5GC is WLAN and the 3GPP system is used for the MPS authorization.  In this case, MPS priority treatment can be provided by:

-	both the WLAN access and the EPC/5GC (i.e., when the WLAN supports MPS), or

-	only the EPC/5GC (i.e., when the WLAN does not support MPS).

The following subclauses provide requirements specific to WLAN access to EPC/5GC.

### 10.1.1	MPS for MMTEL voice and video

#### 10.1.1.1	General

For a UE with WLAN access to the EPC/5GC, the system shall support the requirements in clauses 5-9 for MMTEL voice and voice conference calls and MMTEL video and video conference calls by an authorized MPS Service User using

-	a UE with a subscription for MPS, and

-	a UE that does not have an MPS subscription.

The system shall support the requirements in clauses 5-9 for MPS for MMTEL voice and voice conference calls and MPS for MMTEL video and video conference calls terminating to a UE with WLAN access to the EPC/5GC regardless of the terminating UE subscription for MPS.

#### 10.1.1.2	Authentication and authorization

For a UE with a 3GPP subscription for MPS and with WLAN access to the EPC/5GC, the system shall support MPS for MMTEL voice/video authorization based on the UE subscription information.

For a UE that does not have a 3GPP subscription for MPS and with WLAN access to the EPC/5GC, the system shall support MPS for MMTEL voice/video authorization based on Service User credentials not associated with the UE (e.g., a calling card number, PIN or security token).

#### 10.1.1.3	WLAN Interworking

For a UE with WLAN access to the EPC/5GC, the 3GPP system shall support delivery of the following to the WLAN:

-	indication of MPS for MMTEL voice/video authorization,

-	priority marking of transport parameters of MPS for MMTEL voice/video signalling and media, and

-	QoS characteristics of MPS for MMTEL voice/video signalling and media.

### 10.1.2	MPS for DTS

#### 10.1.2.1	General

For a UE or IoT device with WLAN access to the EPC/5GC, the system shall support the requirements in clauses 5-9 on MPS for DTS sessions for:

-	an authorized Service User using a UE with a subscription for MPS,

-	an authorized Service User using a UE that does not have an MPS subscription,

-	an authorized IoT device with a subscription for MPS, and

-	an authorized enterprise network activation of MPS for DTS for remote MPS subscribed UEs/IoT devices associated with the enterprise network.

##### 10.1.2.2	Authentication and authorization

For a UE or IoT device with a 3GPP subscription for MPS and with WLAN access to the EPC/5GC, the system shall support MPS for DTS authorization based on the UE subscription information.

For a UE that does not have a 3GPP subscription for MPS and with WLAN access to the EPC/5GC, the system shall support MPS for DTS authorization based on Service User credentials not associated with the UE (e.g., a calling card number, PIN or security token).

For an authorized enterprise network that is activating MPS for DTS for a remote UE or IoT device with a 3GPP subscription for MPS, the system shall support MPS for DTS authorization based on the UE or IoT device subscription information.

#### 10.1.2.3	WLAN interworking

For a UE or IoT with WLAN access to the EPC/5GC, the 3GPP system shall support delivery of the following to the WLAN:

-	indication of MPS for DTS authorization,

-	priority marking of transport parameters of MPS for DTS signalling and media, and

-	QoS characteristics of MPS for DTS signalling and media.

### 10.1.3	Non-seamless WLAN Offload (NSWO)

Non-Seamless WLAN Offload (NSWO) can be used by operators to offload selected traffic to a WLAN based on operator policy and roaming agreements for WLAN access (i.e., WLAN access to an EPC). A UE or IoT device with a 3GPP subscription for MPS can be authorized for priority NSWO data fraffic (e.g., Internet traffic).

NOTE:	In the NSWO case, the only aspects in scope are those pertaining to use of the 3GPP system for verification of 3GPP subscription for MPS.

For a UE or IoT device with a 3GPP subscription for MPS and with WLAN access to the EPC/5GC, the system shall support delivery of indication of MPS authorization based on operator policies and roaming agreements.

*** END of 2nd CHANGE***

*** END of CHANGE***