3GPP TSG-SA WG1 Meeting #95e 	S1-213096r02

Electronic Meeting, 23 August – 2 Sept. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.874 | CR | 0001 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FS_AMMT update to Functional requirements for AMMT services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMMT |  |  |  |  |  | Date: |  |  | 2021-08-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR22.874, some of the CPRs in clause 8.2 may needs to be further clarified/improved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | update in clause 8.1 of FS_AMMT TR22.874 for updating the Funactional requirement for AMMT services |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wording is not clarified/improved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************** Start of Change ******************

8.2	Functional requirements for AMMT services

Editor’s Note: 	This section summarizes the available requirements from the various approved use cases based on v0.3.0. The requirements in the tables may be corrected/modified aligned with the newly-added use-cases and the updates to the approved use-cases.

Table 8.2-1 Functional requirements for AMMT services

| Use case | Potential Requirements | Clean-up Steps Taken | Consolidated Potential Requirements |
| --- | --- | --- | --- |
| Session-specific model transfer split computation operations | [P.R.5.5-001] Based on operator policy, the 5G network shall provide the means to allow an authorized third-party to monitor the resource utilisation of the network service that are associated with the third-party. | propose to adopt it as CPR. | [CPR-037] Based on operator policy, the 5G network shall provide the means to allow an authorized third-party to monitor the resource utilisation of the network service that is the third-party. NOTE: Resource utilization in the preceding requirement refers to measurements relevant to the UE’s performance such as the data throughput provided to the UE. |
|  | [P.R.5.5-002] Based on operator policy, the 5G system shall be able to expose QoS information to an authorized 3rd party. The QoS information can include e.g. UE UL/DL bitrate, latency, reliability per location. | propose to adopt is as CPR with some rewording | [CPR-038] Based on operator policy, the 5G system shall be able to provide an indication about a planned change of bitrate, latency, or reliability for a QoS flow to an authorized 3rd party so that the 3rd party AI/ML application is able to adjust the application layer behaviour if time allows. The indication shall provide the anticipated time and location of the change, as well as the target QoS parameters. |
|  | [P.R.5.5-003] The 5G system shall be able to provide the means to predict and expose network condition changes (e.g. bitrate, latency, reliability) to the authorized third party. | Propose to adopt it with some rewording | [CPR-039] 5G system shall be able to provide the means to predict (to the extent possible) and expose predicted network condition changes (i.e. bitrate, latency, reliability) to the authorized third party. |
| AI model management as a Service | [P.R.6.4-001] Subject to user consent, operator policies and the regional or national regulatory requirements, the 5G system shall be able to provide the capability to expose information (e.g. measured data rate, delay, network analytics results) to an authorized third-party application to support the training and monitoring of the AI/ML models. | The two PRs are proposing to make 5GS provide information including measured data rate, delay, analytics result, and prediction for communication to 3rd party, for AI model downloading and training. It is proposed to merge the two PRs “Prediction” part can be merged into CPR-009 The information mentioned in the two PRs require a fast notification especially when the local application server applies. So it is proposed to add a new CPR. | [CPR-040] Subject to user consent, operator policy and regulatory constraints, the 5G system shall support a mechanism to expose monitoring and status information in the form of data rate, communication service availability, communication service reliability, packet error rate and/or delay measurement to a 3rd party AI/ML application. NOTE: such mechanism is needed for AI/ML application to determine an in time transfer of AI/ML model [CPR-0x1] The 5G system shall be able to support notifying information for AI/ML services to local 3rd party application directly (i.e. a fast notification to the authorized 3rd party rather than sending the notification to centralised server and then being distributed to local server) NOTE: Such mechanism is needed for AI/ML application to receive the notification in-time. |
| Prediction of AI/ML model distribution | [P.R.6.7-001] Subject to user consent, operator policy and regulatory constraints, the 5G system shall support the provision of monitoring information or analytics information to a trusted 3rd party AI/ML server for allowing this 3rd party AI/ML server to make a prediction for a suitable AI/ML model to be downloaded to the concerned UE. |  |  |
| Data Transfer Disturbance in Multi-agent multi-device ML Operations | [PR.7.3-001] 5G system shall provide a suitable means for a learning agent of the AI/ML operations service to use when providing a 5GS using AI/ML application with information (e.g., required increase in data rate due to changes in AI/ML operation, etc. requested by AI/ML application (e.g., of the learning agent or of UE)) necessary to minimize or avoid the impact of communication disruption on AI/ML service/performance. | It is proposed to adopt the requirement but the description should be simplified/re-written. |  |
|  | [PR.7.3-002] 5G system shall provide a suitable means for a UE to provide the AI/ML application of a learning agent with information (e.g., experienced data rate, communication disturbance, geographical location to the extent of jurisdiction (e.g., State level, country level), etc.) necessary to minimize or avoid the impact of communication disruption on AI/ML service/performance. | In this case, the 5GS seems to be a transmission pipe for information transmission between UE and Application. More clarification on 5GS impact may be needed. |  |
|  | [PR.7.3-003] 5G system shall provide a means to supply prediction info (e.g., on traffic congestion, the related geographical area/spot) so that a UE or learning agent can minimize the impact of learning data transfer disturbance. | It is proposed to adopt it with the modification “learning agent –> 3rd party” and rewording | [CPR-041] 5G system shall provide a means to supply an authorized 3rd party with event alerting with a predicted time of the event. (e.g., event alerting with predictive time about traffic congestion or moving into/out of a different geographical area). NOTE: A 3rd party AI/ML application may use the prediction information to minimize disturbance in the transfer of learning data and AI/ML model data. |
|  | [PR.7.3-004] 5G system shall be able to adaptively support scheduling (e.g., for network resource utilization for the communication between the 5G system and AI/ML application of a learning agent) in response to dynamic change (increase or decrease) in AI/ML traffic demand due to switching to a different AI/ML mode. [PR.7.3-004a] The 5G system shall be able to obtain from the AI/ML application of a learning agent the information about traffic demand increase or decrease (e.g. start/stop task splitting, change task splitting point) by that application (if available). | propose to adopt it with rewording |  |
| Group Performance “Flocking” Use Case | [P.R.7.4-001] The 5G system shall be able to support ‘aggregated performance’ for a group of UEs where the worst performing member defines the performance of the entire group. E.g. the 5G system could achieve performance for the entire group so as to avoid members achieving either significantly less or more performance than others in the group. | propose to adopt it as CPR with some rewording | [CPR-042] The 5G system shall be able to support changing the QoS policy associated with UE that is part of a synchronous FL group with group-based management, or adjusting the group membership (i.e. adding or removing group members), with the considerations of location, user preference, mobility prediction and/or capability of each group member based on requests from 3rd party. NOTE: the performance of a synchronous FL group depends on proper in-time transfer of AI/ML model data by all group members. |
|  | [P.R.7.4-002] The 5G system shall be able determine whether a required QoS for each member in a group can be maintained. | It is proposed to adopt the three potential requirements into CPRs | [CPR-0x2] The 5G system shall be able to guarantee a required QoS for each member in a group can be maintained. [CPR-0x3] The 5G system shall be able to expose QoS information for a group of UEs to an authorized service provider. [CPR-0x4] The 5G system shall be able to support charging exemption or rewarding for the data transfer for ML or Federated learning, based on an agreement between MNO and 3rd party. |
|  | [P.R.7.4-003] The 5G system shall be able to expose QoS information for a group of UEs to an authorized service provider. |  |  |
|  | [P.R.7.4-004] The 5G system shall be able to support charging exemption or rewarding for the data transfer for ML or Federated learning, based on an agreement between MNO and 3rd party. |  |  |

************* End of Change ***************