3GPP TSG-SA WG1 Meeting #95-e	S1-213098r3

Electronic Meeting, Aug 23 – Sept 2, 2021	(revision of S1-212006)

Title:	FS_TACMM: Resolving the Editor’s Notes on the use case Haptic feedback for a personal exclusion zone

Agenda Item:	7.12.1

Source:	InterDigital

Contact:	Atle Monrad <atle dot monrad at interdigital dot com>

Discussion

This document proposes resolving the Editor’s Notes on the use case “Haptic feedback for a personal exclusion zone”. The new format of the KPI table is introduced.

Proposal

It is proposed to modify the below use case to remove one of the Editor’s Notes in clause 5.5 in 3GPP TR 22.847 v0.3.0.

* * * *   First Change   * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T, “Technology Watch Report: The Tactile Internet”, August 2014.

[3]	O. Holland et al., “The IEEE 1918.1 “Tactile Internet” Standards Working Group and its Standards,” Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[4]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications".

[5]	S. K. Sharma, I. Woungang, A. Anpalagan and S. Chatzinotas, "Toward Tactile Internet in Beyond 5G Era: Recent Advances, Current Issues, and Future Directions," in IEEE Access, vol. 8, pp. 56948-56991, 2020

[6] 	3GPP TS 22.261: " Service requirements for the 5G system ".

[7] 	Kwang Soon Kim, et al., " Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[8]	"SAE J3186: Application Protocol and Requirements for Maneuver Sharing and Coordinating Service", version 0.28, developing document, Apr. 2021.

[9] 	SAE J3224, "V2X Sensor-Sharing for Cooperative & Automated Driving" version 0.06, developing document, Apr. 2021.

[10] 	M. During and K. Lemmer, “Cooperative maneuver planning for cooperative driving,” IEEE Intell. Transp. Syst. Mag., vol. 8, no. 3, pp. 8–22, Jul. 2016.

[11]	D. Soldani, Y. Guo, B. Barani, P. Mogensen, I. Chih-Lin, S. Das, "5G for ultra-reliable low-latency communications". IEEE Network. 2018 Apr 2;32(2):6-7.

[12]	O. Dubuisson and P. Fouquart, "ASN.1: Communication between Heterogeneous Systems", Morgan Kaufmann Publishers Inc., San Francisco, CA, USA, 2001.

[13]	IEEE SA, P1918.1 - Tactile Internet: Application Scenarios, Definitions and Terminology, Architecture, Functions, and Technical Assumptions, https://standards.ieee.org/project/1918_1.html

[14] 	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[15]	 K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[16]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[17] 	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[18]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[19] 	M. Back et al., "The virtual factory: Exploring 3D worlds as industrial collaboration and control environments," 2010 IEEE Virtual Reality Conference (VR), 2010, pp. 257-258

[20] 	S. Schulte, D. Schuller, R. Steinmetz and S. Abels, "Plug-and-Play Virtual Factories," in IEEE Internet Computing, vol. 16, no. 5, pp. 78-82, Sept.-Oct. 2012

[bb]	Arnon, Shlomi, et al. "A comparative study of wireless communication network configurations for medical applications." IEEE Wireless Communications 10.1 (2003): page 56-61.

## * * * *   Next Change   * * * *

### 5.5.6	Potential New Requirements needed to support the use case

[PR. 5.5.6-1] The 5G network shall support a mechanism to allow an authorized 3rd party to provide QoS policy for flows of multiple UEs associated with an application. The policy may contain e.g. the expected 5GS handling and the associated triggering event.

[PR. 5.5.6-2] The 5G system shall support a mechanism to apply QoS policy for flows of multiple UEs associated with an application received from an authorized 3rd party.

[PR 5.1.6-3] The 5G system shall provide a network connection to address the KPIs for immersive multi-modal navigation applications, see Table 5.5.6-1.

Table 5.5.6-1: Potential Key performance requirements for a personal exclusion zone in dangerous remote environments.

| Use Cases | Characteristic parameter (KPI) |  |  |  |  |  | Influence quantity |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency (ms) |  | Service bit rate: user-experienced data rate |  | Reliability |  | Message size (byte) |  | # of UEs | UE Speed |  | Service Area |  |  |
| Immersive multi-modal navigation applications Remote Site Local Site (DL) | 0-50 [11], [12] |  | 1-4k pkts/s (w/o compression) 100-500 pkts/s, (w/ compression.) |  | [99.999 %] |  | 1 DoF: 2 to 8 10 DoF: 20 to 80 100 DoF: 200 to 800 |  | - | Stationary or Pedestrian |  | TBD |  | Haptic feedback |
|  | <400 [11] |  | 1-100 Mbit/s |  | [99.999 %] |  | 1500 |  | - | Workers: Stationary/ or Pedestrian, |  | TBD |  | Video |
|  | <150 [11] |  | 5-512 kbit/s |  | [99.9 %] |  | 50 |  | - | Stationary or Pedestrian |  | TBD |  | Audio |
|  | <300 |  | 600 Mbit/s |  | [99.9 %] |  | MTU |  | - | Stationary or Pedestrian |  | TBD |  | VR |
| Local Site Remote Site (UL) | <300 |  | 12 kbit/s [bb] |  | [99.999 %] |  | 1 500 |  |  | Stationary or Pedestrian |  | TBD |  | Biometric / Affective |
|  | <400 [11] |  | 1-100 Mbit/s |  | [99.999 %] |  | 1 500 |  | - | Workers: Stationary/ or Pedestrian, UAV: [30-300mph] |  | TBD |  | Video |
|  | <150 [11] |  | 5-512 kbit/s |  | [99.9 %] |  | 50 |  | - | Stationary or Pedestrian |  | TBD |  | Audio |
|  | <300 |  | 600 Mbit/s |  | [99.9 %] |  | MTU |  | - | Stationary or Pedestrian |  | TBD |  | VR |

Editor’s Note:	KPIs for personal exclusion zone in dangerous remote environments may need further discussion and verification.

[PR 5.5.6-4], The 5G system shall support the following synchronization thresholds to support immersive multi-modal navigation applications, see Table 5.5.6-2.

Table 5.5.6-2: Potential Key performance requirements for synchronization thresholds for a personal exclusion zone in dangerous remote environments.

| synchronisation threshold |  |  |
| --- | --- | --- |
| audio-tactile | audio delay: [50 ms] | tactile delay: [25 ms] |
| visual-tactile | visual delay: [15 ms] | tactile delay: [50 ms] |

* * * *   End of Change   * * * *