3GPP TSG-SA WG1 Meeting #95-e 	S1-213103

Electronic Meeting, 23 Aug –2 Sep 2021	(revision of S1-21xxxx)

Source:	China Mobile

pCR Title:	pCR on consolidated requirements of FS_TACMM

Draft Spec:	3GPP TR22.847

Agenda item:	7.12

Document for:	Approval

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com

Abstract: This pCR provide the consolidated functional requirements of FS_TACMM

1. Reason for Change

In TR22.847, several functinal requirements have reached consensus in the former meeting, include the following ones:

| PR number | Description | Consolidate action |
| --- | --- | --- |
| PR 5.6.6-1 | The 5G system shall support a mechanism to allow an authorized 3rd party to provide QoS policy for flows of multiple UEs associated with an application. The policy may contain e.g. the expected 5GS handling and the associated triggering event. | Create a new requirement: [CPR-1] The 5G system shall support a mechanism to allow an authorized 3rd party to provide QoS policy for flows of multiple UEs associated with an application. The policy may contain e.g. the expected 5GS handling and the associated triggering event. |
| PR 5.6.6-2 | The 5G system shall support a mechanism to apply QoS policy for flows of multiple UEs associated with an application received from an authorized 3rd party. | Create a new requirement: [CPR-2] The 5G system shall support a mechanism to apply QoS policy for flows of multiple UEs associated with an application received from an authorized 3rd party. |
| PR 5.7.6-1 | The 5G system shall support a mechanism to ensure users’ QoE of the multi-modal communication service involving one or multiple devices at either end of the communication. QoE refers to the difference of the physical interaction across the 5G network and the same manipulation carried out locally. | Create a new requirement: [CPR-3] The 5G system shall support a mechanism to ensure users’ QoE of the multi-modal communication service involving one or multiple devices at either end of the communication. QoE refers to the difference of the physical interaction across the 5G network and the same manipulation carried out locally. |
| PR 5.7.6-2 | The 5G system shall support a mechanism for a 3rd party application server to provide real-time feedback on the traffic characteristics and service requirements of the multiple streams of a multi-modal communication session. | Create a new requirement: [CPR-4] The 5G system shall support a mechanism for a 3rd party application server to provide real-time feedback on the traffic characteristics and service requirements of the multiple streams of a multi-modal communication session. |
| PR 5.7.6-3 | The 5G system shall support a mechanism to assist the synchronisation between the multiple streams (e.g., haptic, audio and video) of a multi-modal communication session in order to avoid the negative impact on the user experience. | Create a new requirement: [CPR-5] The 5G system shall support a mechanism to assist the synchronisation between the multiple streams (e.g., haptic, audio and video) of a multi-modal communication session in order to avoid the negative impact on the user experience. |

And there are two more functional requirements are for FFS:

| PR number | Description | Consolidate action |
| --- | --- | --- |
| PR 5.8.6-1 | 5G system shall be able to support the interaction with applications on UEs or data flows grouping information within one tactile and multi-modality communication service. | Create a new requirement: [CPR-6] 5G system shall be able to support the interaction with applications on UEs or data flows grouping information within one tactile and multi-modality communication service. |
| PR 5.8.6-2 | 5G system shall be able to provide a dynamic mechanism to transfer different data flows with different latency to achieve a certain transmission time difference within one tactile and multi-modality communication service. | Merge into [CPR-1] as latency requirements can be carried by QoS policy: [CPR-1] The 5G system shall support a mechanism to allow an authorized 3rd party to provide QoS policy for flows of multiple UEs associated with an application. The policy may contain e.g. the expected 5GS handling and the associated triggering event. |

2. Proposal

It is proposed to agree the following changes to 3GPP TR22.847.

* * * First Change * * * *

# 6	Consolidated requirements

## 6.1	Functional requirements

[CPR-1] The 5G system shall support a mechanism to allow an authorized 3rd party to provide QoS policy for flows of multiple UEs associated with an application. The policy may contain e.g. the expected 5GS handling and the associated triggering event, expected synchronization thresholds between the multiple streams (e.g., haptic, audio and video)..

[CPR-2] The 5G system shall support a mechanism to apply QoS policy for flows of multiple UEs associated with an application received from an authorized 3rd party.

[CPR-3] The 5G system shall provide support for IEEE P1918.1 architecture to ensure users’ QoE of the multi-modal communication service involving one or multiple devices at either end of the communication. QoE refers to the difference of the physical interaction across the 5G network and the same manipulation carried out locally.

[CPR-4] In support of IEEE P1918.1 architecture, the 5G system shall enable a mechanism for a 3rd party application server to provide real-time feedback on the traffic characteristics and performance requirements of the multiple streams of a multi-modal communication session.

NOTE: Real-time feedback is one of the functions enabled in the tactile service manager (TSM) (logical entity defined in the IEEE P1918.1 architecture), which allow the application layer to send information to the network domain about the certain events, typically when the actual QoS perceived by the application layer does not meet the requirements.

[CPR-6] 5G system shall be able to support the interaction with applications on information about which UEs or data flows grouping belong to one tactile and multi-modality communication service.

* * * End of Change * * * *