3GPP TSG-SA WG1 Meeting #95-e 	S1-213106r2

Electronic Meeting, 23 Aug –2 Sep 2021	(revision of S1-21xxxx)

Source:	China Mobile

pCR Title:	pCR on updating KPI table of remote control robot use case

Draft Spec:	3GPP TR22.847

Agenda item:	7.12

Document for:	Approval

Contact: 		Xiaonan Shi, shixiaonan@chinamobile.com

Abstract: This pCR proposed to update the KPI table of remote control robot use case based on the same format.

* * * First Change * * * *

## 5.2	Remote control robot

### 5.2.1	Description

Human can use Remote control robot to operate some actions which they are not able to be on the spot. With real-time and synchronous visual, audio and haptic feedback, remote robot operator will perform reactions suitable for the situation, and remote control robot can follow operator’s action to do the exact work. This can be used in many different scenarios. It can be applied to remote care for elderly, remote detonation, remote operation, remote maintenance of facility, remote firefighting, etc.

### 5.2.2	Pre-conditions

Alice is the operator of a remote robot for maintaining underground pipe. She has a sticker which has the same Dof and structure with the maintain tool on the remote robot. Both Alice and robot is connected to a 5G network with the ability to transfer video, audio and haptic information.

### 5.2.3	Service Flows

- Remote robot is at the spot of a damaged underground pipe, using integrated camera and sensor to send back video, audio and haptic information;

- Alice hold the control stick and can receive the haptic information, and receive the video, audio information synchronous on the screen and from the sound.

- After analysing these information, Alice perform the next move on the operator sticker.

- The haptic information including force and Dof transfer to remote robot, and the robot performs the same action.

### 5.2.4	Post-conditions

Alice can remotely control the robot to finish the maintaining work.

### 5.2.5	Existing features partly or fully covering the use case functionality

In TS 22.263, there are requirements for supporting video, imaging and audio for professional applications.

### 5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-1] 5G system shall be able to support tactile and multi-modality communication service with following KPIs.

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  | Remarks |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area |  |  |
| Remote control robot | 1-20ms in air latency | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | [99.99%] | - | high-dynamic | TBD | Haptic feedback |  |
|  | 20-100ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | [99.99%] | - | Stationary or Pedestrian | TBD | Haptic feedback |  |
|  | 5 ms | 1-100 Mbit/s | [99.9%] | - | Stationary or Pedestrian | TBD | Video |  |
|  | 5 ms | 5-512 kbit/s | [99.9%] | - | Stationary or Pedestrian | TBD | Audio |  |
|  | 5 ms | < 1Mbit/s | [99.999%] | - | Stationary or Pedestrian | TBD | Sensing information |  |
| Editor note: Coordination of different modalities need FFS. |  |  |  |  |  |  |  |

[PR 5.2.6-2] The 5G system shall support a mechanism to allow an authorized 3rd party to provide QoS policy for flows of multiple UEs associated with an application. The policy may contain e.g. the expected 5GS handling and the associated triggering event.

* * * Next Change * * * *

### 5.4.6	Potential New Requirements needed to support the use case

[PR 5.4.6-1] 5G system shall be able to support real-time multimodality communication for interactive haptic control and feedback with KPIs as summarized in Table 5.4.6-1.

Table 5.4.6-1: Multi-modality communication service performance requirements.

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  | Remarks (NOTE 1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency (NOTE 2) | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area |  |
| Skillset sharing low- dynamic robotics (including teleoperation) Controller to controlee | < [5-10] | 0.8 - 200 kbit/s (with compression) | [99,999%] | - | Stationary or Pedestrian | TBD | Haptic (position, velocity) |
| Skillset sharing low- dynamic robotics (including teleoperation) Controlee to controller | < [5-10] | 0.8 - 200 kbit/s (with compression) | [99,999] | - | Stationary or Pedestrian | TBD | Haptic feedback |
|  | < 10 | 1-100 Mbit/s | [99,999] | - | Stationary or Pedestrian | TBD | Video |
|  | < 10 | 5-512 kbit/s | [99,9] | - | Stationary or Pedestrian | TBD | Audio |
| Highly dynamic/ mobile robotics Controller to controlee | < [1-5] | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | [99,999] (with compression) [99,9] (w/o compression) | - | high-dynamic | TBD | Haptic (position, velocity) |
| Highly dynamic/ mobile robotics Controlee to controller | < [1-5] | 0.8 - 200 kbit/s | [99,999] (with compression) [99,9] (w/o compression) | - | high-dynamic | TBD | Haptic feedback |
|  | < 1-10 | 1-10 Mbit/s | [99,999] | - | high-dynamic | TBD | Video |
|  | < 1-10 | 100-500 kbit/s | [99,9] | - | high-dynamic | TBD | Audio |
| NOTE 1: Haptic feedback is typically haptic signal, such as force level, torque level, vibration and texture. NOTE 2: The latency requirements are expected to be satisfied even when multimodal communication for skillset sharing is via indirect communication (i.e., relayed by another intermediate node). |  |  |  |  |  |  |  |