3GPP TSG-SA WG1 Meeting # 95e	S1-213074

# Electronic Meeting,  23 August  2021 - 02 September 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0560 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing PALS Normative Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Kyonggi University |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | PALS |  |  |  |  |  | Date: |  |  | 2021-08-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adding service requirements for PALS into TS 22.261 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new section, 6.X. capturing PALS normative requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PALS service requirements will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========= First Change ==========

## 6.X Providing Access to Local Services

### 6.X.1	Description

Providing access to local services refers to the capability to provide access to a network and a set of services offered by local service providers, the network operator or third parties. The access may be local and bounded in time and geographic location. The user may become aware of the available local access and services, and the process to gain and terminate access to the network (and service) should be efficient, and convenient from a user experience standpoint.

Providing access to local services creates new opportunities for users and service providers. For example, access can be provided in areas where there is no other MNO’s coverage (for example, on a fairground established far from other infrastructure), or the access and local services operation can be established as needed (on a short-term basis), without the need for long term business relationships, permanently installed equipment, etc.

This type of local service access can be promoted and arranged through different channels. Principally the local service operators (e.g. brick and mortar businesses, entertainment venues, construction contractors, first responder agencies, etc.) will provide information and proper incentive or instructions to potential users so that they will seek to access the local services.

### 6.X.2	Requirements

#### 6.X.2.1	General

In the requirements below, it is assumed that:

- Both the home network and the hosting network can be a PLMN or NPN.

- Only subscribers of a public network can roam into a PLMN.

#### 6.X.2.2	Configuration of Localized Services in Hosting Network

The 5G system shall support suitable mechanisms to allow automatically establishing localized service agreements for a specific occasion (time and location) and building temporary relationship among hosting network operator and other service operators including network operators or 3rd party service providers.

The 5G system shall support means for the service operator to request the hosting network via standard mechanisms to provide access to 3rd party services at a specific period of time and location. This period of time shall be flexible, so that a change in service provision can be decided at any time (e.g., to cancel or prolong local services in the locality of service delivery) based on localized services agreements.

Based on localized services agreements, the 5G system shall provide suitable means to allow the service operator to request and provision various localized service requirements, including QoS, expected/maximum number of users, event information for discovery, network slicing, required IP connectivity etc, and routing policies for the application of the localized services via the hosting network.

The 5G system shall support means for a hosting network to create policies and configure resources for the requested time and location for the 3rd party services based on the received request.

The 5G system shall support means for a hosting network to notify the service operator of the accepted service parameters and routing policies.

Subject to regulatory requirements and localized service agreements, the 5G system shall allow a home network operator to automatically negotiate policies with the hosting network for allowing the home network’s subscribers to connect at a specific occasion, e.g., time and location, for their home network services.

Subject to the automatic localized services agreements between the hosting network operator and home network operator, for UE with only home network subscription and with authorization to access hosting networks the 5G system shall support:

- access to the hosting network and use home network services or selected localized services via the hosting network,

- seamless service continuity for home network services or selected localized services when moving between two hosting networks or a host network and the home network.

The 5G system shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of their spatial extent (in terms of a particular topology, for example a single cell), as specified by a 3rd party.

The 5G system shall support a mechanism to enable configuration of a network that provides access to localized services such that the services can be limited in terms of the resources or capacity available, to correspond to requirements that apply only to the locality of service delivery, as specified by a 3rd party.

The 5G system shall support means for a hosting network to provide a 3rd party service provider with information for automatic discovery of the hosting network by the UEs to allow access to specific 3rd party services.

#### 6.X.2.3	User Manual Selection of Localized Services via Hosting Network

The hosting network shall allow a UE to manually select temporary localized services which are provided via local breakout at the hosting network.

NOTE 1: localized services are provided via local breakout at the hosting network based on interworking scenarios for hosting network owned/collaborative services.

The 5G system shall support a mechanism to allow a user to manually select a specific local hosting network.

#### 6.X.2.4	UE Configuration, Provisioning, Authentication and Authorization

Subject to localized services agreements, the 5G system shall enable a home network operator to authorize a UE for using its home network services via a hosting network for a certain period of time and/or location.

The 5G system shall allow a trusted 3rd party to provide UEs with localized service policy (e.g. QoS, network slice in the hosting or home network, service restriction such as time and location) via the hosting network or the UE’s home network.

The 5G system shall enable a UE to use credentials provided by the hosting network with or without coordination with the home network of the UE, to make use of localized services via the hosting network with a certain time (including starting time and the duration) and location validity.

The 5G system shall be able to allow the home network to steer its UE(s) to a hosting network with the consideration of the location, times, coverage of the hosting network and services offered by the home network and hosting network.

#### 6.X.2.5	UE Discovery, Selection and Access

Subject to operator’s policy and agreement between a 3rd party service provider and operator, the 5G system shall enable a UE to receive and use configuration provided by a 3rd party service provider  to discover and access a hosting network and localized services, including the considerations of prior service agreement with a 3rd party service provider and no prior subscription to hosting network. If the UE is able to obtain services from two networks simultaneously, it may additionally select the hosting network. If the UE cannot maintain the connection to the home network while selecting the hosting network, the selection shall only be done on request by the user, i.e., using manual selection.

The 5G system shall support secure means for a UE to select and access localized services which may be provided by a 3rd party service provider via a hosting network, independent of prior subscription to the hosting network or 3rd party service provider.

The 5G system shall enable the home network to instruct a UE to automatically select a hosting network for accessing localized services when specified conditions (e.g. predefined time, location) are fulfilled.

The 5G system shall be able to prevent a UE to re-access the hosting network after the UE terminated the localized services if the authorization for the localized services is no longer valid (e.g., can be based on certain conditions such as time or location of the user).

#### 6.X.2.6	Hosting Network Localized Services and Home Operator Services

The 5G system shall enable the home network operator to indicate to the UE what services are preferred to be used from the home network when the UE connects to a hosting network and the requested service is both available from the hosting and the home network.

Based on localized service agreements, the hosting network shall be able to provide required connectivity and QoS for a UE simultaneously connected to the hosting network for localized services and its home network for home network services.

A UE shall be able to connect to its home network via the hosting network, if supported by the hosting network and the home network based on localized service agreements.

#### 6.X.2.7	Returning to Home Network

The 5G system shall provide mechanisms to mitigate user plane and control plane overload caused by a high number of UEs returning from a temporary local access of a hosting network to their home network in a very short period.

The 5G system shall provide mechanisms to minimize the impact on the UEs communication e.g.  to prevent user plane and control plane outages when returning to a home network with other high number of UEs in a very short period, after terminating their temporary local access to a hosting network.

#### 6.X.2.8	Charging

The 5G system shall be able to collect charging information for the use of localized services at the hosting network and provide the charging records to UEs’ home operators based on localized service agreements and charging policies provided by the localized service providers.

#### 6.X.2.9 	Regulatory Services

A hosting network using the 5G system shall be able to support regulatory services (e.g. PWS, LI, and emergency calls), based on regional/national regulatory requirements.

#### 6.X.2.10	Multicast/Broadcast

The operator of a hosting network shall support a mechanism allowing different local service providers and content providers to disseminate their services and content over broadcast/multicast transport. This mechanism should also provide a means to include diverse content in the same transmission, e.g., to include advertisements with other content, or to include multiple content in the same media delivered to the user.

A hosting network shall provide multicast and broadcast services in an energy efficient manner to UEs receiving this service.

A hosting network shall support resource efficient content delivery through multicast/broadcast.

A hosting network shall support a mechanism to provide low latency signalling for efficient content delivery  to many UEs.

Subject to home operator policy, a hosting network shall be able to prioritize specific multicast and broadcast services for local access over home routed access, even if the same service is available in both networks.

========= End of Change ==========