3GPP SA WG1 Meeting #97e 	S1-220077r1

Electronic Meeting, 14 - 24 Feb 2022	(revision of xx-yyxxxx)

# Source:		China Telecom

# Title:			New SID on Relay Network for Extended Coverage

# Document for:		Approval

# Agenda Item:		4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Relay Network for Extended Coverage

Acronym:	FS_RelayNEC

Unique identifier:	xxxxx

Potential target Release: Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 720005 | New Services and Markets Technology Enablers (SMARTER) | Previous normative work includes one-hop relay |
| 840034 | Enhanced Relays for Energy Efficiency and Extensive Coverage (REFEC) | Pervious normative work includes multi-hops relay |

Dependency on non-3GPP (draft) specification:

None

3	Justification

With the development of radio technologies, cellular systems are expected to move to higher frequency bands. If the same deployment of base stations is kept, there will be white spots that have no cellular coverage. One of the cost-effective solutions is using relay UEs to cover the white spots. Operators could reduce their investment in infrastructure, the user could get a better user experience, and relay owners could get the reward from sharing their devices as relays.

Comparing with IAB nodes, relay UEs have several advantages. Relay is easy to install. Everyone could provide their UE as relay UE. No radio cell planning is needed for operators. Also, operators are looking at how to provide the incentive scheme for the relay UE owners.

Communication could have high reliability and high data rate with the multi-path relay. If the relay in the one-hop relay or one of the relays in the multi-hop relay is down or out of coverage, the connection would be failed. With the multi-path relay, the connection between the remote UE and gNB has multiple paths. Within each path, there could be one-hop relays or multi-hop relays. If one of the relays is down or moves, the data could either transmit through other paths or other available devices nearby. Moreover, with multiple paths transmitting simultaneously, the user could have a higher data rate.

It is noted that in TS 22.261, service requirements for multi-hop relaying and different traffic flows of a remote UE to be relayed via different indirect paths are specified. However, the existing requirements does not cover the cases for network-assisted scenarios of different traffic flows of a remote UE through different paths, the service requirements for multi-hop multi-path relay of network-assisted path selection and path switching are not covered. Moreover, the service requirement does not cover the case for transmitting same traffic flows via different indirect paths and to distinguish the redundant traffic in this case.

As the relay would move from time to time, service continuity is sometimes hard to ensure when the relay UEs are moving. Stationary relay UE is a way to enhance reliability for relay network connection. Stationary relay UE is the relay UE that is static when it is active and has a continuous power supply. Prioritization could be placed to stationary relay UE because it won’t have the power shortage problem and it is stationary for a long period. For example, stationary relay UE could be placed at the coffee shop in the city centre. The shop owner could set his phone as a relay UE at the shop for the passer-by and the customers. The shop owner could plug in the charger for his phone and earn the reward based on the duration of being the relay or on the amount of data being transmitted or received, etc. As a result, it is crucial to select the stationary relay UE with prioritization for reliability reason.

Operators would like to encourage people to set up their own stationary relay UEs, they would need the charging information including the identification of the stationary relay UE to apply different charging policies for the stationary relay UEs and the normal mobile relay UEs.

Relay networks under operator control could be more secured. Relay networks should be secured so that people would have confidence in sharing their UEs as relays and using others’ UEs as relays for a better user experience.

In conclusion, there are not still gaps on multi-hop multi-path relays. This study will investigate the use cases and potential requirements to support the enhancement of relays, including multi-hop multi-path relays and the stationary relays.

4	Objective

The objectives of this study are to define:

- Use cases and identify corresponding requirements, including:

  - Multi-hop multi-path relay connection, including network-assisted path selection and path switching;

  - Same traffic flows of a remote UE to be relayed via different indirect network connections paths;

  - Stationary relay UE, including relay UE selection, provisioning, etc.;

- Other aspects, including charging and security;

- Gap analysis between the identified requirements and the existing requirements.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.XXX | Study of Relay Communication for Extended Coverage | TSG#98 (Dec 2022) | TSG#99 (Mar 2023) | Yuying Zhang, China Telecom, zhangyy45@chinatelecom.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A | N/A | N/A | N/A |

6	Work item Rapporteur(s)

Yuying Zhang, China Telecom, zhangyy45@chinatelecom.cn

7	Work item leadership

SA1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom

CATT

ZTE

AT&T