3GPP TSG-SA1 Meeting #98e	S1-221238

Electronic Meeting, 09 – 19 May 2022	(revision of S1-221134r1)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.280 | CR | 0152 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call restriction based on subparts/elements of functional alias |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2022-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Existing call restriction is based on source and destination MCX User identities (including functional alias(es)). It is not allowed to deny/permit calls based on subpart/elements of functional alias(es) of MCX Users. An example of a Railway functional alias of an MCX User is a specific driver on a specific train assigned to a particular Railway organization. An example of a subpart/element of a functional alias of MCX Users is their role e.g., all train drivers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed changes allow call restriction based on subparts/elements of functional aliases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Call restriction for Railway needs are not fully covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9a.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

### 5.9a.2	Requirements

[R-5.9a-001] The MCX Service shall provide a mechanism for the MCX User to activate one or more functional alias(es) for use by the MCX Users within that MCX Service system (i.e., the primary MCX Service system of the functional alias).

[R-5.9a-001a] The MCX Service system shall be able to use the functional alias as a unique identifier for MCX Users as Participant of an MCX Service Group, e.g., to display a functional alias as speaker identification or for the use in group member lists.

[R-5.9a-001b] The MCX Service system shall provide a mechanism for an MCX User to assign a functional alias for use on an MCX Service Group.

[R-5.9a-001c] The MCX Service system shall provide a mechanism for an MCX User to assign an activated functional alias for an outgoing MCX Private communication.

[R-5.9a-002] The MCX User shall be reachable by its functional alias(es) from within the MCX Service system where the functional alias was activated.

[R-5.9a-002a] An MCX User shall be reachable by its functional alias from the primary MCX system while migrated to partner MCX Service systems.

[R-5.9a-003] The MCX Service shall provide a mechanism for the MCX User to deactivate a functional alias.

[R-5.9a-004] The MCX Service shall upon request provide the MCX User a list of functional aliases from which the user can select for activation/deactivation.

NOTE: 	The list may contain functional aliases based on a certain context, like location of the MCX User, operational schedule, etc.

[R-5.9a-005] The MCX Service shall provide a mechanism for an MCX Service Administrator to create, delete, manage functional aliases, and for each functional alias indicate either if it can be simultaneously active for multiple MCX Users up to a per-alias configurable number, or if it is allowed to be taken over by an authorized MCX User, or none of these two options.

[R-5.9a-006] If an MCX User attempts to activate a functional alias that is already active for another MCX User, and is not allowed to be simultaneously active for multiple MCX Users or the number of simultaneous MCX Users is reached to the upper limit, the MCX Service shall inform the MCX User that the functional alias is already in use.

[R-5.9a-007] If an MCX User attempts to activate a functional alias that is already active for at least one other MCX User, and that functional alias is allowed to be simultaneously active for multiple MCX Users and the upper limit of number of simultaneous MCX Users is not reached, the MCX Service shall activate the functional alias for the MCX user and inform all other MCX User(s) with the same functional alias.

[R-5.9a-008] If an authorized MCX User attempts to activate a functional alias that is already used by another MCX User, and that functional alias is allowed to be taken over, and is not indicated for simultaneous activation to multiple MCX Users, the MCX Service shall offer the MCX User to take over the functional alias from the MCX User using the alias.

[R-5.9a-008a] If an authorized MCX User attempts to activate a functional alias that is already active for at least one other MCX User, and the upper limit of number of simultaneous MCX Users is reached, the MCX Service shall reject the MCX User’s request.

[R-5.9a-009] If an authorized MCX User takes over the functional alias that is already active for another MCX User, the MCX Service shall activate the functional alias to the MCX User and inform the previous MCX User that the alias has been deactivated.

[R-5.9a-010] The MCX Service shall allow the MCX User to perform an activation/deactivation of an unlisted functional alias that is defined in the MCX Service system.

[R-5.9a-011] An authorized MCX User shall be able to interrogate the MCX Service system of the alias(es) active for a certain MCX User.

[R-5.9a-012] The MCX Service shall provide a mechanism for an MCX Service Administrator to authorize a MCX User to activate, deactivate, interrogate and take over a functional alias.

[R-5.9a-013] VOID

[R-5.9a-014] The MCX Service shall allow the functional alias to be structured as a combination of organizationally meaningful elements (e.g., user.agency@organization.country, role.mission@department.company, function.equipment.ID@city, label1.label2.label3@firstname.familyname).

[R-5.9a-015] The MCX Service system shall allow an MCX Service Administrator to make use of information (e.g., operational schedules, locations, velocity or direction) from external sources to create or delete a functional alias.

[R-5.9a-016] The MCX Service system shall allow the MCX Service Administrator to assign a communication priority for a functional alias.

[R-5.9a-017] The MCX Service system shall allow the MCX Service Administrator to assign a time limit to a functional alias after which the functional alias will be deactivated.

[R-5.9a-018] The MCX Service shall support automatic activation and de-activation of a functional alias based on the operational criteria (e.g., MCX User ID, login/logoff from the MCX Service system, specific external information supplied by external systems).

[R-5.9a-019] The MCX Service shall provide a mechanism for an MCX Service Administrator to define a functional alias with related geographic areas that can be associated to MCX Users for the purpose of routing Location dependent communications, as part of handling MCX Service Private Communication requests, when the determination of the receiving party is based on the initiating MCX User’s current Location.

[R-5.9a-020] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure for a particular functional alias or a specific element or combination of specific elements in the functional alias, a set of functional aliases under the same authority to which a Private Communication (without Floor control) can be made by an MCX User registered to this particular functional alias.

[R-5.9a-021] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure for a particular functional alias or a specific element or combination of specific elements in the functional alias, a set of functional aliases under the same authority from which Private Communication (without Floor control) is allowed to an MCX User registered to this particular functional alias.

[R-5.9a-022] For private calls to functional aliases the authorizations in clause 5.9a.2 shall replace the authorizations defined in clause 6.7.3.

[R-5.9a-023] The MCX Service system shall support private calls and private emergency calls to a functional alias simultaneously activated by multiple MCX Users.

[R-5.9a-024] The MCX Service system shall enable an MCX Service Administrator to select the mechanism to be used for handling of private calls to functional aliases that are allowed to be simultaneously active for multiple MCX Users.

[R-5.9a-025] The MCX Service system shall be able to provide a mechanism to inform the MCX User about the functional alias activation/deactivation status (e.g., activation/deactivation accepted, activation/deactivation pending, activation/deactivation rejected).

[R-5.9a-026] The MCX Service shall provide the functional alias(es) of the transmitting MCX Service Group Member that enter the communication late in accordance with sub-clause 5.3.

[R-5.9a-027] The MCX Service shall provide a Location information report for every authorized MCX User associated with one functional alias in accordance with sub-clause 6.12.

[R-5.9a-028] The MCX Service shall provide the functional alias(es) associated with the MCX Service User ID, of the transmitting Participant to the receiving MCX Users (UEs) unless the transmitting Participant's identity is restricted in accordance with sub-clause 6.4.3.

[R-5.9a-029] The MCX Service shall provide, upon request, the list of currently affiliated members of an MCX group which encompasses the MCX Service User ID and the associated functional alias(es) of each member in accordance with sub-clause 6.4.5.

[R-5.9a-030] The MCX Service shall provide a mechanism for the enforcement of uniqueness of a functional alias within a Mission Critical organisation in accordance with sub-clause 6.9.

[R-5.9a-031] The MCX Service shall provide a mechanism for an authorized MCX User to obtain a list of functional aliases currently activated in the system either periodically or as a onetime request.

NOTE: 	The mechanism may allow for filtering criteria e.g., geographic area.

* * * End of first change * * *