### 10.1.2	Domains priority and selection for UE attempts to emergency call

#### 10.1.2.1	Voice and GTT only

A UE that is connected to a domain in which it is possible for the UE to make non emergency calls using the particular media requested by the user, shall use that domain to attempt an emergency call unless serving network policy (based on regulatory requirements and operator needs) requires the UE, including an unauthenticated UE, to attempt the emergency call on a specific domain first.

If the UE is connected to more than one domain in which it is possible for the UE to make non emergency calls using the particular media requested by the user, the UE shall attempt an emergency call on the same domain it would use to originate a non-emergency call using the same media unless serving network policy (based on regulatory requirements and operator needs) requires the UE, including an unauthenticated UE, to attempt the emergency call on a specific domain first.

In the case where an emergency call attempt by a UE fails, the UE should automatically make a second attempt on the other domain if the UE supports it.

If the user aborts the emergency call setup during the subsequent automatic attempt and immediately tries to set up an emergency call again, then the UE shall immediately attempt in the domain in which the user aborted the emergency call.

Where the 5G system supports remote UE access to emergency calls via relay UEs, the following requirements apply:

-	The UE shall first attempt to make an emergency call via any available network. If the UE fails to find a network that allows emergency calls, the UE shall attempt an emergency call via a relay UE.

-	The 5G system shall treat on-going emergency calls and emergency call attempts by relay UEs and remote UEs with all the priority treatment requirements and capabilities defined for direct UE to PLMN emergency calls.

#### 10.1.2.2	Other media

The following applies in addition to 10.1.2.1.

If an emergency call attempt that includes a request for both (i) voice and/or GTT and (ii) other media cannot be supported or fails in all connected access types in the PS CN domain, the UE shall attempt the emergency call in the CS domain if available and shall only include the request for voice and/or GTT.