3GPP TSG SA WG 1 Meeting 98e		 S1-221052r2

Electronic Meeting, 9 – 19 May 2022

Title:	[Draft] Reply LS on emergency service support over ProSe Relays

Response to:	S2-2203130 from SA2

Release:	Release 18

Work Item:	FS_5G_ProSe_Ph2

Source:	Apple [SA1]

To:	SA2

Cc:	-

Contact person:	Mona Mustapha at Apple dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 CR0641 to TS22.261, CR0xxx to TS22.101

# 1	Overall description

SA1 thanks SA2 for their LS in S2-2203130.

SA1 has agreed the attached CRs in connection with SA2 questions as indicated below. In one CR, we add the following new requirement:

Subject to local regulatory requirements, the UE shall first attempt to make an emergency call via any available network. If the UE fails to find a network that allows emergency calls, the UE shall attempt an emergency call via a relay UE.

Q1: When a 5G ProSe UE-to-Network Relay offers emergency service support, does it imply that the ProSe UE-to-Network Relay needs to be responsible for Remote UE's emergency service according to local regulation, as indicated in TS 22.101, i.e. to support Remote UEs without SIM/USIM/ISIM, or for valid Ues only?

SA1 response to Q1:

SA1 notes that the requirement in TS22.101 below applies to all UEs, regardless if indirect network connection or direct network connection is used:

"Emergency calls shall be supported by the UE without a SIM/USIM/ISIM being present."

Q2:  Since the 5G ProSe UE-to-Network Relay may be roaming, under which conditions would it be required that the emergency call is routed to PSAP of the same country as the Remote UE?

SA1 response to Q2:

Refer to the attached CR to TS 22.261.

Based on local regulatory requirements, where the 5G system supports remote UE access to emergency services via relay UEs, the 5G system shall route emergency services sessions to an appropriate PSAP for the location of the remote UE, which can be in the same country of the remote UE or not.

Q3: Should the 5G ProSe UE-to-Network Relay and Remote UE be required to belong to the same PLMN?  For example, in case of network sharing where Remote and Relay Ues may be connected via different CN PLMNs.

SA1 response to Q3:

SA1 interprets that 'belong to' means that the remote and relay UEs are connected to the same CN PLMN. In this case, there is no restriction on whether the 5G ProSe UE-to-Network Relay and Remote UE need to be connected to the same CN PLMN. How the call is connected towards the PLMN is not defined in Stage 1, as long as emergency call capabilities are maintained during and after the emergency call in the same way as for a direct UE to network emergency call. For network sharing, the UEs may be connected to different Core Networks.

Q4: While a Relay UE is providing emergency connectivity services support for a Remote UE, if the Relay UE needs to make emergency access of its own, what if any requirements for the Relay UE to continue to provide emergency access to the Remote UE?  SA2 assumes that a Relay UE may provide emergency service access to only one Remote UE at any time.

SA1 response to Q4:

Refer to the attached CR to TS 22.101.

Where the 5G system supports remote UE access to emergency services calls via relay UEs, the following requirements apply:

-	The 5G system shall treat on-going emergency calls and emergency call attempts by relay UEs and remote UEs with all the priority treatment requirements and capabilities defined for direct UE to PLMN emergency calls.

The requirements only cover relative priority between normal calls and emergency call.

# 2	Actions

To SA2:

ACTION: 	SA1 asks SA2 to take the above clarification into account.

# 3	Dates of next TSG SA WG 1 meetings

SA1#99e	22 Aug – 1 Sep 2022		Electronic Meeting

SA1#100	07-11 Nov 2022			TBD, North America (fallback: Electronic Meeting)