3GPP TSG-SA WG1 Meeting #98e 	S1-221100R4

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	CATT, China Unicom

pCR Title:	Pseudo-CR on use case of security for non-N2  sharing network

Draft Spec:	3GPP  TR 22.851 V.0.0.0

Agenda item:	7.5

Document for:	Approval

Contact:	Hui Xu xuhui@catt.cn

Abstract: This clause describes the sharing scenarios and potential security requirements for non-N2 sharing network in TR22.851

1. Introduction

The use case of non-N2 sharing network is an important use case in the network sharing. The contribution describes the  use case and potential requirements of security for non-N2 sharing network scenario.

2. Reason for Change

Update the “Use Cases” section 5 of the new TR 22.851.

3. Conclusions

It is proposed to put the use case into the TR 22.851.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.851.

* * * First Change * * * *

5 Use cases

5.A	Use case of security and topology hiding

5. A.1	Description

If the connection between radio access network and home core network is not always available, it is inevitable to use connection between operators to achieve interoperability. In order to avoid increasing technical complexity, the principle is to avoid modifying more new interfaces, which makes it easier for operators to reach an agreement on network interconnection. However, there is a need to consider information security between different networks.

5. A.2	Pre-conditions

It is assumed that:

1. OP 1 is the Hosting Operator of a shared access network.

2. OP 2 is a Participating Operator in the shared access network.

3. The shared access network does not have a direct connection with the core network of OP 2.

4. UE 2 is a subscriber of OP 2.

5. A.3	Service Flows

OP 1

5G Network

topology hiding

UE 2 of  OP2

OP 2

5G Network

topology hiding

UE 2 of OP2

UE 2 moves between OP2 and OP1

Figure 5.A.3-1: Network security scenario between the shared access and the core network of the participating operator.

It is assumed that the Hosting Operator and Participating Operator support mobility between the two networks, then:

1.   UE 2 moves from OP2’s network to the shared network of OP1.

2.   Information about UE 2 needed for UE movement may be transferred from OP2’s network to OP1’s network.

3.   Both OP1 and OP2 want the connection between networks to be under security protection.

4.   UE 2 returns to OP2’s network from the shared network of OP1;

5.  Information about UE 2 needed for UE movement may be transferred from OP1’s network to OP2’s network when the UE returns to OP 2.

6.   Both OP1 and OP2 want the connection between networks to be under security protection when UE 2 returns to the home coverage area;

Some information related to serving network elements may be transferred through the connection between networks during an active communication, for example, IP address and network element identification.

The necessary information of border element between networks of Hosting RAN operator and Participating Operator are known in order to serve the normal network connection. At the same time, the non-border element information of one operator’s network should be hidden from the other operator’s network to as large extent as possible.

5. A.4	Post-conditions

The communication of  UE 2 succeed, when UE 2 moves between shared access network and Participating Operator’s network.

5. A.5	Existing feature partly or fully covering use case functionality

None

5. A.6	Potential New Requirements needed to support the use case

[PR 5.A.6-001] The 5G system shall be able to ensure security protection between networks of Hosting RAN operator and Participating Operators when a user crosses the border of the shared network.

Editor’s note: The term “security protection” in this requirement needs to be clarified before approval

[PR 5.A.6-002] The 5G system shall minimize sharing of information related to non-border network elements, e.g. Geographical location, network topology etc, between the Hosting RAN operator and Participating Operator.

Editor’s note: This requirement needs to be revisited before approval