3GPP TSG SA WG1 Meeting #98e	S1-220145r4

# Electronic Meeting, May 9-19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.011 | CR | 0340 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on PLMN search for FPLMN Registered UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT |  |  |  |  |  | Date: |  |  | 2022-05-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 has discussed the questions in an LS from CT1 to SA1 (S1-221176/ C1-220816) The UE is registered on a Forbidden PLMN (FPLMN) to receive disaster roaming service. The UE is not in the home country and there is no PLMN of the same country as the current serving VPLMN stored in the User Controlled PLMN Selector with Access Technology" data file (UPLMN list) in the SIM/ME or the "Operator Controlled PLMN Selector with Access Technology" data file (OPLMN list) in the SIM/ME. Question 1) Is the UE allowed or even mandated to scan for all available PLMNs in order to obtain service on an allowable PLMN when registered for disaster roaming service? Question 2) Is the UE allowed or even mandated to attempt service on such other allowable PLMNs when registered for disaster roaming service or shall the UE continue to use disaster roaming service on the FPLMN? SA1 agrees that this searching behavior is not only to determine the suitability of a PLMN for registration, but also on an ongoing basis. Thus a UE registered on a FPLMN as allowed by MINT may to continue to scan for allowable PLMNs and attempt service with such allowable PLMNs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies that the registration in a FPLMN may be ended in preference for an allowable PLMN if scanning determines that an allowable PLMN is available to register on. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 1 specification is not clear regarding whether a UE shall continue to search for available PLMNs when registered on a FPLMN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

BEGIN CHANGE

#### 3.2.2.5	Periodic network selection attempts

If the Operator controlled signal threshold per access technology is not set on the USIM, a UE in Automatic Mode shall make periodic attempts to look for a higher priority PLMN including associated Access Technology of the same country as the currently received PLMN including associated Access Technology or for a higher priority PLMN including associated Access Technology that uses a shared MCC (e.g. MCC=901). If the currently received PLMN including associated Access Technology uses a shared MCC, also a higher priority PLMN including associated Access Technology using any non-shared MCC shall be considered. For the ranking of PLMNs the UE shall use the order used in clause 3.2.2.2. In the case that there is no associated Access Technology identifier, the mobile shall assume that all Access Technologies provided by a PLMN are of equal priority. Moreover, periodic network selection shall not lead to change of access technology within the registered PLMN.

When a Disaster Condition is applicable, a UE in Automatic Mode that is registered to a PLMN in the "Forbidden PLMN" data field in the SIM/USIM may make periodic attempts to look for an allowable PLMN of the same country as the currently received PLMN.

In the case that the UE has stored a list of equivalent PLMNs, the UE shall only select a PLMN if it has a higher priority than all the PLMNs, in the list of equivalent PLMNs, which are of the same country as the currently registered PLMN.

NOTE 1: 	In the context of this 3GPP TS, the term country is to be interpreted not as a political entity but as a single Mobile Country Code (MCC). For example the USA and India have multiple MCC. Such cases are in fact treated as exceptions in the 3GPP specifications. For all other countries, multiple MCCs may be used, however the specifications have not taken this into account and there could be adverse effects such as the UE being unable to detect that multiple MCCs are within the same country.

NOTE 2:	There are some situations where one MCC represents multiple political entities or where an MCC is independent of political entities. Examples include Shared MCCs as defined in ITU-T E.212 [19], where international satellite operations typically use the Shared MCC with value 901.

In the case that there are multiple EHPLMNs available, the UE shall not attempt to select a higher priority EHPLMN when an EHPLMN has already been selected. The priorities of EHPLMNs are only applicable when the UE is on a VPLMN and multiple EHPLMNs are available.

The UE shall only make reselection attempts while in idle mode. In case of GPRS terminals, the UE shall only make reselection attempts while in Idle or Stand-by mode. In case of terminals operating E-UTRA and/or NR, the UE shall only make reselection attempts while in Idle or RRC-Inactive mode.

In the case of a UE receiving an eMBMS transport service when in idle mode, the UE may postpone the higher priority PLMN search and any reselection attempt until the eMBMS transport service has finished or been stopped.

The interval between attempts shall be stored in the SIM/USIM.

Only the service provider shall be able to select for which of the previous situations, periodic network selection shall be attempted and to set the interval value.

For UEs supporting only Access Technologies other than the following: NB-IoT, GERAN EC-GSM-IoT and Category M1 [17] of E-UTRAN enhanced-MTC the UE shall interpret the interval value to be between 6 minutes and 8 hours, with a step size of 6 minutes.

For UEs only supporting any of the following, or a combination of, NB-IoT, GERAN EC-GSM-IoT [18], and Category M1[13] of E-UTRAN enhanced-MTC, the UE shall interpret the interval value to be between 2 and 240 hours, with a step size of 2 hours between 2 and 80 hours and a step size of 4 hours between 80 and 240 hours.

For UEs supporting a combination of IoT and non-IoT Access Technologies the UE shall interpret the interval value to determine the timer value as above based on the Access Technology currently in use by the UE.

One interval value shall be designated to indicate that no periodic attempts shall be made.

In the absence of a permitted value in the SIM/USIM, or the SIM/USIM is phase 1 and therefore does not contain the datafield, then a default value of 60 minutes, shall be used by the UE except for those UEs only supporting any of the following, or a combination of: NB-IoT, GERAN EC-GSM-IoT [18], and Category M1 [17] of E-UTRAN enhanced-MTC. For those UEs a default value of 72 hours shall be used.

NOTE:	Use of values less than 60 minutes may result in excessive UE battery drain.

Periodic network selection based on Operator controlled signal threshold criteria

If the Operator controlled signal threshold per access technology is set on the USIM, the UE shall perform the periodic network selection procedure considering all PLMNs, of higher or lower priority than the current PLMN, in the same order as in clause 3.2.2.2, with the following conditions:

-	the UE shall select a higher priority PLMN/access technology combination if the target PLMN’s access technology signal quality is equal to or higher than the Operator controlled signal threshold per access technology.

-	the UE shall select a lower priority PLMN/access technology combination only if the RPLMN’s access technology signal quality is lower than the Operator controlled signal threshold per access technology, and the target PLMN’s access technology signal quality is equal or higher than the Operator controlled signal threshold per access technology.

If no PLMN, including the RPLMN, fulfils the Operator controlled signal threshold criteria, the UE shall perform the normal periodic network selection for higher priority PLMNs (as described above), without applying the Operator controlled signal threshold per access technology.

END CHANGE