3GPP SA WG1 Meeting # 98-e	S1-221167r6

Electronic Meeting, 09 - 19 May 2022	(revision of S1-221167)

# Source:	Novamint, China Telecom, TNO, ESA, Avanti, Intelsat, Eutelsat, Sateliot, GateHouse, Hughes Network systems, Viasat, IIIT Hyderabad, CATT, China Mobile, Xiaomi, Ali Company, Hispasat, Lockheed Martin, Qualcomm

# Title:	New SID on satellite access Phase 3

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on satellite access - Phase 3

Acronym: FS_5GSAT_Ph3

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 770002 | Study on using Satellite Access in 5G | Previous Rel-16 work covering satellite access in 5GS related use cases and requirements (ref. TR22.822: FS_5GSAT) |
| 790001 | New Services and Markets Technology Enablers– Phase 2 | Stage 1 work item of 5G system. |
| 720005 | New Services and Markets Technology Enablers | Stage 1 work item of 5G system |
| 800010 | Integration of Satellite Access in 5G | The study will take into account stage-1 Normative Work on the support of satellite |
| 860005 | Integration of satellite systems in the 5G architecture (5GSAT_ARCH) | The study will take into account stage-2 Normative Work on the support of satellite |

Dependency on non-3GPP (draft) specification:

# 3	Justification

TS22.261 has some requirements for satellite access and satellite backhaul in the 5G system. RAN WGs have defined satellite-enabled NG-RAN. SA2 already enhanced 5G system architecture to support satellite-enabled RAN node and satellite backhaul on NG interface and is studying the case of UPF on GEO satellite.

But some new capabilities should be addressed in REL 19:

- Store and forward operation with discontinuous feeder link for delay-tolerant IoT

Store and Forward (S&F) operation for a satellite allows to provide autonomously service to UEs without the satellite being always connected to a gateway (i.e. S&F allows service link access to continue to be operational even at times when the feeder link is not connected). This is particularly relevant for delay-tolerant IoT services via NGSO space segment.

In this context, each satellite of a constellation acts similarly to a SMSC and store the message until there is a feeder link available so it is important to identify the service requirements associated to this capability such as permission and authorization, UE awareness of the context or the status of its message or other aspects such as validity, security…

Analysis of use cases may lead to additional service requirements that would guide the architecture work in both SA2 and RAN3.

- GNSS independent operation

Currently, 3GPP satellite access assumes that User Equipment rely on GNSS and service. However, many UEs do not implement GNSS receiver or cannot use it in different contexts.

Support of GNSS free operation would allow to provide satellite access to UE without GNSS receiver or with no access to GNSS service in particular to support vertical (e.g. IoT) and other usages (e.g. MBB for videosurveillance).

Analysis of use cases may lead to additional service requirements related to several aspects among which permission, authorization and access control.

- Positioning Enhancements for satellite access

Current positioning requirements, as defined in Rel-16, were mainly targeting terrestrial access, and some requirements or KPIs can not be supported in the case of satellite access, e.g. the TTFX ( 1~10ms).

New use cases for satellite access need to be investigated as well as their requirements on positioning, e.g. use cases for high accuracy and low latency for positioning via NTN access.

- Communication between UEs under the same satellite’s coverage

In some scenarios, a group of ships or cars may roam to places far from the territorial network and then need to communicate via satellite access. Whereas, the communication among members will be subject to long delays and limited data rate because the communication data has to travel the feeder link twice, which may not satisfy the application layer requirements, e.g. low latency required by mission critical services, and consume many backhaul resources (including resources of ISL and feed link).

Therefore, it is proposed to study the use cases of UEs served by the same satellite for low latency communication and identify the necessary requirements. Study relevant use cases requiring higher efficiency on the satellite uplink and downlink and/or requiring lower latencies and identify the necessary requirements.

# 4	Objective

The objectives of this study are to study new use cases and related regulatory requirements and identify new potential service requirements and enhancement for 5G system over satellite, including:

-	Operation with intermittent/temporary satellite connectivity for delay-tolerant communication service

- Allowing to support delay tolerant communication service via satellite (e.g. NGSO) when satellite connectivity is intermittently/temporarily unavailable, e.g. to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment.

-	GNSS independent operation

- Support of GNSS independent operation would allow to provide satellite access to UE without GNSS receiver or with no access to GNSS service.

-	Positioning Enhancements

- Gap analysis between existing positioning requirements for terrestrial access and the requirements that can be satisfied via NTN access, and determine potential updates to NTN positioning KPIs.

- Study new relevant regulatory requirements if any

-	Communication between UEs under the same satellite’s coverage

- Analysis of potential additional service requirements e.g. regarding latency

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 22.XXX | Study on satellite access - Phase 3 | TSG#98 | TSG#99 | Thierry Bérisot, Novamint, (tberisot@novamint.com) Xu Xia, China Telecom, (xiaxu@chinatelecom.cn) |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Rapporteur: Thierry Bérisot, Novamint, tberisot@novamint.com

Co-rapporteur: Xu Xia, China Telecom, xiaxu@chinatelecom.cn

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Novamint

Avanti

ESA

Eutelsat

GateHouse

Hughes Network systems

IIIT Hyderabad

Intelsat

Sateliot

TNO

Viasat

China Telecom

CATT

China Mobile

China Unicom

Xiaomi

OPPO

ZTE

Ali company

Hispasat

Lockheed Martin

Qualcomm