| 3GPP TR 22.851 V0.2.0 (2022-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group TSG SA; Feasibility Study on Network Sharing Aspect (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	7

5	Use cases	7

5.1	Use case without direct connections between the shared access and the core networks of the participating operators	7

5.1.1	Description	7

5.1.2	Pre-conditions	7

5.1.3	Service Flows	9

5.1.4	Post-conditions	9

5.1.5	Existing feature partly or fully covering use case functionality	9

5.1.6	Potential New Requirements needed to support the use case	10

5.2 	Use case on Service Continuity and QoS	10

5.2.1	Description	10

5.2.2	Pre-conditions	10

5.2.3	Service Flows	11

5.2.4	Post-conditions	11

5.2.5	Existing feature partly or fully covering use case functionality	12

5.2.6	Potential New Requirements needed to support the use case	12

5.3	use case on mobility between sharing parties	12

5.3.1	Description	12

5.3.2	Pre-conditions	12

5.3.3	Service Flows	13

5.3.4	Post-conditions	14

5.3.5	Existing feature partly or fully covering use case functionality	14

5.3.6	Potential New Requirements needed to support the use case	14

5.4	Use case on International Roaming Users in a Shared Network	15

5.4.1	Description	15

5.4.2	Pre-conditions	15

5.4.3	Service Flows	15

5.4.4	Post-conditions	15

5.4.5	Existing feature partly or fully covering use case functionality	16

5.4.6	Potential New Requirements needed to support the use case	16

5.X	Use case of X	16

5.X.1	Description	16

5.X.2	Pre-conditions	16

5.X.3	Service Flows	16

5.X.4	Post-conditions	16

5.X.5	Existing feature partly or fully covering use case functionality	16

5.X.6	Potential New Requirements needed to support the use case	16

6	Conclusion and recommendations	16

Annex <Z> (informative): Change history	18

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

Network sharing enables operators to maximize rollout and improve overall network quality. The 3GPP system supports different types of network sharing since Rel-5, most of which are inherited by 5G (Rel-15 onward). With the expanding of 5G deployments more network sharing scenarios can arise, depending on different operators’ strategies, commercial agreements, and specific rules/legislation in different countries. One of the challenges for the operators is the maintenance of the interconnections (e.g., number of network interfaces) between the shared NG-RAN and Participating Operators’ core networks, especially in the case of a large number of the shared base stations. It is therefore suggested to investigate other type of network sharing scenarios, where a NG-RAN is shared among multiple operators without necessarily assuming a direct connection between the shared radio access network and the Participating Operator’s core network.

1	Scope

The present document investigates use cases and potential new requirements related to 3GPP system enhanced support of specific 5G network sharing deployment scenarios, in particular where there is no direct interconnection between the shared NG-RAN and Participating Operators’ core networks. It includes the following aspects:

-	Mobility and service continuity, when moving from a non-shared 4G/5G network to a shared 5G network and vice versa, with focus on CN aspects.

-	Potential security requirements.

-	Charging requirements (e.g., based on traffic differentiation in specific network sharing geographical areas).

-	User/service experience (e.g., maintain the communication latency for voice, and SMS) when accessing the shared network, including scenarios of home-routed traffic or local breakout.

-	Other aspects, e.g., regulatory requirements, emergency services, PWS support.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- 3GPP TS 22.101: "Service principles".

- 3GPP TS 22.261: "Service requirements for the 5G system".

- 3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

- 3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

- 3GPP TS 22.011: "Service accessibility".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Indirect Network Sharing describes the communication between the shared access NG-RAN and the participating operator’s core network being routed through the hosting operator’s core network.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

HTA	High Traffic Areas

LTA	Low Traffic Areas

MOCN	Multi-Operator Core Network

NG-RAN	Next Generation Radio Access Network

SST	Slice/Service Type

4	Overview

Editor’s Note: An Overview will be added in the future.

5	Use cases

5.1	Use case without direct connections between the shared access and the core networks of the participating operators

5.1.1	Description

As stated in TS 22.261 [3] the increased density of access nodes needed to meet future performance objectives pose considerable challenges in deployment and acquiring spectrum and antenna locations. RAN sharing is seen as a technical solution to these issues.

Sharing access networks and network infrastructure has become more important part of 3GPP systems.

When two or more operators have respectively deployed or plan to deploy 5G access networks and core networks, a MOCN configuration can be considered for network sharing between these operators, i.e., a Multi-Operator Core Network (MOCN) in which multiple CN nodes are connected to the same radio access and the CN nodes are operated by different operators.

One of the challenges for the partners’ network operators is the maintenance generated by the interconnection (e.g., number of network interfaces) between the shared RAN and two or more core networks, especially for a large number of shared base stations.

For these reasons it is suggested to investigate other type of network sharing scenarios, where a 5G RAN is shared among multiple operators without necessarily assuming a direct connection between shared access and the core networks of the participating operators.

5.1.2	Pre-conditions

Two (or more) operators provide coverage with their respective radio access networks in different parts of a country but together cover the entire country.

There is an agreement between all the operators to work together and to build a shared network, but utilizing the different operator’s allocated spectrum appropriately in different parts of the coverage area (for example, Low Traffic Areas, LTA and High Traffic Areas, HTA).

The Hosting RAN operator 1, as illustrated below, can share its NG-RAN with the Participating Operators with or without direct connections between the shared access and the core networks of the participating operators.

The following pre-conditions apply:

1. OP1 owns the NG-RAN to be shared with three other operators; OP2, OP3, and OP4.

2. NG-RAN is shared with certain conditions, e.g., within a specific 5G frequency band or within specific area.

3. NG-RAN does not have direct connections between the shared access and the core networks of the participating operators OP2 and OP3.

4. OP4 has a MOCN arrangement with OP1.

5. In this example UE 1 is subscribed to OP1, UE 2 is subscribed to OP 2, UE 3 is subscribed to OP3, and UE 4 is subscribed to OP4.

Shared NG-RAN of OP 1

OP 2 Core Network

OP 3 Core Network

OP 4 Core Network

OP 1 Core Network

Indirect Connection

Indirect Connection

N2

N2

UE 2 of OP2

UE 3 of OP3

UE 4 of OP4

UE 1 of OP 1

UE2's screen:

name of

OP 2

Figure 5.1.2-1: Different options both direct and indirect connections between the shared access and the core networks of the participating operators

OP 1 Core

Network

Shared NG-RAN of OP 1

OP 2 Core Network

OP 3 Core Network

Indirect Connection for UE2

UE 2 of OP2

UE2's screen:

name of

OP 2

UE 3 of OP3

Indirect Connection for UE3

Figure 5.1.2-2: Indirect Network Sharing involves core network of Hosting operator, as indirect connections between the shared access and the core networks of the participating operators

5.1.3	Service Flows

1. UE1 can successfully attach to NG-RAN, and the display of the network operator name is the name of OP1.

2. UE2 can successfully attach to NG-RAN, and the display of the network operator name is the name of OP2.

3. UE3 can successfully attach to NG-RAN, and the display of the network operator name is the name of OP3.

4. UE4 can successfully attach to NG-RAN, and the display of the network operator name is the name of OP4.

5.The service provider of UE 1 is OP1.

6.The service provider of UE 2 is OP2.

7.The service provider of UE 3 is OP3.

8.The service provider of UE 4 is OP4.

For UEs accessing the shared RAN, the network of the Hosting Operator needs to know which participating operator a UE is registered to and what type of network sharing (e.g., MOCN or otherwise) is in place for that participating operator.

5.1.4	Post-conditions

The hosting network will be able to provide services to all participating operators' users.

5.1.5	Existing feature partly or fully covering use case functionality

Network sharing has been studied in previous releases, where related normative stage 1 requirements are introduced in 3GPP TS 22.101 [2] and 22.261[3].

3GPP TS 22.101 [2] introduces general requirements of network sharing, stated as follows:

Network sharing shall be transparent to the user.

The specifications shall support both the sharing of:

(i)	radio access network only;

(ii)	radio access network and core network entities connected to radio access network.

NOTE:	In a normal deployment scenario only one or the other option will be implemented.

The provisioning of services and service capabilities is described in 3GPP TS 22.101 [2].

The provision of services and service capabilities that is possible to offer in a network shall not be restricted by the existence of the network sharing It shall be possible for a core network operator to differentiate its service offering from other core network operators within the shared network.

It shall be possible to control the access to service capabilities offered by a shared network according to the core network operator the user is subscribed to.

As described in 3GPP TS 22.261 [3] the UE uses the list of PLMN/RAT combinations for PLMN selection, if available, typically during roaming situations. In non-roaming situations, the UE and subscription combination typically matches the HPLMN/EHPLMN capabilities and policies, from a SST (slice/service type) perspective. That is, a 5G UE accessing its HPLMN/EHPLMN should be able to access SSTs according to UE capabilities and the related subscription.

Furthermore, the 5G system shall support selection among any available PLMN/RAT combinations, identified through their respective PLMN identifier and Radio Access Technology identifier, in a prioritised order. The priority order may, subject to operator policies, be provisioned in an Operator Controlled PLMN Selector lists with associated RAT identifiers, stored in the 5G UE.

The 5G system shall support, subject to operator policies, a User Controlled PLMN Selector list stored in the 5G UE, allowing the UE user to specify preferred PLMNs with associated RAT identifier in priority order.

5.1.6	Potential New Requirements needed to support the use case

[PR 5.A.6-001] The 5G system shall be able to support network sharing with indirect connection between the shared access network and one or more Participating Operators’ core networks.

[PR 5.A.6-002] The 5G system shall be able to support means for participating operators to provide their operator name to a registered UE, for display to the user.

5.2 	Use case on Service Continuity and QoS

5.2.1	Description

The requirements to the services provided by the Participating Operator and the Hosting RAN operator for a UE moving between their service areas needs to be clearly defined. These service considerations are based on their user subscriptions and charging requirements from the Participating Operator. The service principle is not expected to be significantly different from the MOCN access sharing. The business here includes not only the operator’s name displayed in the UE UI, but also the service logics provided by both the Participating Operator and the Hosting RAN operator for the services, such as voice, SMS and data communications for the UE.

5.2.2	Pre-conditions

Assumptions,

1. OP 1 is the Hosting RAN operator of NG-RAN access network.

2. The core network of OP 2 does not have direct connection with OP1’s shared NG-RAN.

3. There is connection between the OP1’s CN and OP2’s CN.

4. UE 1 belongs to OP 1. UE 2 and UE N belong to OP 2.

5.2.3	Service Flows

OP1 IMS

network

OP1 network

with E-UTRAN

and shared NG-RAN

OP2 IMS

network

OP2 network

with E-UTRAN

and NG-RAN

connection between the OP1’s CN and OP2’s CN

UE 2 of

OP2

UE 1

of OP1

UE N

of OP2

Figure 5.2.3-1: Basic service scenario without direct connections between the shared access and the core networks of the participating operators

1. UE 2 successfully registers to OP 2’s PLMN via the shared NG-RAN.

2. UE 1 successfully registers to OP 1's PLMN.

3. UE N successfully registers to OP 2's PLMN, via OP2’s wireless access network.

4. UE 2 initiates a service, for example a 5G voice call to user N, and succeeds under the shared network.

- 	OP 2 and OP1 do not need to expose the IMS network to each other, e.g., UE2 may also access services provided by their home environment in the same way even if the UE2 moves to the coverage of shared NG-RAN of OP1, and it is not necessary to assume that the IMS network of OP1 is involved in this scenario.

-	When UE 2 moves to OP 2's 4G area from the shared network, the call continues.

- 	When UE 2 moves to OP 2's 5G area from the shared network, the call does not drop.

- UE2 may also use other services under the shared network, just as it usually does in the OP2’s network while OP1 and OP2 have different network services and service capabilities.

- The network sharing partners may set a specific sharing allocation model for a network sharing method communicating with. It is necessary to understand the number of the users, and how long users using a certain shared network method will take. The collection of charging information associated with the sharing method that the UE accesses with should be possible.

5.2.4	Post-conditions

The service of UE 2 succeeds, when UE 2 moves between shared access network and Participating Operator’s network.

5.2.5	Existing feature partly or fully covering use case functionality

SA1 has performed various studies on services in previous releases. For the definition of service continuity and the description of user experience see TS 22.261 [3]. The description of service capabilities is stated in TS 22.101 [2]. Some typical requirements for service in network sharing are:

The provision of services and service capabilities that is possible to offer in a network shall not be restricted by the existence of the network sharing It shall be possible for a core network operator to differentiate its service offering from other core network operators within the shared network.

It shall be possible to control the access to service capabilities offered by a shared network according to the core network operator the user is subscribed to.

The 3GPP System shall support service continuity for UEs that are moving between different Shared RANs or between a Shared RAN and a non-shared RAN.

In addition to the charging requirements of 5G system introduced in Chapter 9 of TS 22.261 [3], the following requirement is included in TS 22.101 [2]:

Charging and accounting solutions shall support the shared network architecture so that end users can be appropriately charged for their usage of the shared network, and network sharing partners can be allocated their share of the costs of the shared network resources.

And Further Charging requirements related are specified in TS 29.513 [4].

5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-001] The 5G core network shall be able to support collection of charging information associated with a UE accessing a shared network using indirect network sharing.

5.3	use case on mobility between sharing parties

5.3.1	Description

It is worth mentioning that 5G networks have been designed to be able to provide shared facilities from the beginning. This means that in the case that the 4G network has both non-shared and shared E-UTRAN at the same time, there could be a number of different type of coverage in the same region:

- Non-shared E-UTRA coverage,

- Shared E-UTRA coverage,

- Non-shared 5G NR coverage,

- Shared 5G NR coverage.

This study introduces a new sharing method arises as without direct connection between shared NG-RAN and the core network of participating operators. Therefore, interoperability regarding mobility management needs to be considered when introducing the potential requirements of this new sharing method with existing network.

This use case will discuss how a UE with a subscription from OP2 can move between OP2’s own 4G or 5G access networks and either OP1’s or OP3’s shared 5G networks (described in scenario 1) as well how the UE from OP2 can move between OP1’s and OP3’s shared 5G networks (described in scenario 2).

5.3.2	Pre-conditions

1. It is assumed that OP1, OP2 and OP3 has deployed 4G and 5G networks.

NOTE: The home 4G and 5G network of OP1, OP2 and OP3 may be deployed with non-shared and shared wireless access technology.

- Both OP1 and OP3 are Hosting RAN Operators, which shared NG-RAN with Participating OP2.

- UE subscribe to OP2’s home PLMN.

- UEs may register successfully to OP1 and OP3’s shared 5G network.

2. Both operators (i.e., OP1 and OP3) agreed to share their networks via indirect connection between the shared radio access network and the OP2’s core network.

3. Potential scenario1: The coverage of OP1 and OP3’s shared 5G network may overlap with OP2’s 4G network; may also overlap with OP2’s 5G network (i.e., at OP1 and OP2’s border, at OP3 and OP2’s border).

4. Potential scenario2: The coverage of OP1’s shared 5G network may overlap with OP3’s shared 5G network (i.e., at OP1’s and OP3’s border).

5. According to the mutual agreement of the sharing parties, UE access with one of the wireless technology, e.g., UE may access to shared 5G network, when the coverage of OP1’s shared 5G network may overlap with OP2’s 4G network.

5.3.3	Service Flows

Mobility scenarios are shown in the following pictures of shared network, including:

Figure 5.3.3-1a:  Scenario 1, a UE with a subscription from OP2 makes a handover between OP2’s own 4G access networks and either OP1’s or OP3’s shared 5G networks.

Figure 5.3.3-1b: Scenario 1, a UE with a subscription from OP2 moves between OP2’s own 5G access networks and either OP1’s or OP3’s shared 5G networks.

NOTE 1: OP1_5G/OP3_5G are OP1/OP3’s shared 5G network via indirect connection between the shared radio access network and the OP2’s core network.

NOTE 2: OP2_5G/OP2_4G is OP2’s network, may be MOCN networks or non-shared network.

1. The mobility management needs to be supported when UE crosses the border between the shared network managed by Hosting operator and the OP2’s own 4G access networks in the scenario 1 (As shown as ① in figure 5.3.3-1a).

2. The mobility management needs to be supported when UE crosses the border between the shared network managed by Hosting operator and the OP2’s own 5G access networks in the scenario 1 (As shown as ② in figure 5.3.3-1b).

Figure 5.3.3-2:  Scenario 2, a UE with a subscription from OP2 moves between coverage of OP1’s and OP3’s shared 5G access networks

3. And the mobility management needs to be supported when UE crosses the border between the two shared networks managed by different Hosting operators in the scenario 2 (As shown as ③ in figure 5.3.3-2).

5.3.4	Post-conditions

All forms of mobility (i.e., between participating operators RAN and shared RAN for both CONNECTED mode and IDLE mode UE, see clause 3.1 of TS 23.122 [5]) successfully processed in a sharing scenario without direct connections between the shared access and the core networks of the participating operator.

5.3.5	Existing feature partly or fully covering use case functionality

SA1 has performed various studies on mobility and network sharing in previous releases, where related normative stage 1 requirements are introduced in 3GPP TS 22.101 [2] and 22.261 [3].

3GPP TS 22.261 [3] introduces requirements of Diverse mobility management, stated as follows:

The 5G system shall support inter- and/or intra- access technology mobility procedures within 5GS with minimum impact to the user experience (e.g. QoS, QoE).

3GPP TS 22.261 [3] describes various access related requirements, stated as follows:

Based on operator policy, the 5G system shall support steering a UE to select certain 3GPP access network(s).

3GPP TS 22.101 [2] introduces requirements of mobility of network sharing, stated as follows:

It shall be possible to support different mobility management rules, service capabilities and access rights as a function of the home PLMN of the subscribers.

The above requirements are based on MOCN.

5.3.6	Potential New Requirements needed to support the use case

[PR 5.3.6-001] In case of indirect network sharing, the 3GPP System shall support mechanisms to minimize service interruptions for UEs that are moving between different Shared RANs or between a Shared RAN and a non-shared RAN (managed by Hosting and Participating Operators).

5.4	Use case on International Roaming Users in a Shared Network

5.4.1	Description

There are network sharing scenarios in the process of 5G deployment due to different operators’ business consideration, such as network planning, operation and other factors, which introduce demand for 5G network sharing with indirect connection between the shared access network and a participating operator’s core network, especially in countries with a wide range of rural areas, where seamless 5G radio coverage is hard to achieve. Benefit from 5G network sharing technology, an operator can not only save investment for 5G deployment, but also expand 5G coverage in vaster area as well. However, in this case, it is also required to allow inbound international roaming subscribers to use the shared resources of a hosting operator. Therefore, it is suggested to investigate how to realize the scenario in this situation.

5.4.2	Pre-conditions

In the 5G scenario, an operator may deploy its network as an applicable shared network to other operators.

As illustrated in Figure 5.4.2-1, three operators (OP1, OP2, OP3) are involved:

- OP1, as a Hosting Operator, owns an applicable shared NG-RAN which can be shared to other operators (e.g., OP2).

- OP2, as a Participating Operator, has 5G network sharing agreement with OP1. In addition, OP2 may have its own non-shared 5G network.

- Shared NG-RAN of OP1 does not have direct connections between the shared access and the core networks of the participating operators OP2.

- OP3 is a roaming partner of OP2, but has no roaming agreement with OP1.

- UE subscribes to OP3’s network as a foreign operator’s network.

Figure 5.4.2-1: Scenario of International Roaming Users in a Shared Network

5.4.3	Service Flows

When a UE of OP3 travels to the coverage of OP2’s NG-RAN, it can enjoy services provided by their home environment in another country via OP2’s network.

When a UE of OP3 travels to the coverage of OP1’s shared NG-RAN, it can enjoy services provided by their home environment in another country via OP1’s shared access network and OP2’s network.

5.4.4	Post-conditions

It is the same for the service experience and treatment of OP2’s UE and OP3’s UE in the coverage of OP1’s shared RAN, based on the agreement between operators.

5.4.5	Existing feature partly or fully covering use case functionality

TS 22.011 [6] introduces requirements on roaming in shared networks, as described below:

The following requirements are applicable to GERAN, UTRAN, E-UTRAN and NG-RAN sharing scenarios:

-	When network sharing exists between different operators and a user roams into the shared network it shall be possible for that user to register with a core network operator (among the network sharing partners) that the user’s home operator has a roaming agreement with, even if the operator is not operating a radio access network in that area.

Other roaming related requirements are described in TS 22.101 [2]:

3GPP specifications should be in compliance with the following objectives:

e)	to provide support of roaming users by enabling users to access services provided by their home environment in the same way even when roaming.

5.4.6	Potential New Requirements needed to support the use case

[PR 5.4.6-001] The 5G system shall enable the shared access network of a hosting operator with indirect connection between the shared access network and a participating operator’s core network to provide services for inbound roaming users.

NOTE:  Inbound roaming users mentioned above refer to the subscribers of a foreign operator having a roaming agreement with one participating operator.

5.X	Use case of X

5.X.1	Description

5.X.2	Pre-conditions

5.X.3	Service Flows

5.X.4	Post-conditions

5.X.5	Existing feature partly or fully covering use case functionality

5.X.6	Potential New Requirements needed to support the use case

6	Conclusion and recommendations

Annex <Z> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 5.2022 | SA1#98e | S1-221092 | - | - | - | Initial Skeleton | 0.0.0 |
| 5.2022 | SA1#98e | S1-221271 | - | - | - | Introduction of TR 22.851 | 0.1.0 |
| 5.2022 | SA1#98e | S1-221292 | - | - | - | Scope of TR 22.851 | 0.1.0 |
| 5.2022 | SA1#98e | S1-221272 | - | - | - | Add the basic use case of network sharing | 0.1.0 |
| 9.2022 | SA1#99e | S1-222392 | - | - | - | Add Abbreviations | 0.2.0 |
| 9.2022 | SA1#99e | S1-222393 | - | - | - | Add definition and term | 0.2.0 |
| 9.2022 | SA1#99e | S1-222394 | - | - | - | Introduce use case on International Roaming | 0.2.0 |
| 9.2022 | SA1#99e | S1-222395 | - | - | - | use case on mobility | 0.2.0 |
| 9.2022 | SA1#99e | S1-222396 | - | - | - | use case on Service and QoS | 0.2.0 |