3GPP SA1 CAMEL Ad Hoc 	Tdoc SA1-000390

Sophia Antipolis

9th – 11th May 2000

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 22.078 |  | Version: |  |  | 3.4.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | CAMEL Phase 3 |
| --- | --- |

| Source: | Lucent Technologies | Date: |  |
| --- | --- | --- | --- |

| Subject: | Removal of Editor’s notes and corrections to Annex 1 |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification | x |  | Release 99 | x |
|  |  |  |  | UMTS |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | This CR proposes the removal of outstanding editors’ notes. In addition, tables A-1 and A-2 in Annex A now include a reference to the sections for which the columns are applicable. |
| --- | --- |

| Clauses affected: | 18.8.1.2, 18.8.4, A.1, A.2 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: | Outstanding editors’ notes can be found at Section A.1 (Information provided to the CSE). It is not yet clear how this is to be resolved. |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

****   FIRST MODIFIED SECTION    ****

### 18.8.1	Barring of all outgoing calls

#### 18.8.1.1	Mobile originated calls

No interaction. The Barring of all outgoing calls supplementary service will be invoked. Thus, originating CAMEL based services will not be invoked.

#### 18.8.1.2	Forwarded Calls

No interaction. If the Barring of all outgoing calls supplementary service is active and operative, it shall prevent the registration or activation of Call Forwarding as specified in TS 22.082.

****   NEXT MODIFIED SECTION    ****

### 18.8.4	Barring of all incoming calls

No interaction. The Barring of all incoming calls supplementary service shall be invoked. Thus, terminating CAMEL based services will not be invoked.

****   NEXT MODIFIED SECTION    ****

# 20	Interactions with Optimal Routeing (OR)

Invocation of OR shall not have any impact of any CAMEL based service.

If OR is applied to a late Call Forward then the interrogating PLMN shall invoke a mobile originated CAMEL based service, if required for the served subscriber.

$(begin$(CAMEL2$)

If OR of a basic mobile-to-mobile call is invoked, mobile originating services based on CAMEL phase 2 which rely on the destination of the MO call leg being determined by the dialled number (in particular, prepayment services) will not necessarily operate correctly.

If OR of late call forwarding is invoked from an IPLMN which is also the forwarding subscriber’s HPLMN, then mobile terminating services based on CAMEL phase 2 which rely on the destination of the leg from the IPLMN being determined by the MSRN (in particular, prepayment services) will not necessarily operate correctly.

$(end$(CAMEL2$)

$(begin$(CAMEL3$)

If a call is subject to basic OR, VPLMN-A shall pass the address defining the ultimate destination of the call (whether VPLMN-B, HPLMN-B or the forwarded-to destination) to the CSE of the originating subscriber.

If a call is subject to OR of late call forwarding from an IPLMN which is also the forwarding subscriber’s HPLMN, then the IPLMN shall pass the forwarded-to number to the CSE which handles mobile terminating CAMEL-based services for the forwarding subscriber.

$(end$(CAMEL3$)

Specific interaction is described in TS 22.079 [2].

****   NEXT MODIFIED SECTION    ****

Annex A (normative):
Information Tables

# A.1	Information provided to the CSE

The following table shows the information that is transferred towards the CSE on various events. The numbers reflect the applicable Camel phase (1, 2, 3).

|  | Call set-up request procedure 1 (section 5.3.1 for MO calls) | Call set-up request procedure 2 (section 5.3.1 for CF calls) | Call set-up request procedure 3 (section 5.3.2.1 for MO calls | Call set-up request procedure 4 (section 5.3.2.1 for CF calls | Unsuccessful call establishment | Unsuccessful call establishment (MT) | Incoming call request procedure | Procedures for serving network dialled services 1 (section 7.1 MO calls) | Procedures for serving network dialled services 2 (section 7.1 CF calls) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Event met | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| IMSI | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| Calling Party’s Number | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| Calling Party’s Category | 1 | 1 | 3 | 3 | - | 3 | 1 | 3 | 3 |
| Additional Calling Party Number | - | 1 | - | 3 | - | 3 | 1 | - | 3 |
| Called Party BCD Number | 1 | - | 3 | - | 3 | - | - | 3 | - |
| Called Party Number | - | 1 | - | 3 | - | 3 | 1 | - | 3 |
| Original Called Party Number | - | 1 | - | 3*1 | - | 3 | 1 | - | 3*1 |
| Redirecting (Party) Number ???? | - | 1 | - | 3 | - | 3 | 1 | - | 3 |
| Redirection Information | - | 1 | - | 3 | - | 3 | 1 | - | 3 |
| Service Key | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| ISDN Bearer Capability | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| High Layer Compatibility | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| Basic Service Code | 1 | 1 | 3 | 3 | 3 | 3 | 1 | 3 | 3 |
| Call Identification Information | 1 | 1 | 3 | 3 | 3 | - | - | ? | ? |
| Location Information of the Calling Subscriber | 1 | - | 3 | - | 3 | - | - | 3 | - |
| Location Number of the Calling Subscriber | - | - | - | - | - | 3 | 1 | - | - |
| Location information of the called subscriber | - | - | - | - | - | 3 | 1 | - | - |
| Subscriber State of the called subscriber | - | - | - | - | - | 3 | 1 | - | - |
| Cause - $(CAMEL3$) | - | - | - | - | 3 | 3 | - | - | - |
| Time and Time Zone Information - $(CAMEL2$) | 2 | 2 | 3 | 3 | 3 | 3 | 2 | ? | ? |
| Calling Party LSA (if available) $(CAMEL3$) | 3 | - | 3 | - | - | - | - | 3 | - |
| NAEA Carrier Identification Code (CIC) -$(CAMEL3$) | 2 | 2 | 3 | 3 | 3 | 3 | 2 | 3 | 3 |
| NAEA Carrier Selection Information (pre-subscribed or on-demand) -$(CAMEL3$) | 2 | 2 | 3 | 3 | 3 | - | 2 | 3 | 3 |
| CUG Index if received from the calling subscriber | 3 | - | - | - | - | - | - | - | - |
| CUG Interlock Code | - | 3 | - | - | - | - | 3 | - | - |
| CUG Outgoing Access Indicator | - | 3 | - | - | - | - | 3 | - | - |

Table A-1: Information transferred towards the CSE

*1: If any other CAMEL dialogue has modified called party number then the modified number is reported to the CSE of dialled services.

[CR editor’s note: S1 tdoc S1-99674 has no column for MT unsuccessful case. However, it indicates that some elements are supported in CAMEL ph 2 for unsuccessful cases. This fact needs clarification in future meetings.]

# A.2	Information sent by the CSE

The following table shows the information that is sent by the CSE on various events. The numbers reflect the applicable Camel phase (1, 2, 3).

|  | Call set-up request procedure 1(section 5.3.1) | Call set-up request procedure 2(section 5.3.2.2) |  | Unsuccessful call establishment (MO) |  | Call disconnection procedure (MO) |  |  |  | Incoming call request procedure |  | Unsuccessful call establishment (MT) |  | Call disconnection procedure (MT) |  | Procedures for serving network dialled services |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Called Party Number | 1 |  | 3 |  | 2 |  | 2 |  |  |  | 1 |  | 2 |  | 2 |  | 3 |  |  |
| Calling Party Number | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| Calling Party’s Category | 1 |  | 3 |  | 2 |  | 2 |  |  |  | 1 |  | 2 |  | 2 |  | 3 |  |  |
| Calling IMSI | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| ISUP CUG information | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| Additional Calling Party’s Number | 1 |  | 3 |  | 2 |  | 2 |  |  |  | 1 |  | 2 |  | 2 |  | 3 |  |  |
| Original Called Party Number | 1 |  | 3 |  | 2 |  | 2 |  |  |  | 1 |  | 2 |  | 2 |  | 3 |  |  |
| Redirection Party Number | 1 |  | 3 |  | 2 |  | 2 |  |  |  | 1 |  | 2 |  | 2 |  | 3 |  |  |
| Redirection Information | 1 |  | 3 |  | 2 |  | 2 |  |  |  | 1 |  | 2 |  | 2 |  | 3 |  |  |
| Alerting Pattern | - |  | - |  | - |  | - |  |  |  | 2 |  | - |  | - |  | - |  |  |
| ISDN Access related Information | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| ISDN Bearer Capability | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| High Layer Compatibility | - |  | - |  | - |  | 3 |  |  |  | - |  | - |  | - |  | - |  |  |
| Basic Service Code | - |  | - |  | - |  | 3 |  |  |  | - |  | - |  | - |  | - |  |  |
| Called Party to be Created | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| New Call Segment | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| In Service Compatibility Response | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| Service Interaction Indicators Two | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| Location Number | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| NAEA Carrier Identification Code (CIC) - $(CAMEL2$) | 2 |  | 2 |  | 2 |  | 2 |  |  |  | 2 |  | 2 |  | 2 |  | 3 |  |  |
| NAEA Carrier Selection Information (pre-subscribed or on-demand) - $(CAMEL2$) | 2 |  | 2 |  | 2 |  | 2 |  |  |  | 2 |  | 2 |  | 2 |  | 3 |  |  |
| NAEA Originating Line Identification (OLI) - $(CAMEL2$) | 2 |  | 2 |  | 2 |  | 2 |  |  |  | 2 |  | 2 |  | 2 |  | 3 |  |  |
| NAEA Charge Number (CN) - $(CAMEL2$) | 2 |  | 2 |  | 2 |  | 2 |  |  |  | 2 |  | 2 |  | 2 |  | 3 |  |  |
| CSE Address | - |  | - |  | - |  | - |  |  |  | - |  | - |  | - |  | - |  |  |
| CUG Interlock Code | 3 |  | 3 |  | - |  | - |  |  |  | 3 |  | - |  | - |  | - |  |  |
| CUG Outgoing Access Indicator | 3 |  | 3 |  | - |  | - |  |  |  | 3 |  | - |  | - |  | - |  |  |
| Service Interaction Indicators | 3 |  | 3 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |

Table A-2: Information sent by the CSE

# A.3	GPRS Information provided to the CSE

The following table shows the information that is transferred towards the CSE on various GPRS events. The numbers reflect the applicable Camel phase (3).

|  | Attach procedure | PDP activation / Session Establishment | PDP activation / Session Establishment Acknowledgement |
| --- | --- | --- | --- |
| Event met | 3 | 3 | 3 |
| Type of monitoring | - | 3 | 3 |
| MSISDN | 3 | 3 | 3 |
| IMSI | 3 | 3 | 3 |
| Service Key | 3 | 3 | 3 |
| Location information at least to the resolution of Routing Area of the attaching subscriber | 3 | 3 | 3 |
| Time stamp information | 3 | 3 | 3 |
| Time zone information | 3 | 3 | 3 |
| GPRS MS Class | 3 | 3 | 3 |
| PDP transport protocol, i.e. IP or X.25 | - | 3 | 3 |
| Quality of Service information (subscribed, requested, negotiated) | - | 3 | 3 |
| Destination address information | - | 3 | 3 |
| GPRS charging correlation ID | - | - | 3 |
| Destination address information | - | - | 3 |

Table A-3: GPRS Information transferred towards the CSE

CR editor’s note for MCC’s attention: The above table has had its rows and columns resized to allow the correct display of the cell contents.