| 3GPP/SMG Meeting #? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | N2-000404 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia Antipolis, France, 10-11 May 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.078 |  |  |  |  |  | CR |  |  | 046 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.3.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | SA #8 |  |  |  |  | for approval |  |  |  |  |  |  | x |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | CAMEL ad hoc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 11 May 2000 |  |  |  |  |  |
| Subject: |  |  | Update of CAMEL roaming issues (section 16) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | CAMEL phase 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | Section 16 is outdated. It should be more generic and future proof. Also CAMEL phase 3 issues should be covered, such as SMS, GPRS etc. It is defined that the HLR may deny an incoming call attempt if the IPLMN (GMSC) does not support the appropriate CAMEL phase. This is not a new requirement, it rather clarifies the HLR/HPLMN options an operator or vendor can select. Similarly, the HPLMN may force the SendRoutingInformation (SRI) to be sent from the HPLMN GMSC by returning an "OR not allowed" error. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

****    FIRST and LAST MODIFIED SECTION    ****

# 16	Exceptional procedures or unsuccessful outcome

16.1	Roaming in non-supporting networks

The HPLMN shall control handling of roaming, when a CAMEL subscriber attempts to register in a network not supporting CAMEL without relying on extra functionality in network entities not supporting CAMEL. The HPLMN can decide on a per subscriber basis whether to allow roaming, or deny individual services (e.g. applying ODB, denying location up-date).

If the HPLMN allowsroaming, the OSSs are not supported for the roaming subscriber.

16.2	Call Set-up from a non-supporting interrogating PLMN

If the CAMEL feature is not supported in the IPLMN the following will happen:

-	Mobile originating calls:

Not applicable.

-	Mobile terminating calls:

Mobile terminating OSSs are not supported (in the IPLMN) if the HPLMN decides to allow the MT call attempt. The HPLMN may also decide to bar the incoming call attempt, or force the routeing interrogation to take place in the HPLMN.

## 16.3	Roaming in a VPLMN which supports a lower phase of CAMEL

If a CAMEL subscriber attempts to register in a VPLMN which supports CAMEL, the VPLMN shall indicate in the registration request to the HPLMN the phase of CAMEL which the VPLMN supports. If the VPLMN supports only CAMEL phase  that is lower than subscribed the HPLMN shall take such action (including denying the registration request or transferring to the VPLMN subscription information appropriate to the CAMEL phase supported in the VPLMN) as may be decided by the HPLMN operator. If a certain service requires a certain CAMEL phase (e.g. MO SMS requires at minimumleast CAMEL phase 3) and the VPLMN does not support that CAMEL phase, the HPLMN may decide to deny roaming or allow roaming without that particular CAMEL OSS.

## 16.4	Service attempt from a VPLMN which supports a lower phase of CAMEL

If the served subscriber requests a basic service (call, short message, GPRS attach, GPRS PDP context etc.) which requires the VPLMN to contact the CSE, the VPLMN shall indicate to the CSE which phase of CAMEL has been negotiated between the HPLMN and the VPLMN for this service. If the VPLMN supports a CAMEL phase that is lower than that subscribed and the CSE determines that as a consequence a service which is provisioned for the subscriber will not operate correctly, the CSE shall take such action (including denying the call request or handling the call using only VPLMN supported CAMEL  capabilities supported in the VPLMN) as may be decided by the CSE operator.

## 16.5	Call setup from an IPLMN which supports a lower phase of CAMEL

The IPLMN shall indicate to the HPLMN which phases of CAMEL it supports. The HPLMN may decide to bar the incoming call attempt before contacting the CSE, or force the routeing interrogation to take place in the HPLMN. When the IPLMN contacts the CSE for instructions to handle an MT call, the IPLMN shall indicate to the CSE the phase of CAMEL which has been negotiated between the HPLMN and the IPLMN for this callprior to If the IPLMN supports a lower  CAMEL phase than subscribednegotiated between the HPLMN and the IPLMN and the CSE determines that as a consequence a service which is provisioned for the subscriber will not operate correctly, the CSE shall take such action (including denying the call request or handling the call using only supported CAMEL  capabilities negotiated between the HPLMN and the IPLMN) as may be decided by the CSE operator.