Source: 			Rapporteur

Title:				22.129 CR for Release 2000 (Handover requirements)

Document for:	Discussion and approval

Introduction

The attached document is the proposed CR to TS 22.129 for Release 2000.  The CR is applied to version 3.3.0 (it is assumed CR012 was approved at TSG SA#8!).

Summary of Changes

The following changes have been included:

- Incorporation of the text from section 7.7 of TR 22.976 v2.0.0 into section 4 of TS 22.129

- Removal of text relating to SoLSA

- Slight clarification of wording of text on performance requirements (following suggestion from Nokia!)

Note that changes of terminology from UMTS and GSM are not included.  There are some occurrences where the use of UTRAN/GERAN would not be appropriate; following discussion at TSG-SA, this terminology is to be further discussed for Release 2000.  Changes will be made following this is necessary.

Proposal

It is proposed that this CR is agreed for presentation to S1 Plenary for approval.

| 3GPP SA1 R00 drafting meeting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | S1-00???? |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Goodwood, UK, 3-7 July 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.129 |  |  |  |  |  | CR |  |  | xxxxx |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.3.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | SA#9 |  |  |  |  | for approval |  |  |  |  |  |  |  |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  | X | Core Network |  |  |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Rapporteur |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 30 June 2000 |  |  |  |  |  |
| Subject: |  |  | Handover requirements for Release 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  | X |  |  |
| Reason for change: |  |  | Inclusion of Handover requirements for Release 2000, based on TR 22.976 v2.0.0 section 7.7. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 3, 4, 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

# 3	Definitions and Abbreviations

## 3.1	Definitions

For the purposes of the present document, the following definitions apply:

Connection mode (for a bearer service): characterises the type of association between two endpoints as required by the bearer service for the transfer of information. A bearer service is either connection-oriented or connectionless. In a connection oriented mode, a logical association called connection needs to be established between the source and the destination entities before information can be exchanged between them. Connection oriented bearer services lifetime is the period of time between the establishment and the release of the connection.

Connectionless (for a bearer service): In a connectionless bearer, no connection is established beforehand between the source and the destination entities; the source and destination network addresses need to be specified in each message. Transferred information cannot be guaranteed of ordered delivery. Connectionless bearer services lifetime is reduced to the transport of one message.

GSM coverage: an area where mobile cellular services are provided in accordance with GSM standards

UMTS coverage: an area where mobile cellular services are provided in accordance with UMTS standards.

Multi mode terminal: UE that can obtain service from at least one UTRA radio access mode, and one or more different systems such as GSM bands or possibly other radio systems such IMT-2000 family members.

Handover: The process in which the radio access network changes the radio transmitters or radio access mode or radio system used to provide the bearer services, while maintaining a defined bearer service QoS.

Intra PLMN handover: Handover within the same network, i.e. having the same MCC-MNC regardless of radio access system. Note: this includes the case of UMTS <>GSM handover where MCC-MNC are the same in both cases.

Inter PLMN handover: Handover between different PLMNs, i.e. having different MCC-MNC.

Inter system handover: Handover between networks using different radio systems , e.g. UMTS – GSM.

UTRA Radio access mode: the selected UTRA radio access mode i.e. UTRA-FDD; UTRA-TDD.

Radio system: the selected 2nd or 3rd generation radio access technology, e.g. UMTS or GSM.

Service continuity: handling of changes (e.g. bearers) in the network such that the user is provided with the experience of an active service (e.g. voice) being seamlessly maintained. In general Service continuity is supported by handover or equivalent procedures.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

UE		User equipment

# 4	General Principles governing service continuity

This section describes the general principles for service continuity within the UTRAN, within the GERAN and between the UTRAN and other radio systems such as GERAN. As a principle, the requirements on service continuity characteristics should be according to the network on which the service is maintained.

## 4.1	Service Continuity Scenarios

Service continuity shall support the following scenarios:

- Continuity of active CS services when moving within UTRAN, within GERAN and between UTRAN and GERAN coverage areas.

- Continuity of active IM services when moving within UTRAN within GERAN coverage areas (that support IM services)

- Continuity of active IM services when moving from UTRAN and GERAN coverage areas (that support IM services) to GERAN coverage area (that does not supporting IM services).   In this case, at least the voice component of the IM service shall be maintained.  This scenario is required to support operators’ network rollout, e.g islands of UTRAN R00 amongst GERAN R99 or earlier releases of GSM.

- Continuity of active CS services when moving from GERAN coverage areas not supporting IM services to UTRAN or GERAN coverage areas supporting IM services to enable the user to invoke IM services.

- Continuity of active GPRS sessions when moving within UTRAN, within GERAN and between UTRAN and GERAN coverage areasFor all scenarios, the specifications shall cover both service continuity within the same PLMN (intra-PLMN) and between PLMNs (inter-PLMN).

Additionally, if a serving network for whatever reason is no longer able to provide the currently invoked tele- or IM-services over the currently assigned bearer(s) but can provide equivalent services (fully or partially) over a different bearer to the user, it shall support the transition of the original tele- or IM-service to the equivalent service.  This transition shall proceed in such a way that a user is provided with the experience of a service (e.g. voice) being maintained.

4.2	Service Continuity requirements

The table and text in this section show the service continuity requirements from the scenarios described above.

### Service continuity for CS and IM services

| Service continuity required? |  | To CS services |  | To IM Services |  | To CS ANSI 136 basic services |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | UTRAN | GERAN | UTRAN | GERAN |  |
| From CS Services | UTRAN | Yes | Yes | Yes (note 1) | Yes (notes 1,3) | Note 4 |
|  | GERAN | Yes | Yes | Yes (note 1) | Yes (notes 1,3) | Note 4 |
| From IM services | UTRAN | No | Yes (note 2) | Yes | Yes (note 3) | Note 4 |
|  | GERAN | No | Yes (note 2) | Yes (note 3) | Yes (note 3) | Note 4 |
| From CS ANSI 136 basic services |  | Note 4 | Note 4 | Note 4 | Note 4 | Out of scope |

Table 1: Service continuity requirements between CS services and IM services

The following notes apply to table 1:

Note 1:	Seamless service continuity of the voice medium of a voice call to an IP multimedia call is required.

Note 2: 	Seamless service continuity of the voice medium of an IP multimedia call to a CS call is required.

Note 3: 	Service continuity of IM Services is not applicable to and from GERAN Release 99 as the IM Services service classes are not supported by GERAN Release 99. Note that the "voice-only" service is seen as a special case of the IP Multimedia service.

Note 4: 	Requirements dependent on the result of the associated work item (UMTS Interworking with ANSI-41 networks).

In addition to the requirements in table 1, service continuity is required from CS Multimedia services (UTRAN and GERAN) to CS Multimedia services (UTRAN and GERAN).

### Service continuity for GPRS

Service continuity of GPRS sessions (conversation, streaming, interactive and background) is required within the GERAN, within the UTRAN, and between the UTRAN and GERAN.

Note: 	Service continuity for conversational, streaming and interactive GPRS sessions is not applicable to and from GERAN Release 99, due to the lack of support for the conversational, streaming and interactive GPRS within  GERAN Release 99. In the case of continuity to GERAN R99, although the QoS required may not be available in the tareget, the bearer shall be maintained as long as possible. The user makes the decision whether to continue with the call based on the application behaviour.

### Service continuity between CS Services and GPRS, and between IM Services and GPRS

Service continuity is not applicable in the following scenarios:

- between CS Services and GPRS

- between IM Services and GPRS.

Note that this is considered not applicable from a user perspective; for example continuity of a IM Service to a GPRS bearer is not applicable from the user’s perspective, although the GPRS bearer may support the IM Service.

The handover matrix

oos = out of scope of UMTS specifications

1= supporting standards required for UMTS  release 99.

x= supporting standards required, not necessarily for release 99.

4.1	Requirements for Service Capabilities

UMTS standardises service capabilities, not services. As part of the service capabilities it is envisaged that applications may wish to respond to events related to handover that either has occurred, is about to occur or could potentially occur. The service capabilities described in this section should be available at least to UE hosted applications.

The following list is of uses is exemplary and is not intended to be exhaustive:

-	An application may wish to accept or reject offered QoS;

-	An application may wish to cope to the effect that handover has on a service, for example facsimile retransmission;

-	An application may wish to preferentially choose radio resources, for purposes such as SoLSA.

It is therefore required that the service capability set available to an application be able to provide an indication that handover has occurred or could occur with information about the type of handover and radio resources involved. The service capabilities should support QoS negotiation.

### 4.2	General Operational Considerations

### 4.2.1	Coverage environment

Mechanisms defined to support service continuity between UMTS and other radio systems (such as other IMT 2000 family members, or GSM) or UTRA radio access modes should effectively cope with a number of coverage scenarios:

-	Limited UMTS coverage in a 'sea' of coverage provided by another radio system or UTRA radio access mode,  or, vice-versa;

-	Selective operation at a geographical boundary, with extensive UMTS coverage on one side and extensive coverage from another radio system on the other side;

-	Geographically co-located areas of UMTS coverage and another radio system.

However the standards should impose no restrictions or assumptions on how an operator might deploy or operate the network in both GSM and UMTS.

### 4.2.2	Inter PLMN Handover Issues

Handovers to support service continuity between PLMNs should remain an optional feature to implement. It is envisaged that handover would take place due to changing radio conditions caused e.g. by movement of the terminal causing it to leave the coverage area of a PLMN.

The following networks may be involved with an inter-PLMN handover procedure. These concepts are illustrated in Annex A:

-	The user’s home network, i.e. the operator where the user’s subscription may be found;

-	The user’s visited network where the subscriber user is currently registered, i.e. the network where the subscriber user has performed the last successful update location procedure. As long as the subscriber user is roaming within the home network, home and visited network are identical;

-	The user's serving network covering the cell that serves the subscriber. After successful completion of the update location update procedure, the serving network is identical with the visited network. After an inter-PLMN handover, the visited network is different from the serving network until a location update procedure has been successfully completed (excepted the case that the subscriber returns into the visited network);

-	The target network covering candidate target cell(s) for inter-PLMN handover. The target network has overlapping radio coverage with the serving network but not necessarily with the visited network.

The minimum requirements for inter-PLMN HO are:

-	Continuity of an active call across the handover procedure, where this would be possible for intra-PLMN handover;

-	Charging, billing and accounting for inter-PLMN handover should be according to the principles defined in [3]. For R'99 the mechanisms currently used in GSM should be provided as a minimum (charging for handover leg is based on vistited network tariff, etc., settlement between operators is based on bulk metering, etc.);

-	The ability to check with the home network whether the user is permitted to handover  from the visited network to a target network;

-	The decision whether the handover request is accepted must be taken by the target network;

-	Invocation of the handover procedure only occurs if the target network provides the radio channel type required for the respective call;

-	The avoidance of "network hopping", i.e. successive handover procedures between neighbouring networks for the same call;

-	The possibility of user notification of inter-PLMN HO (e.g. possible tariff change) when it occurs.

For R99 there can only be one target PLMN for HO in addition to the serving PLMN for R99 in a given geographical area.

### 4.2.3	Charging and Network Management

Means shall be standardised which allow charging records to record the time of handover in the case of inter-PLMN operator handover. Charging records must be able to reflect the level of UTRA radio access, operation mode and network type after handover.

A capability to provide network management information relating to frequency of occurrence and type of handover should be defined.

### 4.2.4	Cost and efficiency

The UTRAN standards shall facilitate the cost-effective implementation both on the network and on the terminal side, of multi mode operation between GSM and UMTS. Impacts on the GSM network shall be minimised. Such handover shall not require user intervention.

### 4.2.5	Security

Security requirements relating to handover shall be elaborated in a separate document ([4]), but should embody the principle that handover shall not compromise the security of the network providing the new radio resources; the (possibly different) network providing the original radio resources; and the terminalUE. The security mechanisms should also cater for appropriate authentication processes and meet the requirements of national administrations in terms of lawful interception.

## 4.3	Performance Requirements

### 4.3.1	Temporary degradation of service caused by handover

During intra UMTS handover or handover from UMTS to GSM, degradation shall be no greater than during intra GSM handover.

The duration of the discontinuity experienced by PS and CS real time services should be shorter than that in the handover of GSM CS speech calls.

Table 2 in the section below give the absolute requirements for IP multimedia services and GPRS.

### GERAN/UTRAN handover performance requirements for IP multimedia service and GPRS

| Service Classes | Application | Degree of symmetry | Data rate | Key performance parameters and target values |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Max. service interruption (Audio/Video) mute) | User Data Transmission Requirement |  |  |
|  |  |  |  |  | Max. data trans. Delay | Max. service impact | Information loss |
| Conver-sational Class | Telephony | Two-way | 4-25 kb/s | <=150ms | TBD | NA | TBD |
| Streaming Class | Streaming audio | One-way | 32-128 kb/s | TBD | TBD | NA | TBD |
|  | Streaming video | One or two way | 32-384 kb/s | TBD | TBD | NA | TBD |
|  | Bulk data transfer/retrieval | One-way |  | NA | TBD | TBD | None |
| Interactive Class | Interactive best effort data | Two-way |  | TBD | TBD | TBD | None |
| Background Class | Best effort data | One-way |  | NA | TBD | TBD | None |

Table 2: GERAN/UTRAN service continuity performance requirements for IP multimedia and GPRS

TBD =		To Be Determined. It is very important to get these TBD replaced with true value as soon as possible

NA =	Not Applicable

None =	Not allowed

****   NEXT MODIFIED SECTION    ****

# 7	Requirements for Handover from GSM to UMTS

## 7.1	Operational Requirements

### 7.1.2	GSM bands

The standard shall support handover from any combination of GSM bands supported by the GSM standards.

## 7.2	Performance Requirements

During handover from GSM to UMTS,  temporary degradations shall be no greater than for GSM to GSM handovers.

## 7.3	Specific Requirements for Individual Service Handover from GSM to UMTS

### 7.3.1	Speech

AMR, EFR, FR and HR calls shall be mapped to the default UMTS speech codec.

### 7.3.2	Short Message Service

There are no requirements related to handover for short message service.

### 7.3.3	Cell Broadcast

There are no requirements related to handover for cell broadcast.

### 7.3.4	USSD

In GSM, USSD is a connection mode teleservice according to the definition above: in USSD the association between endpoints is called a 'transaction' ([5]). In GSM, USSD transaction from the UE can terminate at the local MSC, the VLR or the HLR. It is not required to standardise means to handover into UMTS of transactions with the local MSC. The need to standardise handover of transactions with the VLR and HLR is for further study.

### 7.3.5	Facsimile

It is not required that a facsimile transmission that is active between UE and network at the time of handover from GSM to UMTS completes successfully.

### 7.3.6	GSM Data Bearer Services

#### 7.3.6.1 Circuit Switched Data

Note: 	The requirements in this section should not delay the release 99 standardisation process and may need review.

Standards shall be defined to permit the possibility of handover of a GSM circuit switched data bearer to UMTS which shall result in a UMTS connection oriented data bearer service. Means shall be provided for the application to indicate minimum acceptable QoS for service continuation after handover. If this cannot be provided by theUMTS network handover will not take place (which may result in call loss once the UE moves outside GSM coverage).

Means shall be defined which make it possible to limit any temporary degradation on handover so it meets the performance specified by GSM service requirements for circuit switched data handover.

#### 7.3.6.2	Packet Switched Data

It is required to handover a user context between GPRS and UMTS. Independently of the used radio interface, the user shall stay connected to an external network (internet, intranet). Any change in the QoS shall be seen at the service access points as a network initiated renegotiation of QoS. If the supported QoS is not acceptable, the MS may terminate the connection/context.

Means shall be defined which make it possible to limit any temporary degradation on handover so it meets the performance specified by GSM service requirements for packet switched data handover.

### 7.3.7	Supplementary services

Where a GSM supplementary services is supported in the target UMTS network then the technical standards should allow handover from GSM to UMTS to have no effect on that service, at least where the GSM and UMTS networks have the same network operator.

## 7.4	Requirements on multiple bearer services handover from GSM to UMTS

Consideration must be given to services that may involve the use of multiple bearer services. For example Class A GPRS terminals will be capable of simultaneously supporting more than one data bearer services. The mapping between GSM/GPRS bearer services and UMTS bearer services will depend upon many factors such as data rate, delay constraints, error rate etc. Means shall be defined to allow handover of several data bearer services from GSM to UMTS. Means shall be defined for the application(s) to indicate minimum acceptable QoS for services continuation after handover.

Nick Sampson, Orange PCS Ltd