3G TS 22.XXX V0.1.0 (2000-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

IP multimedia services (Stage 1);

(Release 2000)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

3GPP, SA

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2000, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA,TTC).

All rights reserved.

Contents

Foreword	4

1	Scope	5

2	References	5

2.1	Normative references	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4 	Introduction	7

5	IP multimedia services principles	7

6 	IP multimedia services requirements	7

6.1	High level requirements	7

6.2	IP Multimedia call requirements	8

6.3	Personalisation and administration	8

6.4 	Multipoint and broadcast requirements	9

6.5	Naming, numbering and addressing	9

7	Service capabilities	9

7.1	Standardised service capability approach	9

7.2 	Benchmark IM supplementary services	10

8	Provisioning of IP multimedia services	10

Annex A 	Example IP multimedia call scenarios (informative)	10

Annex B 	Example supplementary service scenarios (informative)	11

B.1	Barring Services	11

B.2	Call Forwarding	11

B.3	Advice of Charge	12

B.4	Pre-paid Service	12

History	12

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

This TS describes the requirements for the support of IP multimedia services.

IP multimedia services are the new teleservices, including for example IP voice telephony, real time multimedia applications and shared online whiteboards etc.  This TS describes how these IP multimedia services are supported by terminal and network capabilities.

The minimum level of standardisation required to support IP multimedia services is identified.  The existing legacy supplementary services shall not be re-standardised for IP multimedia services, however the same terminal and network capabilities used to build new services shall also be able available to recreate the legacy supplementary services.

IP multimedia services use GPRS as a bearer.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

- A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

## 2.1	Normative references

[1]	TS 22.003: " CS Teleservices supported by a PLMN".

[2]	TS 22.011: "Service Accessibility".

[3]	TS 22.060: "General Packet Radio Service (GPRS) stage 1".

[4]	TS 22.066: "Support of Mobile Number Portability (NMP)".

[5]	TS 22.101: "Service principles".

[6]	TS 22.105: "Services and Service Capabilities".

[7]	TS 22.121: "Virtual Home Environment (VHE), Stage 1".

[8]	TS 22.129: "Handover requirements between UMTS and GSM and other Radio Systems".

[9]	RFC2543: "SIP: Session Initiation Protocol"

[10]	TS 22.078: ”; Customised Applications for Mobile network Enhanced Logic (CAMEL); Service definition - Stage 1”

[11]	TS 22.057: ”; Mobile Station Application Execution Environment (MExE); Service description, Stage 1”

[12]	TS 22.121: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; The Virtual Home Environment"

[13]	TS 22.038: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; USIM/SIM Application Toolkit (USAT/SAT); Service description;Stage 1"

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of this TS, the following definitions apply:

Call : a logical association between two or more users.

IP multimedia service: IP Multimedia Services that require support on the Call Control level carried on top of GPRS (this may include an equivalent set of services to the relevant subset of CS Services).

IP telephony: a voice call that uses IP based protocols (e.g. SIP) and IP for transport of all user data and signalling data. IP telephony is an example of an IP multimedia service which uses only a single medium.

IM CN subsystem: (IP Multimedia CN subsystem) comprises all CN elements for provision of IM services

Multimedia service: a service that handles one or more media simultaneously such as speech, audio, video and data (e.g. chat text, shared whiteboard) in a synchronised way from the user's point of view. A multimedia service may involve multiple parties, multiple connections, and the addition or deletion of resources within a single communication session. Multimedia services may be provided using either circuit switched domain, or packet switched domain, or a combination of circuit switched and packet switched domains.

Supplementary service : is a service which modifies or supplements a basic telecommunication service. Consequently, it cannot be offered to a user as a standalone service. It shall be offered together with or in association with a basic telecommunication service. The same supplementary service may be common to a number of basic telecommunication services.

Teleservice; is a type of telecommunication service that provides the complete capability, including terminal equipment functions, for communication between users according to standardised protocols and transmission capabilities established by agreement between service providers.

## 3.2	Abbreviations

For the purposes of this TS, the following abbreviations apply;

API	Application Programming Interface

CAMEL	Customised Application for Mobile Enhanced Logic

CN	Core Network

CS	Circuit Switched

GPRS	General Packet Radio Service

IM	IP Multimedia

IP	Internet Protocol

MExE	Mobile station application Execution Environment

OSA	Open Service Architecture

O&M	Operations and Maintenance

QoS	Quality of Service

SAT	SIM Application Toolkit

SIP	Session Initiation Protocol

UE	User Equipment

VHE	Virtual Home Environment

WWW	World Wide Web

# 4 	Introduction

IP has opened up a whole range of communication applications, which may allow service providers to develop totally new value added services as well as to enhance their existing solutions. An example of this is currently being experienced on the Internet, where new services are developed and introduced at a pace never seen before in the telecommunications industry.  The open architecture and platforms supported by IP and operating systems may lead to applications and new opportunities that are more difficult to replicate using a standard switched centralised solution. Thus, the main drivers are the new services as the plain CS voice telephony services are gradually superseded by IP multimedia services.

A complete solution for the support of IP multimedia services (including IP Telephony) shall be available. The solution, consists of terminals, GERAN or UTRAN radio access networks and GPRS evolved core network. One of the main objectives for 3GPP specifications is to ensure that the availability and behaviour of these IP applications when used via the 3GPP mobile access is at least as good as when used via other mobile access types.

# 5	IP multimedia services principles

This TS, in general, does not standardise the IP multimedia services, but instead identifies the requirements to enable their support.

Descriptions of IP multimedia service shall be used to identify new standardised service capability requirements.  The IP multimedia services are thus indirectly described via requirements on the network and service capabilities. The IP multimedia services themselves are not standardised, and instead are created using the standardised network and service capabilities.

The existing supplementary services shall in general not be standardised as IP multimedia services, however some equivalent IP multimedia services (e.g. emergency services) shall be standardised.

In order to align IP multimedia services wherever possible with non-3GPP IP services, the general requirement shall be to have compatibility with those non-3GPP IP based services, however this shall be evaluated case by case.

IP multimedia services belong to teleservices as specified in 22.105[6].

Editor's note: The attribute method used for CS teleservices 22.003 [1] is not applicable to IP multimedia services. For IP multimedia services and CS speech teleservice (TS11) interactions needs to be further discussed.

# 6 	IP multimedia services requirements

## High level requirements

The support of IP multimedia services shall be enabled in a flexible manner to allow service providers to differentiate their services in the market place as well customise them to meet specific user needs.  This shall be provided by the use of service capabilities in both networks and terminals, for the creation and support of IP multimedia services.

The following high level requirements shall be supported for IP multimedia services:-

- the IM CN subsystem shall provide sufficient standardised service capabilities to develop and implement circuit switched services which may be required as equivalent IP multimedia services that would be transparent to the end user, with enhanced capabilities and improved user interfaces.

- the IM CN subsystem shall enable service compatibility and access independence, and it shall be possible to implement non-3GPP IP multimedia services to be compatible with the same services when used via other types of accesses, e.g. via fixed lines. 3GPP shall specify IM CN subsystem using GPRS.

- the IM CN subsystem shall enable provision of IP multimedia services with the same (or greater) quality of service as their equivalent GPRS and circuit switched services (if they exist). Selected solutions shall respect the spectrum efficiency (e.g. over the air interface, potential use of header stripping/compression techniques etc.).

- the IM CN subsystem shall enable support of negotiable QoS for IP multimedia services both at the time of a connection setup as well as during the connection

- the IM CN subsystem shall enable the ability to provide, on an end to end basis, a Quality of Voice at least as good as that achieved by the circuit-switched (e.g. AMR codec based) wireless systems

- the IM CN subsystem shall enable the possibility for IP multimedia services to be provided without a reduction in privacy, security, or authentication compared to corresponding GPRS and circuit switched services.

- the IM CN subsystem shall support interworking between the packet and circuit switched services, and with PSTN, ISDN and Internet.

- service accessibility and roaming shall be supported as described in TS 22.011[2].

- service continuity and handovers shall be supported as described in TS 22.129[8].

- emergency services shall be supported as described in TS 22.101[5].

- the IM CN subsystem shall enable support of O&M functionality (e.g. operations, administration, maintenance, provisioning etc.) by the IP capabilities of the network

- the IM CN subsystem enable support of charging and billing

- the routing of user data and signalling shall be optimised between different networks to support IP multimedia  services without quality degradation through the use of two international call legs (e.g. minimal tromboning).

## 6.2	IP Multimedia call requirements

IP multimedia calls provide the ability to make and receive (where applicable) voice, emergency and data calls, even when roaming. This includes interworking with existing voice and data networks for both fixed and mobile users, addressed using the standard phone numbers, e.g. E.164.

The following IP multimedia call requirements shall be supported:-

- the user shall be able to set-up, re-negotiate (i.e. add or delete new media components) and clear IP multimedia calls.

- IP multimedia calls shall support a large variety of different media types. A mandatory codec shall be selected for most used media types, e.g. voice.

- IP multimedia services shall be able to support the user with a consistent behaviour regardless of how they are accessed (e.g. menu on phone, WWW page from home or the office, or voice activated services, in the home network or in a serving network).

- IP Multimedia services shall be able to personalise support of individual media components of a multimedia call in a user profile, and not necessarily handle all media components of a multimedia call in the same way.  An example may be a multimedia call where the voice medium is forwarded to the user and the video medium forwarded to an appropriate terminal or storage device.

- Call completion shall not simply be based on routing to a destination, but take into account the personalised media requirements of the involved parties (e.g. media splitting, media translation etc.).  Interaction with the called user's profile shall be supported, or alternatively direct interaction with the called party may be required to allow for user selection on how the call is completed.

Note: 	Example IP multimedia call scenarios are presented in Annex A.

## 6.3	Personalisation and administration

The IM CN Subsystem shall support the personalisation of IP multimedia services, and it shall be possible to:-

- enable service providers to offer IP multimedia services that allow the user to personalise the services within the user's service environment in a user profile

- enable users to provision and activate their IP multimedia call handling services in various ways (e.g. for all subsequent calls, for the next call etc.)

- enable modified handling of an IP multimedia call as a result of the user's context (e.g. called party's communications capabilities, user's environment etc.) which may be managed by the user (e.g. in a user profile), or may be pertinent to the network

- use the user's context to process IP multimedia calls in the serving and home networks, calling and called party's terminals, and by third parties

- personalise IP multimedia services  to support user and terminal mobility (e.g. a mobile user should be able to select which terminal to use)

## 6.4 	Multipoint and broadcast requirements

Support for a service similar to the CS domain SS Multiparty service shall be required as an IP multimedia service.

Editor's note: Multipoint, Broadcast, Multicast, Multiparty, ASCI etc. types of issues needs to be further studied. Should these be left to rel 2001?

## 6.5	Naming, numbering and addressing

The IM CN subsystem shall enable support of both telecom and internet numbering and addressing schemes. IP multimedia call establishment (both mobile originating and terminating) depending on originator shall be able to be based on E.164  (e.g. +1 23 456 789) or SIP URL (sip:my.name@company.org) [9].

It shall be possible to use same E.164 number for IP multimedia service and CS speech telephony (TS11) [1].  Also different E.164 can be used if wanted.

Number portability for E.164 numbers in the IM CN subsystem for IP multimedia service shall be provided as specified in 22.066 [4].

Editor's note: The 22.066 needs to be reviewed.

Note: 	It is also possible to support "name portability" for IP multimedia services using non-3GPP IP mechanisms.

# 7	Service capabilities

## 7.1	Standardised service capability approach

IP multimedia services (including IM supplementary services) shall, as a principle, not be standardised, allowing service provider specific variations. IP multimedia services can be created using the standardised service capabilities of CAMEL 22.078 [10], MExE 22.057 [11], OSA 22.121 [12] and SAT 22.038 [13] and/or IP mechanisms  in the terminal and network. Some of the IM supplementary services may also exploit functionality at the IP call control level.

Reuse of already implemented applications and services is also important.  CAMEL, MExE, SAT and OSA, which are the standardised service capabilities of VHE in 22.121 [12], shall be improved to support IP multimedia services, e.g. additions to APIs, service capability features, service capability servers, user profile etc.

The following options shall be available in the standards to enable service delivery:

- capability toolkits enhanced to control IP multimedia services, which will allow applications to be deployed in a vendor independent manner

- a framework that enables toolkits not standardised by 3GPP to be used to deliver services (e.g. adoption of IP recommendations to facilitate the IP applications)

- registration mechanisms which allow the network to understand the limitations of the mobile and thereby take appropriate actions.

Note: 	There is a concern that with a large variety of toolkits to create services, service interworking between terminals and networks may be compromised and needs to be addressed.

## 7.2 	Benchmark IM supplementary services

The following IM supplementary services shall be considered as benchmark services for the service capabilities, and are assumed to be widely available in some form.

- IM Call Barring (see B.1)

- IM Call Forwarding  (see B.2)

- IM  Call Transfer

- IM  Call Deflection

- IM Call Hold

- IM Call Waiting

- IM Call Advice of Charge  (see B.3)

- IM Call Caller Identification and restriction

- IM Call Connected Line Identification and restriction

- IM pre-paid service (see B.4)

# 8	Provisioning of IP multimedia services

There are no standardised subscription mechanisms related to any IP multimedia services.

IP multimedia services will require provisioning and configuration by users and service providers. Since the range of services and the services themselves are not standardised, the specific feature codes to provision, enable and configure them cannot be standardised either. Thus there are no requirements on the network capabilities to support provisioning and configuration for specific IP multimedia services.

The standardised service capabilities, personalised Internet web pages and evolving IP mechanisms shall be used to allow user (self) provisioning, configuration and enabling of IP multimedia services.

# Annex A 	Example IP multimedia call scenarios (informative)

The following example scenarios describe the personalised handling of individual media in multimedia services (note that this list is neither complete nor exhaustive):-

- The user is in a voice call, and receives an incoming IP video call. The user decides not to accept the call, but diverts the incoming video to a messaging system.  Further, the user is given an indication that there is a video message in his mail box

- The user is in a voice call, and receives an incoming video call. The user decides to accept the call but wishes to switch between the two calls.

- The user is idle in a network and not involved in a call.  The user modifies his user profile to divert all voice calls other than those from high priority, pre-identified callers (e.g. his boss).  In this scenario all emails and text messages continue to be received regardless of the sender.

- On receiving a call, the calling party's identity is displayed (if not restricted) and user shall be able to decide whether to accept the call, or divert to a messaging system.  The user shall be able to request media handling of the call (e.g. media splitting to different destinations, media conversion).

- The user is busy in a call when receiving an incoming call, but responds to the calling party that he will return the call later.  The user may request that the calling party's details (if not restricted) are stored with a reminder in user's profile.

# Annex B 	Example supplementary service scenarios (informative)

This annex investigates the existing supplementary services, and identifies how they may be provided as equivalent IP multimedia services in a non standardised way by using the standardised service capabilities (i.e. CAMEL, MExE, OSA and SAT).

## B.1	Barring Services

This category of services performs two basic functions:-

- limiting the user's usage of subscribed services (e.g. no roaming, no long distance, session barring, bearer barring, QoS etc.)

- bar calls to control the cost of the calls

- bar incoming calls

The requirements for barring services are oriented less at limiting those basic services which a subscriber is subscribed to, and more at simple filtering of defining which services are made available to users (e.g. by defining their menu options on the terminal, network application etc,) and blocking of specific basic services to number ranges.

Specifying which services are available to users can be done through WAP, MExE, SAT, CAMEL and OSA toolkits.  Outgoing barring services for basic services can be implemented using SIM Toolkit applications based on the number dialled, CAMEL and HLR (e.g. serving network applications), and MExE applications (e.g. MExE services, WAP WTA applications etc.). Incoming call barring services for basic services can be implemented using the network (e.g. CAMEL and HLR), and MExE applications (e.g. MExE services, WAP WTA applications etc.).  Operator Determined Barring may also be supported using these toolkits.  The barring of specific basic services with toolkits requires further investigation.  Therefore, by using service capabilities there is no requirement for a specific call barring service to be standardised.

## B.2	Call Forwarding

This category of services include immediate call forwarding, call forwarding on no reply, call forwarding on not reachable, call forwarding on busy, call forwarding on user initiation and call completion to busy subscriber.  Further, the diversion may also be applied depending on the type of service (e.g. speech, data, multimedia, by media type etc.).

The features which are required in the network (e.g. provided by CAMEL) are:

- immediate call forwarding per medium type

- call forwarding on no reply per medium type

- call forwarding on not reachable per medium type

- a set of primitives that allows the terminal to:

- be notified of incoming calls (including when already engaged in a call)

- hold/transfer/accept any of the incoming calls to another destination

- be notified of success or failure of these actions

Therefore no basic call forwarding features other than those proposed above are required to be standardised.

This allows terminals to be capable of providing the call forwarding features when reachable (with the service logic securely downloaded using MExE, building on WAP's WTA where available), and the home network/serving network to handle call forwarding when the terminal is unreachable (using HLR/CAMEL).

The basic primitives may also provide the capability to offer call waiting, hold and transfer features through the MExE toolkit.  For the case where the terminal is reachable, the selection of the terminal or the network to process the call forwarding may be an option.

## B.3	Advice of Charge

A feature in networks is advice of charge, which is based on the serving network being aware of the teleservice in use, the price for it and the mark-up used in the home network.

In future, the price charged to a subscriber may bear little relation to the charges imposed by the serving network because there may be special offers/discounts, or the service may include elements charged elsewhere (e.g., content charged by a 3rd party).  Charging related information may also be made possible to users in advance of the call being made, in order to give an indication of the potential call costs.

Therefore the advice of charge may originate from the (mostly IN-based) charging services and delivered to the user by various means. Displaying of the received charging information to the user could be with MExE, as well as re-using capabilities to transport the Advice of Charge Information to the terminal and executing an AoC application in the terminal (as defined in 22.024). Different methods may also exist.

## B.4	Pre-paid Service

The pre-paid service allows the subscriber to pay for telecommunication services prior to usage.

A pre-paid service subscriber establishes an account with the service provider to access telecommunications services in the home and roamed-to networks.  Charges for telecommunication services are applied to the pre-paid service account by decrementing the account in real time.  The pre-paid service subscriber may be notified about the account information at the beginning, during, or at the end of the telecommunications service.  When the account balance is low the subscriber may be notified so that the subscriber may refill the account.  When the account balance is below a pre-defined threshold, the subscriber’s use of telecommunications services may be barred.

The support required from the toolkits to support the pre-paid service requires to be identified.

# History

| Document history |  |
| --- | --- |
| Version 0.1.0 | First draft from editor. |