Source: 	Rapporteur

Title:	22.129 CR for Release 2000 (Handover requirements - inter domain service continuity)

Document for:	Discussion and approval

Introduction

The attached document presents a CR to TS 22.129 for release 2000. This CR is applied to version 3.3.0 although it takes into account the contents of a previous CR on handover (S1-00xxxx) that makes changes to subclauses 3,1, 4.1, 4.2, 4.3 and 4.5.2.

Summary of Changes

The following changes have been included:

1.	Removal of reference to certain service continuity scenarios being for further study (subclauses 4.1 and 4.2.1)

2.	Additional text on charging and network management issues (subclause 4.4.3)

3.	Addition of sections on inter-domain handover issues, including candidates for handover and continuity of supplementary services (subclause 4.4.6).

Proposal

It is proposed that this CR is agreed for incorporation in TS 22.129.

| 3GPP SA1 #7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | S1-00???? |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Taastrup, Denmark, 17-21 July 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.129 |  |  |  |  |  | CR |  |  | xxxxx |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.3.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | SA#9 |  |  |  |  | for approval |  |  |  |  |  |  |  |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  | X | Core Network |  |  |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Rapporteur |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 30 June 2000 |  |  |  |  |  |
| Subject: |  |  | Handover requirements for Release 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  | X |  |  |
| Reason for change: |  |  | Inclusion of Handover requirements for Release 2000, based on TR 22.976 v2.0.0 section 7.7. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 4.1, 4.2.1, 4.4.3, 4.4.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

# 4	General Principles governing service continuity

This section describes the general principles for service continuity within the UTRAN, within the GERAN and between the UTRAN and other radio systems such as GERAN. As a principle, the requirements on service continuity characteristics should be according to the network on which the service is maintained.

## 4.1	Service Continuity Scenarios

Service continuity shall support the following scenarios:

1.	Continuity of active CS services when moving within UTRAN, within GERAN and between UTRAN and GERAN coverage areas.

2.	Continuity of active IM services when moving within UTRAN, within GERAN and between UTRAN and GERAN coverage areas (that support IM services)

- Continuity of active GPRS sessions when moving within UTRAN, within GERAN and between UTRAN and GERAN coverage areas.

4.	Continuity of active IM services when moving from UTRAN and GERAN coverage areas (that support IM services) to GERAN coverage area (that does not support IM services). In this case, at least the voice component of the IM service shall be maintained. This scenario is required to support operators’ network rollout, e.g islands of UTRAN R00 amongst GERAN R99 or earlier releases of GSM.

5.	Continuity of active services when transferring from CS service support to IM service support either

a)	when moving from GERAN coverage areas not supporting IM services to UTRAN or GERAN coverage areas supporting IM services or

b)	when the user has initially invoked CS services but is within a coverage area that supports IM services to enable the user to invoke IM services.

For all scenarios, the specifications shall cover both service continuity within the same PLMN (intra-PLMN) and between PLMNs (inter-PLMN).

Additionally, if a serving network for whatever reason is no longer able to provide the currently invoked teleservices or IM-services over the currently assigned bearer(s) but can provide equivalent services (fully or partially) over a different bearer to the user, it shall support the transition of the original teleservices or IM-service to the equivalent service. This transition shall proceed in such a way that a user is provided with the experience of a service (e.g. voice) being maintained.

## 4.2	Service Continuity requirements

The table and text in this section show the service continuity requirements from the scenarios described above.

### Service continuity for CS and IM services

|  |  | To CS services |  | To IM services |  | To CS ANSI 136 basic services (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | UTRAN | GERAN | UTRAN | GERAN |  |
| From CS services | UTRAN | Yes | Yes | Yes (note 1,3) | Yes (note 1, 3) | FFS |
|  | GERAN | Yes | Yes | Yes (note 1, 3) | Yes (note 1, 3) | FFS |
| From IM services | UTRAN | No | Yes (note 4) | Yes | Yes | FFS |
|  | GERAN | No | Yes (note 4) | Yes | Yes | FFS |
| From CS ANSI 136 basic services (Note 2) |  | FFS | FFS | FFS | FFS | Out of scope |
| Note 1: Service continuity of IM Services is not applicable to and from GERAN Release 99 as the IM Services service classes are not supported by GERAN Release 99. Note that the "voice-only" service is seen as a special case of the IP Multimedia service. Note 2: Requirements dependent on the result of the associated work item (UMTS Interworking with ANSI-41 networks). Note 3: Service continuity of the voice call to an IP multimedia call. Note 4: Service continuity of any voice component of an IP multimedia call to CS call. |  |  |  |  |  |  |

Table 1: Service continuity requirements between CS services and IM services

In addition to the requirements in table 1, service continuity is required from CS Multimedia services (UTRAN and GERAN) to CS Multimedia services (UTRAN and GERAN).

### Service continuity for GPRS

Service continuity of GPRS sessions (conversation, streaming, interactive and background) is required within the GERAN, within the UTRAN, and between the UTRAN and GERAN.

Note: 	Service continuity for conversational, streaming and interactive GPRS sessions is not applicable to and from GERAN Release 99, due to the lack of support for the conversational, streaming and interactive GPRS within  GERAN Release 99. In the case of continuity to GERAN R99, although the QoS required may not be available in the tareget, the bearer shall be maintained as long as possible. The user makes the decision whether to continue with the call based on the application behaviour.

## 4.3	Requirements for Service Capabilities

UMTS standardises service capabilities, not services. As part of the service capabilities it is envisaged that applications may wish to respond to events related to handover that either has occurred, is about to occur or could potentially occur. The service capabilities described in this section should be available at least to UE hosted applications.

The following list of uses is provided as an example and is not intended to be exhaustive:

-	An application may wish to accept or reject offered QoS;

-	An application may wish to cope to the effect that handover has on a service, for example facsimile retransmission;

-	An application may wish to preferentially choose radio resources, for purposes such as SoLSA.

It is therefore required that the service capability set available to an application be able to provide an indication that handover has occurred or could occur with information about the type of handover and radio resources involved. The service capabilities should support QoS negotiation.

## 4.4	General Operational Considerations

### 4.4.1	Coverage environment

Mechanisms defined to support service continuity between UMTS and other radio systems (such as other IMT 2000 family members, or GSM) or UTRA radio access modes should effectively cope with a number of coverage scenarios:

-	Limited UMTS coverage in a 'sea' of coverage provided by another radio system or UTRA radio access mode,  or, vice-versa;

-	Selective operation at a geographical boundary, with extensive UMTS coverage on one side and extensive coverage from another radio system on the other side;

-	Geographically co-located areas of UMTS coverage and another radio system.

However the standards should impose no restrictions or assumptions on how an operator might deploy or operate the network in both GSM and UMTS.

### 4.4.2	Inter PLMN Handover Issues

Handovers to support service continuity between PLMNs should remain an optional feature to implement. It is envisaged that handover would take place due to changing radio conditions caused e.g. by movement of the terminal causing it to leave the coverage area of a PLMN.

The following networks may be involved with an inter-PLMN handover procedure. These concepts are illustrated in Annex A:

-	The user’s home network, i.e. the operator where the user’s subscription may be found;

-	The user’s visited network where the subscriber user is currently registered, i.e. the network where the subscriber user has performed the last successful update location procedure. As long as the subscriber user is roaming within the home network, home and visited network are identical;

-	The user's serving network covering the cell that serves the subscriber. After successful completion of the update location update procedure, the serving network is identical with the visited network. After an inter-PLMN handover, the visited network is different from the serving network until a location update procedure has been successfully completed (excepted the case that the subscriber returns into the visited network);

-	The target network covering candidate target cell(s) for inter-PLMN handover. The target network has overlapping radio coverage with the serving network but not necessarily with the visited network.

The minimum requirements for inter-PLMN HO are:

-	Continuity of an active call across the handover procedure, where this would be possible for intra-PLMN handover;

-	Charging, billing and accounting for inter-PLMN handover should be according to the principles defined in [3]. For R'99 the mechanisms currently used in GSM should be provided as a minimum (charging for handover leg is based on vistited network tariff, etc., settlement between operators is based on bulk metering, etc.);

-	The ability to check with the home network whether the user is permitted to handover  from the visited network to a target network;

-	The decision whether the handover request is accepted must be taken by the target network;

-	Invocation of the handover procedure only occurs if the target network provides the radio channel type required for the respective call;

-	The avoidance of "network hopping", i.e. successive handover procedures between neighbouring networks for the same call;

-	The possibility of user notification of inter-PLMN HO (e.g. possible tariff change) when it occurs.

For R99 there can only be one target PLMN for HO in addition to the serving PLMN for R99 in a given geographical area.

### 4.4.3	Charging and Network Management

Means shall be standardised which allow charging records to record the time of handover in the case of inter-PLMN operator handover. Charging records must be able to reflect the level of UTRA radio access, operation mode and network type after handover.

The issue of charging and billing for a call or session that is the subject of handover between CS CN domain and IM CN subsystem is for the specific PLMN operator to resolve; it is not an issue for standardisation. Charging records shall, however, provide sufficient information for the operator to identify those calls or sessions that have been the subject of inter-domain handover.

A capability to provide network management information relating to frequency of occurrence and type of handover should be defined.

### 4.4.4	Cost and efficiency

The UTRAN standards shall facilitate the cost-effective implementation both on the network and on the terminal side, of multi mode operation between GSM and UMTS. Impacts on the GSM network shall be minimised. Such handover shall not require user intervention.

### 4.4.5	Security

Security requirements relating to handover shall be elaborated in a separate document ([4]), but should embody the principle that handover shall not compromise the security of the network providing the new radio resources; the (possibly different) network providing the original radio resources; and the terminal UE. The security mechanisms should also cater for appropriate authentication processes and meet the requirements of national administrations in terms of lawful interception.

### 4.4.6	Inter domain handover issues

#### 4.4.6.1	Candidates for handover

The requirement to handover calls between the CS CN domain and the IM CN subsystem shall apply only to active voice calls, that is, only voice calls shall be candidates for inter-domain handover.

In handing over calls from the CS CN domain to the IM CN subsystem, the user terminal should only have a single voice call active at any time (since multicall only supports a single voice/speech call at any time).

In handing over voice calls from the IM CN subsystem to the CS CN domain, any active voice call (session) can be a candidate for handover. Where multiple voice calls (sessions) are established, the decision as to which call (session) is handed over is for further study (FFS).

Any call or session using resources specific to the source domain (e.g. conference capabilities) that cannot be maintained in the target domain shall not be a candidate for handover.

#### 4.4.6.2	Continuity of supplementary services

It is recognised that services (e.g. supplementary services) may either be implemented or provisioned separately between the CS CN domain and the IM CN subsystem. On handing over a call between the two domains, the availability of such services shall reflect those provisioned in the target domain rather than those provisioned in the originating domain.

The specific interaction of voice calls or sessions that are handed over with active supplementary services (e.g. calls on hold) is for further study (FFS).

Nick Sampson, Orange PCS Ltd