TSG- SA1 Ad Hoc IM Subsystem 	Doc No 160

Slough, UK, 6th to 8th September 2000	Agenda Item #3

Source : 	Vodafone Group (contact tony.sammut@vf.vodafone.co.uk)

Title : 	Amendments to the TS 22.228 V0.3.0 “Service requirements for the IP Multimedia Core Network”

Document for : Discussion

Currently there is no text accompanying this Section. 7.3.2 ‘Authentication/registration’

Vodafone’s position is that it assumes that automatic authentication/registration will occur upon the activation of the UE. This should be explicitly stated in this section. The justification for this position is the existence of the IP telephony service (VoIP) which will be provided. Vodafone requires that the system should operate very much like current GSM for this service in particular, whereby the user activates the terminal and can perform the IP telephony service without requiring a manual authentication/registration task.

##### Revised Text

### 7.3.2	Authentication/registration

Editor's note: The objective of this clause is to identify the verification of user authorisation to use network resources.

Upon activation, the UE shall automatically perform registration with the network and the network shall authenticate the MS.

---------------------------

The phrase “most used” is not specific enough in Section 7.5.1. By changing the phrase to ‘most commonly used’ we imply that the media types are widely spread throughout the user community as opposed to media types which are most heavily used by perhaps only a limited section of the user community.

In addition, S1 must decide for which services a standardised codec must be specified. Every TE unit must be equipped with the standard codec upon deployment, users may add other standards at a later time.

Revised Text

### 7.5.1	User modification of media

A standardised capability negotiation mechanism for the UE and the service provider or content provider interact to inform each other of the capabilities each is able to provide for the support of IP multimedia applications shall be provided.  It shall be possible for the capability negotiation to take place at any time during an active IP multimedia session.

It shall be possible for the UE and service/content provider to also spontaneously inform each other of their capabilities (i.e. following a change in UE capabilities, change in media types etc.).

The following IP multimedia session requirements shall be supported:-

- IP multimedia sessions shall support a large variety of different media types. A default codec shall be selected for most commonly used media types to ensure interoperability, e.g. voice. This codec must be supported by every UE unit, users will be able to customise the UE by adding further codec standards to the UE in the future

- IP Multimedia sessions shall be able to personalise support of individual media components of a multimedia application in a uniquely addressable user profile, and not necessarily handle all media components of a multimedia application in the same way.  An example may be a multimedia application where the voice medium is forwarded to the user and the video medium forwarded to an appropriate terminal or storage device.

- the user shall be able to re-negotiate (i.e. add or delete media components) of IP multimedia applications during an IP multimedia session

---------------------------

If the user can direct an IM application or application component to any terminal, as stated in Section 7.7.1 ‘Personalisation and administration’ it must be explicitly stated that the user must be present at the terminal. Otherwise this statement has implications for information security. The reception of a multimedia service or service component should be dependent on the presence of the user’s SIM within the UE, otherwise this should not be permitted, unless the point to which the IM service or component is being directed is network-based.

Revised Text

## 7.7.1	Personalisation and administration

The user personalisation of IP multimedia applications shall be supported, and it shall be possible to:-

- allow the user to personalise the IP multimedia sessions and IP multimedia applications within the user's service context in a uniquely addressable user profile

Editor's note: clarify and properly use the term "user's context": in fact user the term user profile instead of user context…

- enable users to provision and activate their IP multimedia communication handling of IP multimedia applications in various ways (e.g. for all subsequent communications, for the next communication, for specific components of an IP multimedia application etc.)

- use the user's context in a uniquely addressable user profile to process IP multimedia communications in the serving and home networks, originating and terminating entity's terminals, and by third parties

- permit a user to be able to select on which terminal to receive an IP multimedia application, or components of an IP multimedia application, provided that the terminal is fitted with the user’s SIM

Editor's note: this clause needs to be re-considered in the light of application-level personalisation.

---------------------------

In Section 8 ‘Operator/service provider requirements’ the statement specifically recommends requirements for content providers. We understand that this group cannot specify requirements which apply to content providers, this is out of the scope of the group’s remit.

Revised Text

# 8	Operator/service provider requirements

Editor's note: consider alternative term instead of using "service provider"

This clause identifies the requirements for operator/service providers to support IP multimedia applications.

In addition to operator/service providers, content providers may be a further source of additional services to users, providing further opportunities for operator/service provider revenue generating.  Transparent mechanisms for the support of IP multimedia applications to exploit this opportunity shall be utilised.

---------------------------

In Section 8 ‘Rapid service creation and deployment by a operator/service provider’ it specifies that it shall be possible to “permit a mobile user to be able to select which terminal to use”. Our position is that the user must not be able to perform IM services on any terminal unless the user’s SIM is present in the TE. This is necessary for reasons of information security and fraud prevention. The text must be altered such that it explicitly states that this depends upon the presence of the user’s SIM in the TE.

Furthermore the grammar of the title of the section is incorrect and should be amended

Revised Text

## 8.4	Rapid service creation and deployment by an operator/service provider

It shall be possible to:-

- enable operators/service providers to offer IP multimedia applications that may be personalised

- enable modified handling of IP multimedia communications as a result of the user's context (e.g. terminating entity's communications capabilities, originating and terminating user's context etc.) as defined in the uniquely addressable user profile

- enable support of  terminal mobility (e.g. permit a mobile user to be able to select which terminal to use provided that the user’s SIM is present within the terminal under consideration)

Editor's note: this clause needs to be re-considered in the light of application-level personalisation.