Title: 	On the scope of Push Services

Source: 	“Alcatel, Telia, AT&T Wireless, Comverse, Telefonica, Ericsson, Motorola, Nokia, Openwave, Siemens, Sonera, CommWorks, Vodafone, Mannesmann”

Background

This is a contribution to the joint session between SA1, SA2, and T2 on the scope of standardization of push services within 3GPP.

The work on Push Services in TSG SA2 has made some progress on the TR 23.974 at the last SA2#18 meeting, the 14th-18th of May, in Puerto Rico and in the following e-mail discussions.

TSG SA2 has taken the Push requirements received from TSG SA1 into account. The Push group has reached a consensus on the overall architecture to support push services requirements (as highlighted in S2-010624/S1-010260). This is a proxy-based architecture comprised of the following elements:

- Push Access protocol between the Push Initiator (Push Application Server) and the Push Proxy.

- A push transfer protocol handling the push content delivery between the Push Proxy and the UE.

- A Push Proxy performing functions such as access control, charging, UE presence handling, store and forward, user profile management, content adaptation, etc.

Show of hands at the last TSG SA2#18 Plenary meeting in Puerto Rico indicated that a majority of the companies were against introduction of a new feature in GPRS for “Network Requested PDP Context Activation with allocation of dynamic IP address”

Positions

The following is the positions of the contributing companies:

- The attached document, which includes the conclusion section of TR 23.974, constitutes the current e-mail agreement of the TSG SA2 group working on push services (i.e. no company has objected the current text after several requests from the chairman).

- After formal approval of this conclusion section, the push study can be concluded after minor clean up editing of the TR.  Current SA2 plan is to complete the TR23.974 at the SA2#19 meeting, August 27 – 31, 2001 and approved it.  Then, submit the TR along with the modified WI description to the TSG-SA#13 for approval.

- The Multimedia Messaging Service (MMS) shall be seen as one example of an application, which can be based on the push service. The 3GPP MMS work can continue as a separate feature.

Attached documents

S2-0xxxx“Conclusion section of TR 23.974”   &   for information S2-011164 “On the need for NRPCA 
for Push Services”

Push Service WI