TSG-SA Working Group 2 (Architecture )	TSGS2 (99) 300

19 - 22 April 1999		revised TSGS2 (99) 214

Yokohama

# Source: 	Siemens

# Subject: 	Proposed changes to Draft TR on QoS, Sources of requested bearer parameters

# Purpose: 	for decision

## Reason for change

The Technical Report on QoS shall describe all QoS related issues of UMTS services. This paper identifies sources of UMTS bearer service QoS parameters for bearer setup or change. The description is proposed to be added to chapter 6.1.1 into the QoS Technical Report.

Change Request:

# Source of UMTS Bearer Service Parameters

UMTS bearer service parameters describe the service provided by the UMTS network to the user of the UMTS bearer service. A set of QoS parameters (QoS profile) specifies this service. At UMTS bearer service establishment or modification up to four different QoS profiles have to be taken into account:

- The UE capabilities form a QoS profile which may limit the UMTS bearer service which can be provided.

- The UE or the terminal equipment (TE) within the terminating network may request a QoS profile at UMTS bearer establishment or modification. The application using the UE may request the UE to provide a UMTS bearer service with a specific QoS profile. If the application requests no specific QoS the UE may use a QoS profile configured within the UE (e.g., by AT commands). How the TE derives a QoS profile is out of scope for UMTS.

- A QoS profile in the UMTS subscription describes the upper limits for the provided service if the service user requests specific values. Otherwise, this QoS profile may describe a default service requested by the user.

- Default QoS profile(s) may be configured by the operator for the UMTS bearer services provided by the network.