SA WG2 Meeting #113	S2-160059

25 - 29 January 2016, Frigate Bay, Saint Kitts and Nevis, KN	(revision of S2-16aaaa)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.302 | CR | 2924 | rev | - | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Lawful intercept ePDG requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry UK Ltd, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13 |  |  |  |  |  |  | Date: |  | 2016-01-05 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | SA2 received an LS from SA (S2-160054) indicating that the ePDG selection requirements need to be updated to include support for LI. This CR looks at introducing functionality to support the following statements in the LS 1) A Serving PLMN which deploys an ePDG shall be able to control the selection of an ePDG in the Serving PLMN, when this is required by national regulations applicable to the Serving PLMN. The statement implies that the VPLMN must have an ability to require a UE to select an ePDG or not in the VPLMN (see yellow highlight), and assumes that the VPLMN has deployed ePDGs. 2) When the UE is not served by any PLMN or the Serving PLMN has not deployed an ePDG, then a default system behaviour should be defined that allows the local country authorities to perform lawful interception if required. When the UE is attached to a Serving PLMN that has not deployed any ePDGs, this statement implies that the default behaviour of the device must be to choose an ePDG in the country that the UE is roaming in, so that the chances of satisfying potential local LI requirements are satisfied. When the UE is not served by any PLMN this implies that the UE shall select an ePDG in the local country. It also implies that there must be a system behavior based on the UE ability to determine its location then the UE should choose an ePDG in that country. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ePDG selection procedure is enhanced as follows: - The UE considers an indication intriduced in 23.401 in NAS from the VPLMN when connecting to it, as to whether an ePDG shall be selected in the visited country or not - The selection algorithm is modified to consider both the cases in which the UE has received the indication and it ha snot received the indication - enhancements to the configuration from the HPLMN to enable the UE to select an ePDG in the local country when roaming, in case the UE is connected to a VPLMN that has not deployed any ePDGs. The enhancements enable the HPLMN to configure the UE with information regarding ePDGs knowns to the HPLMN in the local country |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA3LI requirements are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.4.2, 4.5.4.3, 4.5.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | It is expected that, if the solution is adopted, a CR to 23.003 is required to add the new FQDN format suggested. |  |  |  |  |  |  |  |  |

####### Start of Changes #######

### 4.5.4	ePDG Selection

#### 4.5.4.1	General

The UE performs ePDG selection based on a set of information configured by the HPLMN in the UE, and based on the UE's knowledge of the PLMN it is attached to.

A UE connected to one or multiple PDN GWs uses a single ePDG.

#### 4.5.4.2	ePDG FQDNs Construction

When the UE attempts to construct an FQDN for selecting an ePDG in a certain PLMN-x (either a VPLMN or the HPLMN), then the UE shall construct one of the following FQDN formats:

-	Operator Identifier FQDN: The UE constructs the FQDN by using the PLMN-x ID as the Operator Identifier. If the UE does not have the PLMN-x ID of an operator in the country it is located, the UE constructs an MCC-only FQDN using the MCC of the country in which it is located.

-	Tracking/Location Area Identity FQDN: The UE constructs the FQDN by using the identity of the Tracking Area/Location Area it is located in (i.e. based on PLMN-x ID and TAC/LAC). The Tracking/Location Area Identity FQDN is used to support location-specific ePDG selection within a PLMN.

The ePDG FQDN formats are specified in TS 23.003 [16].

The UE selects one of the above FQDN formats as follows:

a)	If the UE attempts to select an ePDG in the registered PLMN and the UE is configured to use for this PLMN the Tracking/Location Area Identity FQDN as defined in point 2) of clause 4.5.4.3; and

b)	the UE knows the TAI/LAI of the area the UE it is located in (e.g. the TAI/LAI from the most recent Attach or TAU/LAU),

then the UE constructs a Tracking/Location Area Identity FQDN. Otherwise the UE constructs the Operator Identifier FQDN.

Also, the UE constructs the Operator Identifier FQDN as a fallback in the case of failure of DNS resolution of a Tracking/Location Area Identity based FQDN.

#### 4.5.4.3	UE Configuration By HPLMN

The UE may be configured (e.g. via H-ANDSF, USIM, etc.) by the HPLMN with the following configuration, whose usage is defined in clause 4.5.4.4:

1)	ePDG identifier configuration: It contains the FQDN or IP address of an ePDG in the HPLMN.

NOTE:	The FQDN in the ePDG identifier configuration may have a different format than the one described in clause 4.5.4.2.

2)	ePDG selection information: It contains a list of network identities, which may include the HPLMN, and for each network identity it indicates whether the selection of an ePDG in this network is preferred or mandatory. It also indicates if selection of an ePDG in this network should be based on Tracking/Location Area Identity FQDN or on Operator Identifier FQDN, as specified in clause 4.5.4.4.

The ePDG selection information may include:

- an "any PLMN" entry, which matches any PLMN the UE is attached to except the HPLMN.

- One or more country codes entries, the country code matching the MCC (Mobile Country Code) of the PLMN the UE is attached to.

If the ePDG selection information contains at least two of "any PLMN", the PLMN the UE is attached to, and the MCC the UE is located in, then the UE shall give precedence  to the entries in the following order:

- PLMN code;

- MCC;

- “Any PLMN”.

#### 4.5.4.4	UE ePDG Selection Procedure

When constructing an FQDN, the UE shall contruct the FQDN as described in clause 4.5.4.2.

The UE shall perform ePDG selection by executing the steps below:

1)	If the UE is attached via 3GPP access to a PLMN-x, then:

a)	if the UE receives the Visited ePDG Required Indication (see 3GPP TS 23.401 sub-clause 4.3.5.8b) indicating that the UE shall select an ePDG in the visited PLMN, then

1)	If the UE is configured with the ePDG selection information defined in point 2) in clause 4.5.4.3 and there is:

i)	an entry matching the PLMN-x in the ePDG selection information, then the UE shall attempt to select an ePDG in this PLMN-x. The UE shall construct an ePDG FQDN for this PLMN-x as described in clause 4.5.4.2, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s);

ii)	no entry matching the PLMN-x in the ePDG selection information, then

-	If there is an entry in the ePDG selection information for PLMN-y, different from PLMN-x, matching the MCC of PLMN-x, then the UE shall attempt to select an ePDG in PLMN-y. The UE shall construct an ePDG FQDN for this PLMN-y as described in clause 4.5.4.2, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s);

-	Otherwise, the UE may attempt to select an ePDG in this PLMN-x. The UE shall construct an ePDG FQDN for this PLMN-x as described in clause 4.5.4.2, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s);

iii)	MCC entry in the ePDG selection information corresponding to PLMN-x, then the UE shall attempt to select an ePDG by constructing the Operator Identifier FQDN using the MCC, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s).

2)	If the UE is not configured with the ePDG selection information defined in point 2) in clause 4.5.4.3 then:

A)	the UE shall attempt to select an ePDG in this PLMN-x. The UE shall construct an ePDG FQDN for this PLMN-x as described in clause 4.5.4.2, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s) or

B)	if 2A) fails then the UE shall attempt to select an ePDG by constructing the Operator Identifier FQDN using the MCC, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s);

3)	In all other cases, the UE shall stop the ePDG selection.

b)		If the UE has received a Visited ePDG Required Indication indicating that the UE is allowed to select an ePDG in any PLMN:

- if the UE is configured with the ePDG selection information defined in point 2) in clause 4.5.4.3 and if there is an

i)	entry matching the PLMN-x in the ePDG selection information as defined in point 2) in clause 4.5.4.3, then the UE shall attempt to select an ePDG in this PLMN-x. The UE shall construct an ePDG FQDN for this PLMN-x as described in clause 4.5.4.2, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s).

ii)	MCC entry in the ePDG selection information corresponding to PLMN-x, then the UE shall attempt to select an ePDG by constructing the Operator Identifier FQDN using the MCC, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s).

- If the UE is 	configured with the ePDG identifier defined in point 1) in clause 4.5.4.3, the UE shall attempt to select in ePDG in the HPLMN. The UE shall either use the configured FQDN and use the DNS server function to obtain the IP address(es) of the ePDG(s) in the HPLMN, or the UE shall use the configured IP address.

- Otherwise, 	the UE shall construct an Operator Identifier FQDN for HPLMN and use the DNS server function to obtain the IP address(es) of the ePDG(s) in the HPLMN.

c)	If the UE has not received the Visited ePDG Required Indication (see 3GPP TS 23.401 sub-clause 4.3.5.8b),

1)	if the UE is configured with the ePDG selection information defined in point 2) in clause 4.5.4.3 and there is no entry matching the PLMN-x in the ePDG selection information, then If there is an entry in the ePDG selection information for PLMN-y, different from PLMN-x, matching the MCC of PLMN-x, then the UE shall attempt to select an ePDG in PLMN-y. The UE shall construct an ePDG FQDN for this PLMN-y as described in clause 4.5.4.2, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s); or

2)	the UE shall attempt to select an ePDG by constructing the Operator Identifier FQDN using the MCC, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s); or

3)	otherwise, if the UE is configured with the ePDG identifier defined in point 1) in in clause 4.5.4.3 is:

1)	configured in the UE, the UE shall attempt to select in ePDG in the HPLMN. The UE shall either use the configured FQDN and use the DNS server function to obtain the IP address(es) of the ePDG(s) in the HPLMN, or the UE shall use the configured IP address.

2)	not configured in the UE, the UE shall construct an Operator Identifier FQDN for the HPLMN and use the DNS server function to obtain the IP address(es) of the ePDG(s) in the HPLMN.

2)	If the UE is not attached via 3GPP access to any PLMN,

the UE is configured with the ePDG selection information defined in point 2) in clause 4.5.4.3, the UE can determine the country it is located in and derive the corresponding MCC, and if there is an entry in the ePDG selection information:

a)	for PLMN-y matching the MCC of the local country, then the UE shall attempt to select an ePDG in PLMN-y. The UE shall construct an ePDG FQDN for this PLMN-y as described in clause 4.5.4.2, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s);

b)	matching the derived corresponding MCC, then the UE shall attempt to select an ePDG by constructing the Operator Identifier FQDN using the MCC, and shall use the DNS server function to obtain the IP address(es) of the ePDG(s

NOTE 1:	It is out of scope of the present specification to define how the UE can identify the country it is located in when it's not attached to any PLMN.

NOTE 2:	The detailed ePDG selection mechanism when the ePDG selection information includes more than one PLMN in the country where the UE is located is defined in stage 3 specifications.

3)	In all other cases, the UE shall select the ePDG in the HPLMN as described in points 1)b)i) and 1)b)ii) above.

NOTE 3:	According to the steps above, the home operator can configure the UE to always select an ePDG in the HPLMN, for example, by:

-	providing neither the ePDG identifier configuration nor the ePDG selection information, in which case the UE always selects an ePDG in HPLMN by using the Operator Identifier FQDN, according to step 1)b)ii) above; or

-	providing ePDG identifier configuration only, in which case the UE always selects an ePDG in HPLMN based on the FQDN or IP address in the ePDG identifier configuration, according to step 1)b)i) above.

NOTE 4:	According to the steps above, the home operator can configure the UE to attempt first to select an ePDG in the VPLMN, for example, by providing the ePDG selection information containing the "any PLMN" entry and the indication of "preferred".

If the UE fails to select an ePDG in a VPLMN, then:

-	If the ePDG selection information for this VPLMN contains the "preferred" indication based on point 2) in clause 4.5.4.3, the UE shall select the ePDG in the HPLMN as described in points 1bi) and 1bii) above.

-	If the PDG selection information for this VPLMN contains the "mandatory" indication, the UE shall stop the ePDG selection.

When the UE fails to select an ePDG in the HPLMN, then:

-	The UE shall stop the ePDG selection.

NOTE 5:	If the HPLMN entry in the ePDG selection information contains "preferred" / "mandatory" indications, these indications are not used during the selection of an ePDG in the HPLMN.

If a selected ePDG is not reachable from an untrusted non-3GPP access, the UE shall attempt to reach the next discovered ePDG, if available.