SA WG2 Meeting #115	S2-163193

23 - 27 May 2016, Nanjing, P.R. China	(revision of S2-162346)

Source:	Qualcomm Incorporated

Title:	Updates to Solution 12.1 for key issue 12 on Authentication Framework

Document for:	Discussion/Approval

Agenda Item:	FS_NexGen /6.10.12

Work Item / Release:	FS_NexGen / Rel-14

Abstract of the contribution: Extends solution 12.1 to cover additional aspects.

# Introduction

This paper builds on Solution 12.1 and addresses the following work tasks for Key Issue 12:

- WT#0 Security Architecture Framework

- WT#1: Authentication and Key Agreement

- WT#2: User Identity Confidentiality

- WT#4: System Signalling protection

- WT#5: User data protection

- WT#6: non 3GPP-access

- -WT#8: small data

# Proposal

It is proposed to add the following solution to the TR 23.799 “Study on Architecture for Next Generation System”.

# >>>Start Changes<<<<

### 6.12.1	Solutions 12.1: Authentication and security framework

This solution addressed the key issue 12 for authentication framework.

In order to address the requirements from SA WG1, the following principles are adopted:

-	Support authentication of UE connecting to the 5G NextGen CN via different access networks, including 3GPP technologies, non-3GPP wireless technologies, fixed broadband access, trusted and untrusted Non-3GPP accesses.

-	The UE is assumed to possess credentials which are verifiable by an entity in the NextGen core network. The procedure for providing credential to UE is out of scope of this solution.

-	Support a unified authentication framework enabling UEs connecting via different access network technologies to access the 5G NextGen CN.

Furthermore, the following assumptions apply to the solution:

-	IMS registration is not considered within the scope of this solution.

Editor's note:	Aspects related to support of network slicing are FFS.

Editor's note:	mechanisms for the support of connectionless based service are FFS.

Editor's note:	Authentication in roaming scenario is FFS.

#### 6.12.1.1	Framework description

Editor's note:	Whether this framework is applicable only to session based connectivity is FFS.

##### 6.12.1.1.1 Security Architecture

In this reference architecture for the unified authentication and security framework it is assumed that for non-3GPP accesses there may be either security protection provided by the AN between the UE and the CN, or no security protection or insufficient security protection provided by the AN between the UE and the NextGen CN.

Figure 6.12.1.1-1 describes the relationship between the network functions and the UE functions for the authentication framework.

NOTE: the functional interfaces identified between the network functions do not necessarily correspond to the NextGen CN interfaces (e.g. as described in Annex G) but represent the exchange of security context between the functions.

Figure 6.12.1.1-1: Reference architecture for Unified authentication framework for non-roaming scenario

The unified authentication framework is defined in the following and shown in figure 6.12.1.1-1.

Supplicant function: The function inside the UE that executes the authentication process with the peer on network side.

AAA Function: The profile repository and authentication function of the Home network, that stores the subscriber profile and the subscriber long-term credentials and authentication algorithms.

CP-AU Authentication Function: A function in the core network that performs UE authentication process and interacts with AAA functionality for carrying out the authentication.

User Plane Protection Function: This function provides the protection (e.g. encryption, integrity protection, etc.) of the user plane information exchanged between the UE and the Access network and/or between the UE and the Core Network (e.g. in case of untrusted non-3GPP accesses). The Protection function may be located the Core Network e.g. when not present in AN.

NOTE:	The authentication between the UE and the CP-AU Function is performed only in the signalling plane.

Editor's note:	The definition of protection mechanisms for control plane and user plane are out of the scope of SA WG2 and it left to SA WG3 and RAN.

CP-CN function: indicates a CP-CN function defined for other mechanisms (e.g. mobility management, session management).

CP-AN Signalling Protection function: applies to control plane information exchanged between the UE and the Access network.

The following functional interfaces are considered:

-	AU-UE: Supports authentication mechanisms between the UE and the entity performing the UE authentication. AU-UE represents the end-to-end set of messages and parameters exchanged between the UE and the CP-AU Authentication Function. The authentication mechanisms supported by this functional interface may be transported over NG1 via other 5G NextGen mechanisms (e.g. encapsulated in mobility management signalling or session management signalling) and forwarded to CP-AU by the CP function implementing the 5G NextGen mechanism (e.g. mobility management CP function), to enable UE authentication for such procedures. The authentication mechanisms supported by this functional interface may otherwise be exchanged between the UE and the CP-AU directly over NG1 and separately from mobility management and session management signalling. In either case, the authentication exchange is end to end between the UE and the CP-AU Authentication Function.

-	AU-AN: Supports interaction over NG2 between the AN and the CP-AU Authentication Function for provision of security context (e.g. depending on the security requirements in terms of ciphering, integrity protection, etc.) upon authentication, connectivity establishment, mobility, etc.

Note: whether the CP-AU provides the security context to the control plane or user plane part of the AN is FFS and depends on the RAN design and the specific AN.

-	AU-CN: supports interaction between CP-AU and CP-CN (CN Signalling Protection function) for distribution of security context from CP-AU to CP-CN.

Editor’s note: The AU-CN may be a reference point towards other functions if the AU is a stand alone functions. Whether AU is collocated with other functions is FFS

-	AU-UP: supports interaction over NG4 between CP-AU and a User Plane Protection Function in the core network (e.g. in a UP-GW) for distribution of security context from CP-AU to user plane protection function.

-	AU-AAA: supports interaction between the CP-AU Authentication Function and AAA for authentication of the UE.

Editor's note:	Some of these interfaces may be actual reference points in the 5G NextGen architecture, others are captured just to represent the logical relation between functional entities and to describe the functional model, as described below.

Figure 6.12.1.1-2: Mapping of functional interfaces.

The definition of an authentication framework for different access systems connecting to the 5G NextGen core network needs to consider the following aspects:

-	The types of UEs in terms of capabilities

-	UEs supporting capabilities specific to 5G NextGen CN (including e.g. protocols and procedures specific to the 5G NextGen system)

-	UEs supporting only a subset of the capabilities specific to 5G NextGen CN (e.g. not supporting NextGen MM).

In order to take these aspects into consideration, this solution proposes an authentication framework based on the following principles:

-	Decoupling authentication and key management from other CN functionality (e.g. mobility management, session management, etc.) to enable the definition of a separate definition of the CP_AU function from other NextGen CN functions. The CP-AU function is a function used for authenticating UEs independently of the UE’s support for 5G NextGen CN capabilities (e.g. support for mobility).

-	Supporting the transport of authentication mechanisms that may be access network specific over a common authentication transport between the AN and the CN

-	Authentication mechanism include 3GPP authentication mechanisms for the 5G RAT and for evolved LTE, 3GPP authentication mechanisms for non-3GPP access networks (e.g. 3GPP mechanisms for trusted Wi-Fi scenarios), and possibly mechanisms for other access technologies that may not be defined by 3GPP. As an example, depending on the specific authentication mechanisms to be supported, the common authentication transport may be AAA to support authentication mechanisms based on EAP.

Editor’s Note: it is FFS if it is possible to assume a single authentication mechanism common to all access networks.

-	The transport of authentication mechanism over the AU-UE logical interface may be performed via other mechanisms depending by procedure (e.g. via MM function if the UE is authenticated upon attachment and the establishment of an MM context).

##### 6.12.1.1.2 Authentication and Key Agreement

The functional allocation for authentication and key agreement is as follows:

-	the CP-AU is a function located in the NextGen CN. The CP-AU performs authentication for the UE agnostically to the access network(s) the UE is connected to.

-

The authentication framework supports a security context hierarchy to introduce flexibility in deriving the required security context, while maintaining access-dependent aspects in the access networks. Specifically, in this solution:

-	The CP-AU generates at UE authentication and subsequently maintains the Authentication Security Context. The Authentication Security Context is generated upon UE authentication and is applicable to all security functionality for the UE, i.e. CP-CN Signalling Protection, CP-AN Signalling Protection, and User Plane Protection. The Authentication Security Context may be used to derive additional security contexts, e.g. for the specific access network or the CP signalling protection, depending on the UE capabilities and the other functionality (e.g. MM, SM, etc.) required to provide services to the UE.

-	The CP-AU Authentication Function, upon successful UE authentication derives from the Authentication Security Context, the UE security context(s) for the specific functionality as applicable, further processing the context as follows, e.g.:

-	for AN UP and CP protection: delivers, over AU-AN in NG2, per-UE access independent security context (e.g. keying material) to the UE and the AN functions.

-	for CP-CN signalling protection: if control plane needs to be protected between the UE and the CN, CP-AU distributes the CP security context required to protect it.

Editor’s Note: which entities besides the UE are provided such security context is FFS and dependent on the solutions for mobility management and session management.

-	The CP-AN receives the access independent security context for the UE and generates and maintains the access network specific UE AN Security Context (e.g. for ciphering and integrity protection) based on the AN Security Context provided by the CP-AU. This includes deriving, in the AN, keying material  for use with that access technology.

-	in case of inter-AN mobility, the source AN may transfer to the target AN the access independent security context received from the CP-AU. Then the target AN derives the access dependent AN Security Context specific to the target AN. As an alternative, the source AN or the target AN may trigger the CP-AU to deliver the access independent security context to the target AN, which in turn derives the access dependent AN Security Context for the UE.

Editor's note: Whether and how established security associations or contexts are transferred between Network Functions and (re)used, e.g. when the UE's serving Network Function(s) change due to mobility is FFS.

Figure 6.12.1.1-3: Security Context Hierarchy in the Unified Authentication Framework

##### 6.12.1.1.3 User Identity Confidentiality

In order to support user identity confidentiality, the following principles apply:

- the identities used by the UE with the NextGen CN for mobility management and session management are protected between the UE and the NextGen CN over NG1 such that they are never transferred in the clear

Editor’s Note: whether a single identity or multiple identities are needed for the various functionalities (e.g. MM, SM, etc.) is FFS, depends on the solutions for such functionalities, and requires SA3 input.

-	for 3GPP accesses, the identities used by the UE with the 3GPP AN are protected during access signalling

-	for non-3GPP accesses, the 3GPP identities used by the UE to gain access to the non-3GPP access are protected during access signalling

Editor’s Note: Any protection of non-3GPP identities not visible to the NextGen CN is FFS and may be out of scope of SA2 and SA3.

##### 6.12.1.1.4 System Signaling Protection

In this solution it is proposed that AN and CN control plane signalling are protected via separate mechanisms.

Editor’s Note: the final determination on the use of a single mechanism or separate mechanism is responsibility of SA3 and depends on solutions for other key issues (e.g. key issue 1).

Control plane signalling for AN is protected between the UE and the AN, and terminated by a function in the AN. The function receives from the CP-AU an access-independent AN Security Context for AN-specific key generation.

Control plane signalling between the UE and a CP-CN function in the NextGen CN is protected over NG1 for other mechanisms (e.g. mobility management, session management).

Editor’s Note: whether control plane protection over NG1 is distributed (e.g. a specific security context is delivered to all CP-CN functions that interface with the UE over NG1) or centralized (e.g. a function acts as termination point of the control plane protection over NG1 for all control plane related to the NextGen CN functionalities) is FFS and depends on the solutions for other key issues.

##### 6.12.1.1.5 User Plane Protection

For 3GPP access technologies, the user data protection is provided over the access link between the UE and the AN as per 3GPP specifications.

For non-3GPP access technologies, the user data protection is provided over the access link between the UE and the AN when the AN is considered trusted by the core network and when the AN provides such functions.

The user data protection may also be provided by a user plane function in the CN, e.g. for access networks that do not provide sufficient user data protection (e.g. untrusted AN) or for scenarios in which additional user data protection is required.

Editor’s Note: input is required from RAN and SA3 regarding the different connectivity modes supported and the impact on the user plane protection.

#### 6.12.1.2	Solution evaluation

Editor's note:	This clause will contain evaluation on the system impacts, e.g. UE, access network and non-access network.

>>>End of Changes<<<<