SA WG2 Meeting #S2-113ah	S2-163534

23 - 26 February 2016, Sophia Antipolis, FR

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Solution for “Key issue on Network Function Granularity and Interconnection between them”

Document for:	Approval

Agenda Item:	NextGen/6.10

Work Item / Release:	NextGen/ Rel-14

Abstract of the contribution: This contribution propose to add a solution for control plane interconnection model in the Next Generation Network Architecture.

# Introduction

The control plane interconnection model specified for Next Generation Network architecture should enable implementation and delivery of software services with more agility. This also implies that 3GPP should carefully decide how functions are modularized, the functionalities exposed to external entities (including UE, 3rd party application server etc). Furthermore, it is important to carry over the modularity of the functions also in the corresponding external interfaces.

The high level requirements for network function granularity and network function interconnection model was agreed in SA2#113 and reference is S2-160754. Also, the problems faced with the existing architecture were outlined in S2-160094. So, this paper does not re-state the need and importance to re-visit the control plane interconnection model in the context of new architecture.

Requirements from S2-160094:

The solution for the interconnection of the control plane network functions should allow

- Network functions to be able to interact with each other, e.g. for new services and features, while avoiding functional and signalling impact to unrelated network functions for a given interaction

- Mechanism for the exchange of information between network functions that results in agile/rapid deployment of new services, e.g. mechanism that allows reuse of procedures, wherever possible

- Ability to deploy network functions in various network configurations

In this paper, we focus on showing how this should be approached. We propose the following architectural principles:

- It is important for the UE to trust that certain functionalities are supported in the network thus important to enable multi-vendor interworking between UE and network functions. However, from the UE perspective, it is irrelevant how and where it exists within the network (e.g. which function module or software supports a certain function).

- It is important to enable multi-vendor interworking between radio and network functions within the core network. At the same time, it is sufficient if a single interface is exposed towards the radio while abstracting the modular (elementary) functions supported in the core network.

- Minimize dependencies between function modules, e.g. decouple mobility management and session management as far as possible

- Enable a new modular UE – Core, RAN-core interface

- Contain access-specific functions in specific adaptation modules

- Remove overlaps in functional scope from network functions, e.g. consolidate session management into a single control plane function

- Simplify Collision detection/resolution

- Consolidate session context state into a single state repository. This allows for control plane functions to be stateless.

The Data Layer (DL) provides the following services to the Core Control Plane Functions:

1)	distributed and real-time storage of opaque data (i.e. allowing the control plane function to create, read, update, delete their own data and subscribe to notifications upon data change);

2)	some possibility of data sharing between the Core Control Plane Functions;

3)	data redundancy.

The Data Layer allows:

a)	for the Core Control Plane Functions to be stateless;

b)	for cloud-based RT data storage and flexible deployment of data and Core Control Plane Functions in centralized or distributed clouds;

c)	Scalability and Elasticity – scale-in, scale-out functions can be leveraged for core control plane functions and the data layer. The data layer can adapt itself to the application functions.

d)	Network can store context in the SDL for Massive number of IoT devices (e.g. application VNF resources do not have to be used up for such devices when they are in IDLE mode).

c)	data analytics and network optimizations (e.g. minimizing mobility management signaling for stationary devices);

d)	for avoiding data duplication between the Core Control Plane Functions.

e)	moving from a message centric towards a data centric network architecture, permitting optimized call flows.

f)	robustness and high availability (seamless Core Control Plane Function's failure).

Current EPC Control plane structure:

To:

As stated in the annex E (E.1.1) (snippet below), we propose to define the service and functionality of the Data Layer and SDM without having to fix the service requester. This could then be re-used by any service requester (i.e. application) within the network.

“Any functionality or service involves two types of network functions: service-requester and service-provider. If a functionality or service is defined without fixing the service-requester then it could be re-used by any service-requester within the network.”.

# Proposal

It is proposed to add the following Solution on Network function interconnections to the TR 23.799 “Study on Architecture for Next Generation System”.

* * * Start of changes * * * * (all new text)

## 6.7	Solutions for Key Issue 7: Network function granularity and interactions between them

### 6.7.1	Solution 7.x: Control Plane Interconnection model with a Data Layer

This is a solution for key issue #7.

#### 6.7.x.1	Architecture description

##### 6.7.x.1	Reference model

This solution assumes the following reference model for non-roaming and roaming control plane interconnection model.

Figure 6.7.x.1-1: Non-roaming control plane interconnection model

Figure 6.7.x.1-2: Roaming control plane interconnection model

NG8:	Reference point between the Core Control Plane Function and the NG SDM for the user subscription data.

NGx:	Reference point between the Core Control Plane Functions and the Data Layer.

##### 6.7.x.1	Architecture principles

Editor's Note: This clause will contain e.g., terminology, overview, architecture description of the solution.

Following are the architecture principles that are proposed for the control plane interconnection model:

1)	It is important for the UE to trust that certain functionalities are supported in the network thus important to enable multi-vendor interworking between UE and network functions. However, from the UE perspective, it is irrelevant how and where it exists within the network (e.g. which function module or software supports a certain function).

2)	It is important to enable multi-vendor interworking between radio and network functions within the core network. At the same time, it is sufficient if a single interface is exposed towards the radio while abstracting the modular (elementary) functions supported in the core network.

3)	Minimize dependencies between function modules, e.g. decouple mobility management and session management as far as possible.

4)	Enable a new modular UE – Core, RAN-core interface. This is to ensure that updates / new features added to a certain function (e.g. MM) do not impact another function (e.g. SM) that supports different set of functionalities. When a certain function A is modified to support a new feature, only function A, with possibly minor impact to Front-end.   This principle also enables the modules to be pluggable per need (e.g. mobility function supports only mobility, not needed for anything else).

5)	Contain access-specific functions in specific adaptation modules.

6)	Remove overlaps in functional scope from network functions, e.g. consolidate session management into a single control plane function.

7)	Enable storage of Control Plane Functions session data in a session data repository.

8)	Enable an open interface between Control Plane Functions and the session data repository.

#### 6.7.x.2	Function description

##### 6.7.x.2.1	Data Layer (DL)

The Data Layer (DL) provides the following services to the Core Control Plane Functions (CCF):

1)	distributed and real-time storage of opaque data (i.e. allowing the CCFs to create, read, update, delete their own data and subscribe to notifications upon data change);

2)	some possibility of data sharing between Core Control Plane Functions (CCF);

3)	data redundancy.

The Data Layer allows:

a)	for the Core Control Plane Functions to be stateless;

b)	for cloud-based RT data storage and flexible deployment of data and Core Control Plane Functions in centralized or distributed clouds;

c) Scalability and Elasticity – scale-in, scale-out functions can be leveraged for core control plane functions and the data layer. The data layer can adapt itself to the application functions (i.e. core control plane function).

d) Network can store context in the SDL for Massive number of IoT devices (e.g. application VNF resources do not have to be used at all for storing context information for such devices when they are in IDLE mode.).

e)	data analytics and network optimizations (e.g. minimizing mobility management signaling for stationary devices);

f)	for avoiding data duplication between the Core Control Plane Functions (CCF).

g)	moving from a message centric towards a data centric network architecture, permitting optimized call flows.

h)	robustness and high availability (seamless Core Control Plane Function's failure).

##### 6.7.x.2.2	NG SDM

The NG SDM stores the user subscription data. It is located in the HPLMN and it supports HSS functionalities.

##### 6.7.x.2.3	Core Control Plane Function (CCF)

Control plane functions are grouped into procedure sets that handle a number of related, ideally self-contained control-plane procedures. A frontend terminates external protocol interfaces, including message ciphering/integrity protection and performs load balancing towards processing entities. Frontends have a coordination function for collision detection & resolution, sequencing and/or selection of procedures.

Figure 6.7.x.2.3-1: Core Control plane function

#### 6.7.x.3	Example Call flows

##### 6.7.x.3.1	 Detach/Attach scenario

Figure 6.7.x.3.1-1 shows the example call flow of a UE re-attaching in the same PLMM to which it was previously attached. As part of the previous Attach scenario, the Core Control Plane Functions (CCF) stored UE related data (including subscription data downloaded from the SDM) in the Data Layer.

During the next Attach, the new CCF serving the UE (CCF in the call flow) accesses the data stored in the Data Layer and proceeds with the Attach procedure without  necessarily requiring the signalling towards the NG SDM.

Figure 6.7.x.3.1-1: Detach/Attach

##### 6.7.x.3.2	 CCF Failover

Figure 6.7.x.3.2-1 shows the example call flow of a UE initiating signalling towards the Core Control Plane Function (e.g. Mobility Management or Session Management signalling) after a CCF failure.

The new CCF serving the UE (CCF 2 in the call flow) accesses the data stored in the Data Layer and proceeds with the UE request successfully.

Figure 6.7.x.3.2-1: CCF Failover

##### 6.7.x.3.3	 Successive UE transactions served by different stateless CCFs

Figure 6.7.x.3.2-1 shows the example call flow of a UE initiating consecutive UE transactions over time, involving different CCPFs (e.g. resulting from scale-in or scale-out operations).

The CCF serving the second UE request (CCF 2 in the call flow) accesses the data stored in the Data Layer by the previous CCF (CCF1 in the call flow) and proceeds with the UE request.

Figure 6.7.x.3.3-1: Successive UE transactions served by different stateless CCPFs

#### 6.7.x.4	Solution evaluation

Editor's note: This clause will contain evaluation on the system impacts, e.g., UE, access network and non-access network.

* * * End of Changes * * *