3GPP TSG SA WG2 Meeting #118bis	S2-170208

16 - 20 January 2017, Spokane, US	(revision of S2-17xxxx)

Source:	Huawei, HiSilicon

Title:	Functionalities of UPF and SMF

Document for:	Approval

Agenda Item:	6.5.3

Work Item / Release:	5GS_ph1/Rel-15

Abstract of the contribution: This contribution proposes the description of functionalities of UPF and SMF.

# Discussion

This contribution proposes the description of functionalities of UPF and SMF in 5G system.

# 2. Proposal

It is proposed to add the following text to the TS 23.501.

* * * Start of changes * * * *

## 6.1	NF Functional description

Editor’s notes: This should include various network functions in the architecture, features and functionalities supported.

### 6.1.x	UPF Functional description

For each UE associated with the data network, 5G UPF provides the user plane functionalities, including:

-	Packet routing and forwarding

-  External PDU session point of interconnect to Data Network

-  Anchor point for Intra-/Inter-RAT mobility

-  Uplink classifier to support routing traffic flows to local data network

-  Branching Point to support multi-homed PDU session

-  QoS handling for user plane, e.g. packet filtering, gating, UL/DL rate enforcement

-  Transport level packet marking in the uplink and downlink

-  Packet inspection

-  Lawful intercept (UP collection)

-  Traffic accounting and reporting

-  IDLE mode downlink packet buffering and initiation of network triggered service request procedure

NOTE: Not all of the UPF functions are required to be supported in an UPF instance of a network slice.

### 6.1.y	SMF Functional description

SMF functionalities include:

-  Session Management, e.g. Session establishment, modify and release, including tunnel maintain between UPF and AN node.

-  UE IP address/prefix allocation (for IP type PDU session)

-  Selection/reselection and control of UPF

-  Configures traffic steering at UPF to route traffic to proper destination

-  Termination of interfaces towards PCF.

-  Control of policy/QoS enforcement

-	Lawful intercept (for session management events and interface to LI System).

-  Charging (for session management events and interface to Charging system)

-  Downlink Data Notification

-  Determine SSC mode of a session (for IP type PDU session)

* * * Next change * * * *