3GPP TSG SA WG2 #120	S2-171927

Busan, Korea; 27th – 31st March 2017

Source:	ORANGE

Title:	TS 23.501: Aligning with the User Data Convergence architecture (UDC)

Spec:	3GPP TS 23.501 v0.3.0

Agenda item:	6.5.1

Document for:	Agreement

1. Introduction

The User Data Convergence (UDC) architecture is defined in TS 23.335. This is a layered architecture separating the data from the application logic where the user data is stored in a logically unique User Data Repository (UDR) allowing access from core and service layer entities, named Application Front Ends, as shown in Figure 1.

Figure 1: UDC reference architecture (Extract of Figure 4.1-1 of TS 23.335)

2. Reason for Change

There are some references to the UDR in TS 23.501 that do not reflect correctly the relation between FE applications and the UDR nor do they correctly reflect the UDC architecture.

The User Data Repository is a functional entity that acts as a single logical repository of user data for multiple Application Front Ends such as an HLR, HSS…therefore the UDM cannot “contain the UDR” nor could the UDR “be present in the UDM”. Instead and in accordance with the concept of UDC, the UDM could be implemented as an Application Front End accessing the 5G user data stored in the User Data Repository through the Ud reference point.

This needs to be reworded to be compliant with TS 23.335.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.501 v0.3.0.

* * * First Change * * * *

### 4.2.3	Non-roaming reference architecture

Figure 4.2.3-1 depicts the non-roaming reference architecture with service-based interfaces within the Control Plane.

Figure 4.2.3-1: 5G System Service-based architecture

NOTE: The AF can use the services exposed by the PCF or the NEF based on the operator deployment as described in clause 6.2.X.

Editor's note:	It is FFS whether there is a scenario in Rel-15 where the AF exposes services to the other CN CP Network Functions.

Editor's note:	It is FFS whether there is a scenario in Rel-15 where the AF may use services exposed by the SMF

Figure 4.2.3-2 depicts the 5G System architecture in the non-roaming case, using the reference point representation showing how various network functions interact with each other.

Figure 4.2.3-2: Non-Roaming 5G System Architecture in reference point representation

NOTE 2:	N9, N14 are not shown in all other figures however they may also be applicable for other scenarios.

NOTE 3:	For the sake of clarity of the point-to-point diagrams, the UDSF, SDSF, NEF and NRF have not been depicted. However, all depicted Network Functions can interact with the UDSF, NEF and NRF as necessary.

NOTE 4:	 The User Data Convergence architecture allows the UDM to store 5G user data in the User Data Repository (UDR). For clarity, the UDR and its connections with other NFs, e.g. PCF, are not depicted in the point-to-point and service-based architecture diagrams.

Figure 4.2.3-3 depicts the non-roaming architecture for UEs concurrently accessing two (e.g. local and central) data networks using multiple PDU Sessions, using the reference point representation. This figure shows the architecture for multiple PDU sessions where two SMFs are selected for the two different PDU sessions. However, each SMF may also have the capability to control both a local and a central UPF within a PDU session.

Figure 4.2.3-3: Applying non-roaming 5G System architecture for multiple PDU session in reference point representation

Figure 4.2.3-4 depicts the non-roaming architecture in case concurrent access to two (e.g. local and central) data networks is provided within a single PDU session, using the reference point representation.

Figure 4.2.3-4: Applying non-roaming 5G System architecture for concurrent access to two (e.g. local and central) data networks (single PDU session option) in reference point representation

* * * Next Change * * * *

### 6.2.7	UDM

The Unified Data Management (UDM) supports the following functionality:

-	Supports Authentication Credential Repository and Processing Function (ARPF). This function stores the long-term security credentials used in authentication for AKA.

-	Stores Subscription information.

NOTE 1: 	In accordance with the UDC architecture, the UDM could be implemented as an Application Front End accessing the 5G user data stored in the User Data Repository through the Ud reference point.

NOTE 2:	The interaction between UDM and HSS is implementation specific.

* * * End of Changes * * * *