SA WG2 Meeting #120	S2-172080

27 - 31 March 2017, Busan, South Korea	(revision of S2-17xxxx)

Source:	LG Electronics, SK Telecom

Title:	TS 23.501: Deactivation of the derived QoS rule

Document for:	Approval

Agenda Item:	6.5.5 QoS concept and functionality

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution:It is proposed to introduce deactivation mechanism for the derived QoS rule based on timer configured during the PDU session establishment.

Discussion

There are several possible candidate solutions for deactivation of the derived QoS rule.

First candidate is using implicit deactivation without any signalling or pre-configuration, i.e. leave it as an implementation of the UE and 5G-CN. There is no need to standardize the deactivation process. However, in order to support uplink QoS verification , the 5G CN and UE needs to have same uplink QoS rule. So implicit deactivation cannot be used because this method does not guarantee synchronised QoS rule between the 5G-CN and UE.

Second candidate is using explicit signalling to deactivate the derived QoS rule. When the reflective QoS is activated via control plane signalling, this solution can be used. However, when the reflective QoS is activated via user plane marking, this solution is inappropriate because key point of user plane activation is avoiding out of band signalling.

Third candidate is introducing the similar method supported in the EPS. In the EPS, the UE reflective QoS procedure may be deleted as follows (from TS 24.139):

5.2.3	Maintaining DSCP marking rules

The DSCP marking table is created when the first DSCP marking rule is created based on the received downlink IP packet.

Depending on the protocol type, the n-tuple is either a 5-tuple (including: source IP address, destination IP address, source port number, destination port number, and protocol type) or a 3-tuple (including source IP address, destination IP address, and protocol type).

The lifetime of the DSCP marking table is same as the associated PDN connection.

A UE may remove the expired entries from the table based on the timestamp. How long the entry shall be maintained in the table is an implementation specific value.

In the EPS, the UE creates table and manages the timestamp updated when sending and receiving corresponding packets. It is UE implementation how long the entry is maintained.

The Similar mechanism can be used for 5G system. When the network indicates reflective QoS activation, the network start the timer with pre-configured value which may be determined during the PDU session establishment. The UE also start the same timer when the reflective QoS indication is received. When the timer is expired, the derived QoS rule is deactivated.

Proposal: When the UPF indicates activation of reflective QoS, it starts a deactivation timer which is pre-configured during the PDU session establishment. Whenever the UPF indicates reflective QoS, it resets the timer. When the UE receives the reflective QoS indication, the UE also starts a deactivation timer and reset the timer whenever it receives reflective QoS indication. The derived QoS rule is deactivated when the timer is expired.

In the last meeting, there were some contributions (S2-170971, S2-171070, S2-171194) to handle deactivation of reflective QoS. Basically those propose user plane solutions which deactivate the reflective QoS by not including RQI or including RQDI. However, if there is no downlink packets associated with the reflective QoS, such mechanisms are not workable and control plane signalling is required to deactivate the reflective QoS. On the other hands, our proposal can deactivate the reflective QoS by using deactivation timer and there is no additional signalling to deactivate the reflective QoS.

Proposal

Add the following to the TS 23.501. Following descriptions will be aligned with other papers if it is agreed.

* * * * Start of 1st Change * * * *

5.7.5.5	Disabling Reflective QoS

5.7.5.5.1	General

The 5GC supports disabling reflective QoS. Reflective QoS can be disabled via User Plane and Control Plane. The 5GC determines whether to disable the reflective QoS function via Control Plane or User Plane based on policies and type of access.

5.7.5.5.2	Reflective QoS Deactivation via User Plane

When the PDU session is established, the SMF inform the UE of deactivation timer value. The SMF configures the UPF to start the deactivation timer when the UPF indicates activation of reflective QoS. Whenever the UE receives the reflective QoS indication, the UE also starts a deactivation timer. The UPF reset the timer when it includes RQI in the encapsulation header on N3 reference point. The UE reset the timer whenever it receives reflective QoS indication. The UE and the UPF removes the derived QoS rule when the deactivation timer is expired.

5.7.5.5.2	Reflective QoS Deactivation via Control Plane

When the 5GC determines to disable reflective QoS via Control Plane, the SMF updates the QoS rule of the UE and the UPF so that the QoS rule does not includes RQI anymore. When the SMF updates the QoS rule without RQI, the UE and the UPF shall remove the derived QoS rule created by the updated QoS rule.

* * * * End of Changes * * * *