SA WG2 Meeting #120	S2-172622

27 - 31 March 2017   Busan, Korea	(revision of S2-172212)

Source:	Deutsche Telekom

Title:	Updates to drafting rules and conventions for Naming of NF services

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	NextGen / Rel-15

Abstract of the contribution: This contribution proposes updates to the drafting rules and conventions for Naming of NF services.

# Discussion

While it is acknowledged that in current release NF Services will be defined in a way that they are directly bound to a particular NF type,  the approach of naming services can still be defined in more future-proof way. I.e. NF Services may not be directly bound to these NFs. Hence, the NF can be seen as a category or type which the actual service is belonging to. Additional, the naming of the NF itself is not necessary when discovering a service.

Furthermore, the service operation should be pushed into another level of service naming. As currently proposed in TS23.502 V0.2.0 Annex A.2, the operation […]request is always a part of the operation naming which does not reveal a deeper meaning.

We propose to re-think the naming of services and another way how to represent the service, its functionality and its resources.

# Proposal

The following changes are proposed to be applied to TS 23.502.

************************************** Start of changes *****************************************

## A.2	Naming

### A.2.1	Service naming

Each NF service provided by a service-based interface shall be named and referred to according to the following nomenclature:

-	Nnnn_ServiceName, where Nnnn is the service-based interface where the NF service is invoked. See TS 23.501 [2] clause 4.2.5 for the list of service-based interfaces in the 5G System Architecture.

Example (illustrative): Namf_Registration.

### A.2.2	Service operation naming

If a service contains multiple independent operations, each operation shall be named and referred to according to the following nomenclature:

-	Nnnn_ServiceName_ServiceOperation[Method], where the ServiceName represents the actual NF service. The ServiceOperation itself defines the available service functionality which can be addressed by a specific operation. The Method(s) is/are the action(s), how the ServiceOperation can be used. It can be created, read, updated or deleted.

Example (illustrative): Namf_Registration_Registration[Create], Namf_Registration_Location[Read]

In general, this operation naming structure for the given example is depicted in a tree-structure diagram:

Figure X: Service Operation Naming and its Methods

************************************** End of changes *******************************************