SA WG2 Meeting #120	S2-172641

27 - 31 March 2017, Busan, Korea	Revision of S2-172069

Source:	Huawei, HiSilicon

Title:	TS 23.501: Update to the IP address management clause

Document for:	Approval

Agenda Item:	5.8.1 IP address management

Work Item / Release:	5GS_ph1/Rel.15

Abstract of the contribution: This contribution proposes some update to the IP address management clause.

1.	Discussion

Update the IP address management clause.

- Change “selected IP version” to “requested IP version”

- Change text in 5.8.1.2 to cover multi-homed PDU session case.

- Clarify how the SM can decide the routing rule. According to the PCF function, the routing rule can be decided based on local configuration or from the policy provided by the PCF.

Current Policy functions from 23.501 A.3.1:

The Policy framework architecture provides the functions for:

application and service data flow detection,

QoS and gating control,

Credit management,

Flow based charging,

Background data transfer policy negotiation,

Management of the PFDs in the PCEF by the 3rd party AS,

Traffic Steering Control for steering traffic for the services on the DN side of the N6 reference point,

Provide a Front End to subscription information relevant for policy decisions in a User Data Repository,

Provide Network selection and Mobility Management related policies (e.g. RFSP index) to the Access and Mobility Management,

2.	Proposal

Add the following changes to the TS 23.501.

* * * * Start of Change * * * *

#### 5.8.1.2	Routing rules configuration

When the UE has one or multiple IPv6 PDU sessions to the same Data Network Name (DNN) with multiple IP prefixes, the UE selects the IP prefix according to routing rules pre-configured in the UE or received from network.

The SMF can decide the routing rules for a UE based on the static local configuration or dynamic policy from the PCF during the PDU session establishment or new IP prefix is allocated to the UE. For IPv6 multi-homed PDU session the SMF can send routing rules along with IP prefixes to the UE to influence the source IP prefix selection in IPv6 Router Advertisement (RA) messages according to RFC 4191 [8]. Such messages are sent via the UPF.

Further, NGC supports the delivery of routing rules together with IP prefix allocation to the UE any time after the PDU session setup procedure is completed.

Editor's note: It is FFS whether the SMF provides routing rules to the UE in case of IPv4 PDU type.

* * * * End of Changes * * * *