SA WG2 Meeting #122E	S2-176745

11–15 Sep 2017, Elbonia	(revision of S2-175355)

Source:	Huawei, HiSilicon

Title:	TS 23.501: Remove ENs of clause 5.6.4

Document for:	Approval

Agenda Item:	13

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution proposes updates and resolve ENs in clause 5.6.4 in TS 23.501.

# Introduction

The contribution proposes the following changes to TS 23.501.

Multiple ENs in 5.6.4 are proposed to be removed or clarified by normal texts. To facilitate the discussion, the detailed explanations are included in the comments corresponding to the changes.

# Proposal

It is proposed to update TS 23.501 as follows:

* * * Begin of Changes * * *

### 5.6.4	Single PDU session with multiple PDU session anchors

#### 5.6.4.1	General

In order to support traffic routing or to support SSC mode 3 as defined in clause 5.6.9.2.3, the SMF may control the data path of a PDU session so that the PDU session may simultaneously correspond to multiple N6 interfaces. The UPF that terminates each of these interfaces is said to support a PDU session anchor functionality. Each PDU session anchor supporting a PDU session provides a different access to the same DN. Further, the first PDU session anchor assigned at the establishment of a PDU session is associated with SSC mode of the PDU session and the additional PDU session anchor(s) assigned within the same PDU session for local access to a DN are independent of SSC mode of the PDU session.

This may correspond to

-	The Usage of an UL Classifier functionality for a PDU session defined in clause 5.6.4.2.

-	The Usage of an IPv6 multi-homing for a PDU session defined in clause 5.6.4.3.

#### 5.6.4.2	Usage of an UL Classifier for a PDU session

In case of PDU sessions of type IPv4 or IPv6 or Ethernet, the SMF may decide to insert in the data path of a PDU session an "UL CL" (Uplink classifier). The UL CL is a functionality supported by an UPF that aims at diverting (locally) some traffic matching traffic filters provided by the SMF. The insertion and removal of an UL CL is decided by the SMF and controlled by the SMF using generic N4 and UPF capabilities. The SMF may decide to insert in the data path of a PDU session a UPF supporting the UL CL functionality during or after the PDU session establishment, or to remove from the data path of a PDU session a UPF supporting the UL CL functionality after the PDU session establishment. The SMF may include more than one UPF supporting the UL CL functionality in the data path of a PDU session.

The UE is unaware of the traffic diversion by the UL CL, and does not involve in both the insertion and the removal of UL CL. In case of a PDU session of IPv4 or IPv6 type, the UE associates the PDU session with either a single IPv4 address or a single IPv6 Prefix allocated by the network.

Editor's note:	Whether (and if yes how) the UE is made aware of UL CL insertion and removal is FFS.

When an UL CL functionality has been inserted in the data path of a PDU session, there are multiple PDU session anchors for this PDU session. These PDU session anchors provide different access to the same DN. In case of a PDU session of IPv4 or IPv6 type, only one PDU session anchor is IP anchor point for the IPv4 address / IPv6 prefix of the PDU session provided to the UE.

NOTE 0:	The mechanisms for packet forwarding on the N6 reference point between the PDU session anchor providing local access and the DN are outside the scope of this specification.

The UL CL provides forwarding of UL traffic towards different PDU session anchors and merge of DL traffic to the UE i.e. merging the traffic from the different PDU session anchors on the link towards the UE. This is based on traffic detection and traffic forwarding rules provided by the SMF.

The UL CL applies filtering rules (e.g. to examine the destination IP address/Prefix of UL IP packets sent by the UE) and determines how the packet should be routed. The UPF supporting an UL CL may also be controlled by the SMF to support traffic measurement for charging, traffic replication for LI and bit rate enforcement (per PDU session AMBR).

NOTE 1:	The UPF supporting an UL CL may also support a PDU session anchor for connectivity to the local access to the data network (including e.g. support of tunnelling or NAT on N6). This is controlled by the SMF.

The insertion of an ULCL in the data path of a PDU session is depicted in Figure 5.6.4.2-1.

Figure 5.6.4.2-1 User plane Architecture for the Uplink Classifier

NOTE 2:	The same UPF may support both the UL CL and the PDU session anchor functionalities.

In Home Routed case the visited operator is only allowed to use local access to a DN in case the home operator has explicitly allowed it.

#### 5.6.4.3	Usage of IPv6 multi-homing for a PDU session

A PDU session may be associated with multiple IPv6 prefixes. This is referred to as multi-homed PDU session. The multi-homed PDU session provides access to the Data Network via more than one PDU session anchor. The different user plane paths leading to the different PDU session anchors branch out at a "common" UPF referred to as a UPF supporting "Branching Point" functionality. The Branching Point provides forwarding of UL traffic towards the different PDU session anchors and merge of DL traffic to the UE i.e. merging the traffic from the different PDU session anchors on the link towards the UE.

The UPF supporting a Branching Point functionality may also be controlled by the SMF to support traffic measurement for charging, traffic replication for LI and bit rate enforcement (per PDU session AMBR). The insertion and removal of a UPF supporting Branching Point is decided by the SMF and controlled by the SMF using generic N4 and UPF capabilities. The SMF may decide to insert in the data path of a PDU session a UPF supporting the Branching Point functionality during or after the PDU session establishment, or to remove from the data path of a PDU session a UPF supporting the Branching Point functionality after the PDU session establishment.

Multi homing of a PDU session applies only for PDU sessions of IPv6 type. The request of PDU session type "IP" or "IPv6" implies the support of multi-homed PDU session for IPv6 in the UE.

The use of multiple IPv6 prefixes in a PDU session is characterised by the following:

-	The UPF supporting a Branching Point functionality is configured by the SMF to spread the UL traffic between the IP anchors based on the Source Prefix of the PDU (which may be selected by the UE based on routing information and preferences received from the network).

-	IETF RFC 4191 [8] is used to configure routing information and preferences into the UE to influence the selection of the source Prefix.

NOTE 1:	This corresponds to Scenario 1 defined in IETF RFC 7157 [7] "IPv6 Multi-homing without Network Address Translation". This allows to make the Branching Point unaware of the routing tables in the Data Network and to keep the first hop router function in the IP anchors.

-	The multi-homed PDU session may be used to support make-before-break service continuity to support SSC mode 3. This is illustrated in Figure 5.6.4.3-1.

-	The multi-homed PDU session may also be used to support cases where UE needs to access both a local service (e.g. local server) and a central service (e.g. the internet), illustrated in Figure 5.6.4.3-2.

-	The UE shall use the method specified in TS 23.502 [3], clause 4.3.5.3, to determine if a multi-homed PDU session is used to support the service continuity case shown in Figure 5.6.4.3-1, or if it is used to support the local access to DN case shown in Figure 5.6.4.3-2.

Figure 5.6.4.3-1: Multi-homed PDU Session: service continuity case

NOTE 2:	The same UPF may support both the Branching Point and the PDU session anchor functionalities

Figure 5.6.4.3-2: Multi-homed PDU Session: local access to same DN

NOTE 3:	The same UPF may support both the Branching Point and the PDU session anchor functionalities

In case of HR roaming, the visited operator is allowed to use an IP anchor in the VPLMN only in case the home operator has explicitly allowed it.

* * * End of Changes * * *