SA WG2 Meeting #122E e-meeting	S2-176796

11 – 15 September 2017, Elbonia	(revision of S2-17xxxx)

Source:	LG Electronics

Title:	TS 23.501 clean up for MICO

Document for:	Approval

Agenda Item:	12

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: It is proposed to clean up MICO description.

Discussion

It is proposed to clean up MICO description.

1) format update to clause 5.4.1.2.1

2) Capitalize only  Only in 5.4.1.3

3) description clean-up for UE initiating MO triggering

4) clarified that UE does not perform registration update via non-3GPP

Proposal

* * * * Start of 1st Change * * * *

5.4.1.2	UE reachability allowing mobile terminated data while the UE is CM-IDLE

5.4.1.2.1	General characteristics

A UE is reachable by CN paging in CM-IDLE state unless the UE applies MICO mode.

5.4.1.3	Mobile Initiated Connection Only (MICO) mode

A UE may indicate preference for MICO mode during initial registration or registration update. The AMF, based on local configuration, communication pattern if available, UE indicated preferences, UE subscription information and network policies, or any combination of them, determines whether MICO mode is allowed for the UE and indicates it to the UE during Registration procedure.

The UE and core network re-initiates or exits the MICO mode at subsequent registration signalling. If MICO mode is not indicated explicitly in Registration procedure and registration procedure is successful, then both the UE and the AMF shall not use the MICO mode i.e. the UE behaves as a normal UE and the network treats the UE as a normal UE.

The AMF assigns a registration area to the UE during the registration procedure. When the AMF indicates MICO mode to a UE, the registration area is not constrained by paging area size. If the AMF serving area is the whole PLMN, based on local policy, and subscription information, may decide to provide an "all PLMN" registration area to the UE. In that case, re-registration to the same PLMN due to mobility does not apply.

If mobility restrictions are applied to a UE in MICO mode, the AMF needs to allocate an allowed area/non-allowed area to the UE as specified in clause 5.3.4.1.

When the AMF indicates MICO mode to a UE, the AMF considers the UE always unreachable while in CM-IDLE. The AMF rejects any request for downlink data delivery for UE in MICO mode and CM-IDLE with an appropriate cause. The AMF also defers downlink transport over NAS for SMS, location services, etc. The UE in MICO mode is only reachable for mobile terminated data or signalling when the UE is in CM-CONNECTED mode.

A UE in MICO mode need not listen to paging while in CM-IDLE. A UE in MICO mode may stop any access stratum procedures in CM-IDLE, until the UE initiates transition from CM-IDLE to CM-CONNECTED mode due to one of the following triggers:

-	A change in the UE (e.g. change in configuration) requires an update its registration with the network.

-	Periodic registration timer expires.

-	MO data pending.

-	MO signalling pending (e.g. SM procedure initiated).

If a registration area that is not the "all PLMN" registration area is allocated to a UE in MICO mode, then the UE determines if it is within the registration area or not when it has MO data or MO signalling.

* * * * Start of 2nd Change * * * *

5.5.1	Registration Management

The UE shall enter RM-DEREGISTERED state on non-3GPP access:

-	at the UE and at the AMF, after performing an explicit Deregistration procedure;

-	at the AMF, after the network non-3GPP Implicit Deregistration timer has expired. -	at the UE, after the UE non-3GPP Deregistration timer has expired.

NOTE:	This is assumed to leave sufficient time to allow the UE to re-activate UP connections for the established PDU sessions over 3GPP or non-3GPP access.

Whenever a UE enters CM-IDLE state for the non-3GPP access, it starts the UE non-3GPP deregistration timer according to the value received from the AMF during a registration procedure.

Over non-3GPP access, the AMF runs a non-3GPP Implicit Deregistration timer. The timer is started whenever the CM state for the UE changes to CM-IDLE over non-3GPP access, with a value longer than the UE's non-3GPP deregistration timer.

For an UE that is registered over Non-3GPP access, a change of the point of attachment (e.g. change of WLAN AP) shall not lead the UE to perform a registration update procedure.

A UE shall not provide 3GPP-specific parameters (e.g. indicate a preference for MICO mode) during initial registration over a non-3GPP access.

* * * End of Changes * * * *