SA WG2 Meeting #122	S2-174356

June, 26 – 30, 2017, San Jose Del Cabo, Mexico

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1174 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | new annex to 23.228 for 5GS support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_ph1 |  |  |  |  |  |  | Date: |  | 2017-06-20 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 5GS |  |  |  |  |  |  |  |  |
| Summary of change: |  | New annex for 5GS IP-CAN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not adapted for 5GS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Change marks reflect compared to the Annex in 23.501 |  |  |  |  |  |  |  |  |

# Introduction

This CR moves the annex D in TS 23.501 into TS 23.228. Different markups have been used to show changes between current Annex D in 23.501 and the CR.

The non-editorial changes are motivated by the following:

- In XX.0 “A single IMS subscription profile is used regardless of UE accessing IMS via different IP-CANs.” Is removed duet to not 5GS specific.

- Introducing the IP address allocation for 5GS agreement introduced in 23.501

- Removing the need for a procedural description for P-CSCF discovery. Current text should be enough.

- Adding XX.10 which is annex W in 23.228 adapted to 5GS

# CR

*** BEGIN CHANGES ***

# Annex XX (Normative):
IP-Connectivity Access Network specific concepts when using 5GS to access IMS

## XX.0	General

This clause describes the main IP-Connectivity Access Network specific concepts that are used for the provisioning of IMS services over 5GS. In subsequent clauses, mainly the differences between Annex E and 5GS aspects is included.

HSS is used to store IMS related subscription and context as shown in the Figure 4.0 "Reference Architecture" specified in clause 4.0. For 5GS, HSS functionality for IMS shall continue to be as standalone regardless if it is co-located or implemented as part of the UDM.

Figure XX.0-1: UDM and HSS collocated or HSS as part of UDM

NOTE:	The HSS shown in Figure XX.0-1 is only considering the functionality required for IMS.

From IMS perspective, the Rx interface as specified in TS 23.203 [54] is continued to be used between the P-CSCF and the Policy Control Function (PCF).

In 5G System the Gm reference point defined in  clause 4.0 and clause 4.4 is supported for the communication between UE and IMS, e.g. related to registration and session control. Therefore, the same interface is used, i.e. Gm, from P-CSCF perspective towards the UE, when UE is in 5G System or any other IP-CAN.

## XX.1	Mobility related concepts

### XX.1.0	General

To support IMS, the UE shall establish a PDU session with PDU session type set to IP for the corresponding DNN (see TS 23.501 [xx], clause 5.6.1), and acquire an IP address according to TS 23.501 [xx], clause 5.8.1.1. If IMS Multimedia Telephony Service as specified in TS 22.173 [16] is to be used within the PDU session, SSC mode 1 shall be set for the PDU session.

If the UE changes its IP address due to changes triggered by the 5GS procedures, then the UE shall re- register in the IMS.

If the UE acquires an additional IP address, then the UE may perform an IMS registration using this additional IP address as the contact address. If IMS registration is performed, this IMS registration may co-exist with the previous IMS registration from this UE and the UE shall be notified that this IMS registration results in multiple simultaneous registrations.

### XX.1.1	Procedures for P-CSCF discovery

All the procedures described in clause 5.1.1 apply with the following addition:

-	For the option where P-CSCF discovery is part of the IP-CAN connectivity establishment the following applies:

-	P-CSCF discovery shall take place during the PDU session establishment procedure. For further details, see clause XX.1.2.1.

#### -

# -

-

## XX.2	QoS related concepts

### XX.2.1	Application Level Signalling for IMS

#### XX.2.1.0	General

When the UE uses 5GS-access for IMS services, it shall be possible to establish at least one QoS flow identified by a QFI for IMS signalling.

#### XX.2.1.1	QoS Requirements for Application Level Signalling

It shall be possible to request prioritised handling over the 5GS system for IMS signalling by including the appropriate QoS rule for signalling traffic as specified in TS 23.501 [xx], clause TBD.

#### XX.2.1.2	Requirements for IM CN subsystem signalling flag

Editor's note:	It is FFS if the flag is needed in 5GS.

### XX.2.2	The QoS requirements for an IMS session

#### XX.2.2.0	General

The selection, deployment, initiation, and termination of QoS signalling and resource allocation shall consider:

-	The general requirements described in clause 4.2.5.

-	The QoS handling is described in TS 23.501 [xx], TS 23.502 [xx] and TS 23.203 [54].

#### XX.2.2.1	Relation of IMS media components and 5GS QoS flows carrying IMS media

All associated media flows (such as e.g. RTP / RTCP flows) used by the UE to support a single media component are assumed to be carried within the same 5GS QoS flow.

### XX.2.3	Interaction between 5GS QoS and session signalling

#### XX.2.3.0	General

The generic mechanisms for interaction between QoS and session signalling are described in clause 5.4.7, the mechanisms described there are applicable to 5GS-accesses as well. With the following clarifications:

-	An IP-CAN bearer in this specification shall be interpreted as a 5GS QoS flow.

-	An IP-CAN session in this specification shall be interpreted as a 5GS PDU session of type IP.

#### XX.2.3.1	Resource Reservation with PCF

Editors note:	Add which procedures the UE and network uses to set-up QoS flows.

### XX.2.4	Network initiated session release - P-CSCF initiated

#### XX.2.4.0	General

In the event of loss of coverage for 5GS radio access, TS 23.501 [xx] and TS 23.502 [xx] defines the N2 release procedure. This procedure releases the QoS flows. This is indicated to the P-CSCF as shown in figure XX.3.

Editor's note:	It is FFS exactly how loss of 5G radio access is indicated to P-CSCF.

#### XX.2.4.1	Network initiated session release - P-CSCF initiated after loss of radio coverage

Figure XX.3: Network initiated session release - P-CSCF initiated after loss of radio coverage

1.	In case of NG-RAN, loss of radio coverage causes the GFBR QoS flow resources to be released in the network and PCF and P-CSCF are notified appropriately.

2.	The P-CSCF decides on the termination of the IMS session. In the event of the notification that the signalling transport to the UE is no longer possible, the P-CSCF shall terminate any ongoing IMS session with that specific UE. If the P-CSCF decides to terminate the IMS session, it indicates this to PCF, which removes the authorization for resources that had previously been issued for this endpoint for this session. (see TS 23.203 [54]).

The following steps are only performed in case the P-CSCF has decided to terminate the session.

3.	The P-CSCF generates a Release (Bye message in SIP) to the S-CSCF of the releasing party.

4.	The S-CSCF invokes whatever service logic procedures are appropriate for this ending session. The S-CSCF of the releasing party forwards the Release to the S-CSCF of the other party.

5.	The S-CSCF invokes whatever service logic procedures are appropriate for this ending session. The S-CSCF of the other party forwards the Release on to the P-CSCF.

6.	The P-CSCF removes the authorization for resources if they have previously been issued for this endpoint for this session. The P-CSCF forwards the Release to the UE.

7.	The UE responds with a SIP OK to the P-CSCF

8.	Depending on the Bearer Control Mode selected for the IP-CAN session, the release of previously reserved resources shall be initiated either by the UE or by the IP-CAN itself. The SIP OK message is sent to the S-CSCF.

9.	The S-CSCF of the other party forwards the OK to the S-CSCF of the releasing party.

10.	The S-CSCF of the releasing party forwards the OK to the P-CSCF of the releasing party.

## XX.3	Address and identity management concepts

### XX.3.1	Deriving IMS identifiers from the USIM

If the UICC does not contain an ISIM application, and the permanent user identity is IMSI then clause E.3.1 applies.

## XX.4	Void

## XX.5	IP version interworking in IMS

A PDU session is associated with either an IPv4 or an IPv6 address. For communication with the IMS, the UE acquire an IP address according to TS 23.501 [xx], clause 5.8.1.1 for the PDU session. The IP address will be either an IPv4 address or and IPv6 address. Hence, a UE will register to the IMS with either an IPv4 or an IPv6 address. Here the P-CSCF and IMS AGW may support both IP versions and/or may do interworking depending IP version used within the IMS. If the P-CSCF and IMS AGW do not support both versions, then network design must ensure than IP address incompatibility does not occur.

## XX.6	Usage of NAT in 5GS

There should be no NAT (or its existence should be kept transparent towards the UE) located between the UPF and the P-CSCF, which is possible as they are either located within the same private network and share same address space, or both the UE and the P-CSCF are assigned globally unique IP addresses (see Annex M).

NOTE:	If the UE discover a NAT between the UE and the P-CSCF, the UE might send frequent keep-alive messages and that may drain the UE battery.

## XX.7	Retrieval of Network Provided Location Information in 5GS

Information related to the location of the user provided by the access network may be required in IMS in order to comply with regulatory requirements (e.g. data retention, lawful interception) and/or in order to enable certain types of added value services based on the user's location.

Depending on usage scenario, the following mechanisms are defined and can be used to retrieve the user location and/or UE Time Zone information from the access network when using 5GS to access IMS:

-	The P-CSCF can retrieve the user location and/or UE Time Zone information using PCC mechanisms as specified in TS 23.203 [54] and in TS 29.214 [11]. . Operator policy determines whether to provide the user location and/or UE Time Zone information from the access network in the INVITE request or within a subsequent message of the dialog.

-	When the user location and/or UE Time Zone information is required from the access network but not already available (e.g. when required in an INVITE request, when it is needed prior to session delivery, or when call is broken out to a MGCF), an IMS AS can trigger the retrieval of the user location and/or UE Time Zone information from the AMF via the HSS/UDM as specified in TS 29.328 [79] and as described in clause 4.2.4a.

Editor's note:	TS 29.328 will probably need to be revised to reflect 5GS.

Information flows on how user location and/or UE Time Zone information can be further distributed within IMS depending on the alternative mechanism used can be found in, Annex R, where the term HSS and HSS/UDM shall be understood as in clause XX.0.

The level of granularity of user location information may be changed at network/trust boundaries. Thus, the level of user location information granularity that can be retrieved by an IMS AS via the HSS/UDM-based procedures in roaming scenarios depends on inter-operator agreement, and needs to be aligned with policies in the P-CSCF.

## XX.8	Geographical Identifier

A network which requires the Geographical Identifier to be generated in the IMS may implement a mapping table between a NG-RAN cell identifier received as part of Access Network Information) and a Geographical Identifier. The P-CSCF or an IMS AS may then, based on operator policy, use this mapping table to convert the user location into a Geographical Identifier, and insert the Geographical Identifier in the SIP signalling, thus enabling routing decision in downstream IMS entities or interconnected network.

Editor's note:	The correct naming of the NG-RAN cell identifier TBD.

## XX.9	Support for Paging policy differentiation for IMS services

P-CSCF may support Paging Policy Differentiation (as defined in TS 23.501 [xx]) for a specific IMS service by marking packet(s) to be send towards the UE related to that IMS service. For such an IMS service, a specific DSCP (IPv4) value and/or a specific Traffic Class (IPv6) value are assigned by local configuration in the P-CSCF.

When Paging Policy Differentiation is deployed in a PLMN, all P-CSCF entities of that PLMN shall homogeneously support it and shall be configured with the same policy for setting the specific DSCP (IPv4) and/or Traffic Class (IPv6) values used by P-CSCF for that feature.

NOTE 2:	It is assumed that the DSCP / Traffic Class header is not rewritten by intermediate routers between the P-CSCF and the UPF.

## XX.10	Support of IMS Services for roaming users in deployments without IMS-level roaming interfaces

### XX.10.1	General

This clause describes the functions that are used to support IMS services for roaming users in deployments without IMS-level roaming interfaces. In this roaming model the UPF holding the UE’s IP point of presence is located in the home PLMN and therefore UE IMS signalling and user plane are routed to home PLMN.

### XX.10.2	Architecture

The architecture to support IMS services for roaming users, including Voice over IMS in deployments without IMS-level roaming interfaces is shown in figure XX.10.2-1

The following architecture requirements apply:

-	P-CSCF (in HPLMN) identifies the serving network (VPLMN) where the UE is located using the procedure defined in clause XX.10.3.

Figure XX.10.2-1: IMS traffic home routed

### XX.10.3	Subscription to changes in PLMN ID at IMS Initial Registration

In LBO roaming model where P-CSCF is located in VPLMN (see Annex M.1), the home network determines the serving PLMN of the UE from the location of the P-CSCF during initial IMS Registration, using the P-CSCF network identifier.

In deployments without IMS-level roaming interfaces, the home network determines the serving PLMN of the UE using procedure defined in TS 23.203 [54], where P-CSCF requests the PCF to report the PLMN identifier where the UE is currently located. The received PLMN ID information is then forwarded in the SIP REGISTER request.

This procedure shall be applied by the P-CSCF at initial UE IMS registration.

Figure W.3-1: Subscription by P-CSCF to changes in PLMN ID during initial IMS Registration

1.	The UE sends a SIP REGISTER request to the P-CSCF.

2.	If this is initial IMS registration then the P-CSCF subscribes to the PCF to be notified of the PLMN ID where the UE is currently attached.

3. The PCF forwards the PLMN ID to the P-CSCF. The P-CSCF stores the PLMN ID.

4.	The P-CSCF includes the received PLMN ID in the SIP REGISTER request before forwarding the request to the I-CSCF.

5.	Normal IMS registration procedure is then completed.

*** END CHANGES ***