SA WG2 Meeting #122	S2-175175

26 - 30 June 2017, San Jose Del Cabo, Mexico                   	(revision of S2-174644)

Source:	CATT

Title:	TS 23.502: Update registration procedure including mobility policy acquirement in the roaming case

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution involves the acquirement procedure of mobility policy when UE is registered in VPLMN.

- Discussion

According to A.2.2.4 in TS 23.501 V0.5.0:

The N24 reference point resides between a PCF in the HPLMN (H-PCF) and a PCF in the VPLMN (V-PCF).

For roaming scenario, the N24 enables the H-PCF to:

-	Provision mobility policy rules to V-PCF in the VPLMN;

-	Handling of UE Context Establishment request sent by the V-PCF as part of UE Registration procedure(s);

It is concluded that when UE is registering in VPLMN, the AMF that UE is registering to requests the mobility policy from V-PCF and V-PCF acquires the mobility policy from H-PCF using N24 interface. However, there is no description on it in registration procedure of TS 23.502 V0.4.0.

2	Proposal

It is proposed to agree on the proposed changes to TS 23.501.

*************** Start of changes *********************

4.2.2.2.2	General Registration

Figure 4.2.2.2.2-1: Registration procedure

1.	UE to (R)AN: AN message (AN parameters, Registration Request (Registration type, Subscriber Permanent Identifier or Temporary User ID, Security parameters, NSSAI, UE 5GCN Capability, PDU session status, PDU session(s) to be re-activated and MICO mode preference)).

Editor's note:	The required AN parameters is FFS.

In case of 5G-RAN, the AN parameters include e.g. SUPI or the Temporary User ID, the Selected Network and NSSAI.

The Registration type indicates if the UE wants to perform an "initial registration" (i.e. the UE is in RM-DEREGISTERED state), a "mobility registration update " (i.e. the UE is in registered state and initiates a Registration procedure due to mobility) or a "periodic registration update" (i.e. the UE is in registered state and initiates a Registration procedure due to the periodic update timer expiry, see clause 4.2.2.2.1). The UE performing an initial registration (i.e., the UE is in RM-DEREGISTERED state) to a PLMN for which the UE does not already have a 5G-GUTI, the UE shall include its SUPI in the registration attempt. In other cases, the 5G-GUTI is included and it indicates the last serving AMF. If the UE is already registered via a non-3GPP access in a PLMN different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the 3GPP access, the UE shall not provide over the 3GPP access the 5G-GUTI allocated by the AMF during the registration procedure over the non-3GPP access. Also, if the UE is already registered via a 3GPP access in a PLMN (i.e. the registered PLMN), different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the non-3GPP access, the UE shall not provide over the non-3GPP access the 5G-GUTI allocated by the AMF during the registration procedure over the 3GPP access.

The Security parameters are used for Authentication and integrity protection. NSSAI indicates the Network Slice Selection Assistance Information (as defined in clause 5.15 of TS 23.501 [2]. The PDU session status indicates the previously established PDU sessions in the UE. The "PDU session(s) to be re-activated" is included to indicate the PDU session(s) that the UE intends to activate.

Editor's note:	The content of the Security parameters is FFS.

Editor's note:	Changes required to support "stickiness" is FFS.

Editor's note:	It is FFS if HO attach indication is to be added.

Editor's note:	It is FFS how UE Radio capabilities are provided.

2.	If a SUPI is included or the Temporary User ID does not indicate a valid AMF the (R)AN, based on (R)AT and NSSAI, if available, selects an AMF.

The (R)AN selects an AMF as described in TS 23.501 [2], clause X.

If the (R)AN cannot select an appropriate AMF, it forwards the Registration Request to an AMF which has been configured, in (R)AN, to perform AMF selection.

3.	(R)AN to new AMF: N2 message (N2 parameters, Registration Request (Registration type, Subscriber Permanent Identifier or Temporary User ID, Security parameters, NSSAI and MICO mode preference)).

When 5G-RAN is used, the N2 parameters include the Location Information, Cell Identity and the RAT type related to the cell in which the UE is camping.

If the Registration type indicated by the UE is periodic registration update, then steps 4 to 17 may be omitted.

4.	[Conditional] new AMF to old AMF: Information Request (complete Registration Request).

If the UE's Temporary User ID was included in the Registration Request and the serving AMF has changed since last registration, the new AMF may send Information Request to old AMF including the complete Registration Request IE to request the UE's SUPI and MM Context.

5.	[Conditional] old AMF to new AMF: Information Response (SUPI, MM Context, SMF information).

Old AMF responds with Information Response to new AMF including the UE's SUPI and MM Context.

If old AMF holds information about active PDU Sessions, the old AMF includes SMF information including SMF identities and PDU session identities.

6.	[Conditional] new AMF to UE: Identity Request ().

If the SUPI is not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE.

7.	[Conditional] UE to new AMF: Identity Response ().

The UE responds with an Identity Response message including the SUPI.

8.	The AMF may decide to invoke an AUSF. In that case, the AMF, shall be based on SUPI, select an AUSF as described in TS 23.501 [2], clause X.

9a.	The AUSF shall initiate authentication of the UE.

The authentication is performed as described in clause X.

Editor's note:	It is FFS how UDM is selected in this step.

Editor's note:	It is FFS whether the AUSF or the AMF initiates the authentication.

If network slicing is used, the AMF decides if the Registration Request needs to be rerouted as described in clause 4.2.2.2.3, where the initial AMF refers to the AMF.

9b	The AMF shall initiate NAS security functions.

The NAS security is performed as described in clause X.

10.	[Conditional] new AMF to old AMF: Information Acknowledged ().

If the AMF has changed the new AMF acknowledge the transfer of UE MM context.

If the authentication/security procedure fails, then the Registration shall be rejected, and the new AMF sends a reject indication to the old AMF. The old AMF continues as if the Information Request was never received.

11.	[Conditional] new AMF to UE: Identity Request ().

If the PEI was not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE to retrieve the PEI.

12.	Optionally the new AMF initiates ME identity check.

The PEI check is performed as described in clause 4.7.

13.	If step 14 is to be performed, the new AMF, based on the SUPI, selects a UDM.

The AMF selects an UDM as described in TS 23.501 [2], clause X.

14.	If the AMF has changed since the last registration, or if there is no valid subscription context for the UE in the AMF, or if the UE provides a SUPI which doesn't refer to a valid context in the AMF, or if the UE registers to the same AMF it has already registered to a non-3GPP access (i.e. the UE is registered over a non-3GPP access and initiates this registration procedure to add a 3GPP access), the new AMF initiates the Update Location procedure. The new AMF provides the access type it serves for the UE to the UDM and the access type is set to "3GPP access". The UDM stores the associated access type together with the serving AMF. This will cause the UDM to initiate a Cancel Location to the old AMF corresponding to 3GPP access, if one exists. The old AMF removes the MM context and notifies all possibly associated SMF(s), and the new AMF creates an MM context for the UE after getting the AMF related subscription data from the UDM.

The Update Location procedure is performed as described in clause X.

Editor's note:	Whether interactions with UDM and PCF includes mobility restriction related information is FFS.

Editor's note:	The PEI is to be provided to the UDM in the Update Location procedure.

Editor's note:	It is FFS how to update the figure with the "UDM initiates Cancel Location to old AMF".

15.	Conditionally the new AMF, based on the SUPI, selects a PCF.

The AMF selects a PCF as described in TS 23.501 [2], clause X.

Editor's note:	It is FFS whether the Permanent User ID is used for PCF selection.

Editor's note:	The conditions when the AMF selects a PCF is FFS.

16.	[Optional] new AMF to PCF: UE Context Establishment Request (SUPI).

If the AMF has not yet obtained Access and Mobility policy for the UE or if the Access and Mobility policy in the AMF are no longer valid, the AMF requests the PCF to apply operator policies for the UE. In roaming case, the interaction between H-PCF and V-PCF is required for the provision of Access and Mobility policy.

17.	PCF to new AMF: UE Context Establishment Acknowledged (Access and Mobility policy data).

The PCF acknowledges the UE Context Establishment Request message and provides the Access and Mobility policy data for the UE to the AMF.

18.	[Conditional] new AMF to SMF: N11 Request ().

The N11 Request is sent in one or both of the following scenarios:

-	If the AMF is changed, the new AMF notifies each SMF of the new AMF serving the UE.The AMF verifies PDU session status from the UE with the available SMF information. In case the AMF has changed, it is assumed that the old AMF provides the available SMF information. The new AMF requests the SMF to release any network resources related to PDU sessions that the UE indicated as not established.

-	If the UE was in MICO mode and the AMF had notified an SMF of the UE being unreachable and that the SMF needs not to send DL data notifications to the AMF, the AMF informs the SMF that the UE is reachable.

NOTE:	the AMF will also notify any other NF that subscribed to UE reachability that the UE is reachable.

Editor's note:	It is FFS whether AMF perform the PDU session status logic or each SMF does it.

19.	SMF to new AMF: N11 Response ().

The SMF may decide to trigger e.g. UPF relocation as described in clause 4.3.5.

If the Registration type indicated by the UE is periodic registration update, then steps 20 and 21 may be omitted.

Editor's note:	It is FFS how SMFs subscribes to AMF services.

Editor's note:	SMF relocation in relation to a Registration procedure is FFS.

20.	[Conditional] old AMF to PCF: UE Context Termination Request ().

If the old AMF previously requested UE context to be established in the PCF, the old AMF terminates the UE context in the PCF.

21.	PCF to old AMF: UE Context Termination Acknowledged ().

22.	New AMF to UE: Registration Accept (Temporary User ID, Registration area. Mobility restrictions, PDU session status, NSSAI, Periodic registration update timer, LADN Information and accepted MICO mode).

The AMF sends a Registration Accept message to the UE indicating that the registration has been accepted. Temporary User ID is included if the AMF allocates a new Temporary User ID. Mobility restrictions is included in case mobility restrictions applies for the UE. The AMF indicates the established PDU sessions to the UE in the PDU session status. The UE removes locally any internal resources related to PDU sessions that are not marked as established in the received PDU session status and for which the UE has requested PDU session establishment and not received SMF response yet. If the PDU session status information was in the Registration Request, the AMF shall indicate the PDU session status to the UE. The NSSAI includes the Allowed S-NSSAIs. If the UE subscription data includes subscribed LADN identification information, the AMF shall include in the Registration Accept message the LADN Information for LADNs, defined in TS 23.501 [2] clause 5.6.5, that are available within the Registration area determined by the AMF for the UE. If the UE included MICO mode in the request, then AMF responds whether MICO mode should be used.

If the "PDU session(s) to be re-activated" is included in the Registration Request, the AMF sends an N11 Message to SMF(s) associated with the PDU Session ID(s) to establish the User Plane(s) for the PDU session(s) in conjunction with the Registration Accept message. The steps 4b to 12 specified in clause 4.2.3.2 are executed to complete the User Plane(s) setup without sending MM NAS Service Accept from the AMF to (R)AN described in step 5 of clause 4.2.3.2.

Editor's note:	It is FFS whether NSSAI is provided to RAN by AMF.

Editor's note:	It is FFS whether LADN subscription information (i.e. information which indicates whether the UE subscribes the LADN service) is included in UE subscription data or is managed by policy information by PCF.

23.	[Conditional] UE to new AMF: Registration Complete ().

The UE sends a Registration Complete message to the AMF to acknowledge if a new Temporary User ID was assigned.

When the "PDU session(s) to be re-activated" is not included in the Registration Request, the AMF releases the signalling connection with UE, according to clause 4.2.6.

Editor's note:	Details of N2 interaction is FFS.

*************** End of changes *********************