SA WG2 Meeting #122	S2-175217

26th – 30th June 2017, Cabo, Mexico

Source:	Huawei, Hisilicon

Title:	On Multiple Network Slice instances

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution analyzes a few aspects of impacts due to introduction of multiple NSi for the same S-NSSAI at SA2#121.

# 1	Background

….

# 2	Proposal

Question 1: Discuss and agree which of the 3 options to implement.

The following changes, implementing Observation 1-4, to TS 23.501 are proposed below.

***** START of CHANGE *****

## 5.15	Network slicing

### 5.15.1	General

A Network Slice may include:

-	the Core Network Control Plane and user plane Network Functions, as described in clause 4.2,

-	the 5G Radio Access Network described in TS 38.xxx [x],

-	the N3IWF functions to the non-3GPP Access Network described in clause 4.2.7.2.

Network slicing support for roaming is described in clause 5.15.6.

Network slices may differ for supported features and network functions optimisations. The operator may deploy multiple Network Slice instances delivering exactly the same features but for different groups of UEs, e.g. as they deliver a different committed service and/or because they may be dedicated to a customer.

A single UE can simultaneously be served by one or more Network Slice instances via a 5G-AN. A single UE may be served by at most eight Network Slices at a time. The AMF instance serving the UE logically belongs to each of the Network Slice instances serving the UE, i.e. this AMF instance is common to the Network Slice instances serving a UE.

Editor's note:	The following text is FFS:

-	The AMF discovery and selection for the set of slices for a UE is triggered by the first contacted AMF in a registration procedure and it may lead to change of AMF. SMF discovery and selection is initiated by the AMF when a SM message to establish a PDU session is received from the UE. The NRF is used to assist the discovery and selection tasks.

A PDU session belongs to one and only one specific Network Slice instance per PLMN. Different Network Slice instances do not share a PDU session, though different slices may have slice-specific PDU sessions using the same DNN.

### 5.15.2	Identification and selection of a Network Slice: The S-NSSAI and the NSSAI

#### 5.15.2.1	General

An S-NSSAI (Single Network Slice Selection Assistance information) identifies a Network Slice.

An S-NSSAI is comprised of:

-	A Slice/Service type (SST), which refers to the expected Network Slice behaviour in terms of features and services;

-	A Slice Differentiator (SD). which is optional information that complements the Slice/Service type(s) to differentiate amongst multiple Network Slices of the same Slice/Service type.

The S-NSSAI can have standard values or PLMN-specific values. S-NSSAIs with PLMN-specific values are associated to the PLMN ID of the PLMN that assigns it. An S-NSSAI shall not be used by the UE in access stratum procedures in any PLMN other than the one to which the S-NSSAI is associated.

Editor's note:	Whether a single value which is a representation of a collection of the S-NSSAIs could also be used as NSSAI is FFS.

The NSSAI is a collection of S-NSSAIs (Single Network Slice Selection Assistance Information). There can be at most eight S-NSSAIs in the NSSAI sent in signalling messages between the UE and the Network. Each S-NSSAI assists the network in selecting a particular Network Slice instance.

The same Network Slice instance may be selected by means of different S-NSSAIs.

Based on the operator's operational or deployment needs, multiple Network Slice instances of a given S-NSSAI may be deployed in the same or in different registration areas. When multiple Network Slice instances of a given S-NSSAI are deployed in the same registration area, the AMF instance serving the UE may logically belong to more than one Network Slice instances of that S-NSSAI, i.e. this AMF instance may be common to multiple Network Slice instances of that S-NSSAI.  When a S-NSSAI is supported by more than one Network Slice instance in a PLMN, any of the Network Slice instances supporting the same S-NSSAI in a certain area may serve, as a result of the Network Slice instance selection procedure defined in clause 5.15.5, a UE which is allowed to use this S-NSSAI. Upon association with an S-NSSAI, the UE is served by the same Network Slice instance for that S-NSSAI until cases occur where e.g. Network Slice instance is no longer valid in a given registration area, or a change in UE’s Allowed NSSAI occurs etc. In such cases, procedures mentioned in clause 5.15.5.2.2 or clause 5.15.5.2.3 applies.

The selection of a Network Slice instance serving a UE and the Core Network Control Plane and user plane Network Functions corresponding to the Network Slice instance is the responsibility of 5GC.

The (R)AN may use Requested NSSAI in access stratum signalling to handle the UE Control Plane connection before the 5GC informs the (R)AN of the Allowed NSSAI. The Requested NSSAI is not used by the RAN for routing when the UE provides also a Temporary User ID.

Editor's note:	Whether (R)AN uses Requested NSSAI is to be checked with RAN WGs.

When a UE is successfully registered, the CN informs the (R)AN by providing the whole Allowed NSSAI for the Control Plane aspects.

Editor's note:	The need for informing the (R)AN by providing the whole Allowed NSSAI is to be checked with RAN WGs.

When a PDU Session for a given S-NSSAI is established using a specific Network Slice instance, the CN provides to the (R)AN the S-NSSAI corresponding to this Network Slice instance to enable the RAN to perform access specific functions.

NOTE:	The details of how the RAN uses NSSAI information is described in TS 38.xxx [x].

***** END of CHANGE *****