# SA WG2 Meeting #S2-125	S2-180025

# 22 -26 January, 2018, Gothenburg, Sweden

# 3GPP TSG CT WG 1 Meeting 107	TDoc C1-175378

# Reno (USA), 27 November - 1 December 2017

Title:	LS on timers for the network-requested PDU session modification procedure and "SSC mode 3"

Response to:	-

Release:	Release 15

Work Item:	5GS_Ph1-CT

Source:	CT1

To:	SA2

Cc:	-

Contact person:	Christian Herrero-Veron

Christian.Herrero at Huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 have analyzed the stage 2 requirements on the network-requested PDU session modification procedure and "SSC mode 3" in TS 23.502 subclause 4.3.5.2. CT1 has identified several issues with the stage 2 text and has several questions regarding the motivation, the objectives and the proposed solution, as follows:

- What is the purpose of the PDU session release timer provided to the UE?

The timer is signalled in the NAS message, but in the CT1 view, the NAS layer in the UE has no use for this timer. If the timer is to be used by upper layers, why is it signalled as an IE in a NAS message? NAS layer in the UE layer has no means of controlling the transfer of IP flows between PDU sessions unless, e.g., a release indication from upper layer is received. Therefore, the NAS layer can either initiate PDU session release of the old session or release it locally or perform no action, without the need for a timer from the network. NAS layer will not check what is occurring in the user-plane corresponding to the PDU session.

- Is the timer supposed to be used by the upper layers in the UE? If so, some use cases would be useful.

Without further input from SA2, CT1 is of the opinion that the UE does not need to run the timer but the SMF can control the release of the old PDU session. If the timer is to be used at the upper layers, CT1 would prefer providing the timer value to the UE by transparent means, e.g., ePCO IE.

# 2	Actions

To SA2

ACTION: 	3GPP TSG CT WG1 kindly ask SA2 to answer above questions and update their specifications.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 108	22 - 26 January 2018	Gothenburg, Sweden, EU

TSG CT WG1 Meeting 109	26 Feb - 2 March 2018	TBD, North America