SA WG2 Meeting #127BIS	S2- 184837

28 May - 1 June 2018, Newport Beach, CA, US	(revision of S2-183154)

Source:	Ericsson

Title:	Solution for KI#12 V2X interworking

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	FS_eV2XARC / Rel-16

Abstract of the contribution: This contribution proposes a solution for V2X interworking as defined in KI#12 focusing on the UE V2X provisioning.

# Discussion

In the Key Issue #12 there are three aspects listed to be addressed and in this contribution one of them is put in focus:

-	How to support roaming/interworking operation of the V2X UE between a V2X system based on EPC and that based on 5GC, e.g. how to handle the V2X policy/parameters from V2X Control Function via EPS and those delivered via 5GS.

The analysis is based on the use of the architecture shown in Annex A.1 in TR23.786.

V2X capable UEs not supporting 5GC NAS will only be connected to EPC and be provisioned using the V2X Control Function in EPC over the V3 reference point. These UEs will never be connected to 5GC.

V2X capable UEs both supporting EPC NAS and 5GC NAS can be connected to either EPC or 5GC. The radio parameters provisioned in the UE for PC5 communication may be the same in EPS and 5GS. The service parameters to be provisioned in the UE may be the same but there will probably also be differences if connected to 5GS or EPS e.g. some services can only be used if accessing via 5G.

The V2X service parameters provisioned in the UE are usually static and only expected to be changed if subscription is updated or some application servers are reconfigured, added or deleted i.e. it is no dynamic parameters that needs continues update to the UE. Besides, the V2X policy/parameters are not session parameters (e.g. session type) that needs to be mapped between PDN connection and PDU session when UE moves between EPC and 5GC, and there is no need for V2X Control Function and PCF in Annex A.1 to interact when UE moves between EPC and 5GC.

The V2X Control Function can be seen as a container keeping a set of static configuration parameters. By this the V2X Control Function cannot be compared and do not have the requirements as the dynamic interworking functionality specified in TS23.501 clause 4.3 e.g. PCF+PCRF.

To avoid adding complexity to 5GC it is proposed to handle the “interworking” between EPC and 5GC as follows, see also figure 1:

1.	A UE connecting to EPC will be provisioned via the V2X Control Function over V3.

2.	A UE connecting to 5GC will be provisioned via the existing 'UE Configuration Update procedure for transparent UE Policy delivery'.

UPF + PGW-U

UE

N26

S5-U

S5-C

S6a

S11

N1

N4

N7

U

N2

S1-U

AMF

S1-MME

HSS + UDM

SMF + PGW-C

N11

N3

HPLMN

VPLMN

N24

N15

N8

UE

E-UTRAN

N10

NG-RAN

MME

SGW

v-PCF + v-PCRF

h-PCF + h-PCRF

v-V2X CF

h-V2X CF

V3

V4

V6

Figure 1: Local breakout roaming architecture for interworking between 5GS and EPC/E-UTRAN incl. V2X Control Function

# Proposal

It is proposed to add the solution below for V2X interworking in TR23.786.

***** First Change *****

# 6	Solutions

## 6.X	Solution #X: Interworking for eV2X

### 6.X.1	Introduction

In the Key Issue #12 there are three aspects listed to be addressed and in this contribution one of this is put in focus:

-	How to support roaming/interworking operation of the V2X UE between a V2X system based on EPC and that based on 5GC, e.g. how to handle the V2X policy/parameters from V2X Control Function via EPS and those delivered via 5GS.

The analyse is based on the use of the architecture shown in Annex A.1 in TR23.786.

### 6.X.2	Functional Description

V2X capable UEs not supporting 5GC NAS will only be connected to EPC and be provisioned using the V2X Control Function in EPC over the V3 reference point. These UEs will never be connected to 5GC.

V2X capable UEs both supporting EPC NAS and 5GC NAS can be connected to either EPC or 5GC. The radio parameters provisioned in the UE for PC5 communication may be the same in EPS and 5GS. The service parameters to be provisioned in the UE may be the same but there will probably also be differences if connected to 5GS or EPS e.g. some services can only be used if accessing via 5G.

The V2X service parameters provisioned in the UE are usually static and only expected to be changed if subscription is updated or some application servers are reconfigured, added or deleted i.e. it is no dynamic parameters that needs continues update to the UE. Besides, the V2X policy/parameters are not session parameters (e.g. session type) that needs to be mapped between PDN connection and PDU session when UE moves between EPC and 5GC, and there is no need for V2X Control Function and PCF in Annex A.1 to interact when UE moves between EPC and 5GC.

The V2X Control Function can be seen as a container keeping a set of static configuration parameters. By this the V2X Control Function cannot be compared and do not have the requirements as the dynamic interworking functionality specified in TS23.501 clause 4.3 e.g. PCF+PCRF.

To avoid adding complexity to 5GC it is proposed to handle the “interworking” between EPC and 5GC as follows, see also figure 1:

1.	A UE connecting to EPC will be provisioned via the V2X Control Function over V3.

2.	A UE connecting to 5GC will be provisioned via the existing 'UE Configuration Update procedure for transparent UE Policy delivery'.

UPF + PGW-U

UE

N26

S5-U

S5-C

S6a

S11

N1

N4

N7

U

N2

S1-U

AMF

S1-MME

HSS + UDM

SMF + PGW-C

N11

N3

HPLMN

VPLMN

N24

N15

N8

UE

E-UTRAN

N10

NG-RAN

MME

SGW

v-PCF + v-PCRF

h-PCF + h-PCRF

v-V2X CF

h-V2X CF

V3

V4

V6

Figure 1: Local breakout roaming architecture for interworking between 5GS and EPC/E-UTRAN incl. V2X Control Function

### 6.X.4	Procedures

UE connected to EPC the existing procedures for V2X provisioning is used as specified in TS23.285.

UE connected to 5GC the existing

-	Clause 4.2.4.3 UE Configuration Update procedure for transparent UE Policy delivery in TS 23.502.

-	Clause 6.1.2 Non-session management related policy control in TS 23.503.

### 6.X.5	Impacts on existing entities and interfaces

No effects on the EPC or 5GC as existing functionality is used.

### 6.X.6	Evaluation

Editor's Note: This clause provides an evaluation of the solution.

***** End of Changes *****