3GPP TSG-SA WG2 Meeting #127Bis	S2-185631

Newport Beach, USA, 28 May – 1 June, 2018	(revision of S2-185534)

Source:	Huawei, HiSilicon

Title:	Solution for KI 1#: Redundant Transmission for URLLC service

Document for:	Approval

Agenda Item:	6.20

Work Item / Release:	FS_URLLC /Rel-16

Abstract: This paper proposes solution to Key Issue #1: Supporting high reliability by redundant transmission in user plane.

1	Introduction

This contribution proposes a solution on how to establish the redundant transmission paths for URLLC service.

Changes in S2-185534:

- Split RT mode 1 and mode 2 into two solutions for different scenaios.

- Add the assumption of the solutions that reliability of UPF and CP NFs (and RAN node in solution 6.Y) are assumed high enough for the URLLC service they serve based on implementation/deployment mechanisms..

- Clarify the difference from current DC architecture.

- Clarify whether HR roaming need to be supported is FFS.

2	Proposal

It is proposed to agree the following P-CR to TR 23.725.

* * * * Start of Change, all new texts * * * *

6.X	Solution for Key Issue #1: Supporting redundant data transmission via single UPF and two RAN nodes

6.X.1	Description

This solution addresses the KI#1 of How to “Supporting high reliability by redundant transmission in user plane”.

Typically telecom-level equipments are able to provide up to five-nine reliability. However, in many cases the end-to-end reliability of real deployed network might be lower than that considering the susceptible deployment environments, especially in the last-mile.

To realize ultra-high reliability for URLLC services in the commercial deployed network, this solution proposes to use redundant transmission over different physical transport paths in CN and RAN to enhance the reliability of upper layer service.

Whether redundant transmission need to be activated for a QoS Flow can be decided by the SMF based on local policies for the DNN or slice, or by the PCF based on its QoS requirement, UE’s subscription and condition of network deployment.

In this solution, it is assumed that the reliability of UPF and CP NFs are high enough to fulfill the reliability requirement of URLLC services served by these NFs, and the UE is under overlapping coverage of the two RAN nodes. The reliabilities of these NFs can be realized based on implementation (e.g. redundant mechanisms provided by NFV platform), which is out of scope of this solution. However a single RAN node cannot provide enough reliability in air interface for the QoS flow, so the redundant packets will be transferred between UPF and UE via two independent N3 tunnel and two RAN nodes to enhance the reliability of service.

Editor’s Note: How to ensure the two N3 tunnels can be transferred via different transport layer paths are FFS.

Editor’s Note: The scenario that N3 UPF involved is FFS.

Editor’s Note: Whether and How to support redundant transmission in HR roaming scenario is FFS.

Editor’s Note: How to support redundant transmission during Handover procedure is FFS.

The redundant transmission is based on the DC architecture, except that same PDU is transferred via both M-RAN and S-RAN nodes. With two different NG-RAN nodes, separate transport layer paths are used for redundant data transmission in user plane.

Figure 6.x.1-1: RT Mode 1 - Redundant transmission with two N3 tunnels between a UPF and two NG-RAN nodes

A HRP (High Reliability Protocol) layer are implemented on the UE and UPF to support packet replication and elimination function.

6.x.2	Procedures

6.x.2.1	Activate Redundant Transmission during PDU Session Establishment procedure

The redundant transmission will be established during a PDU Session establishment procedure or a PDU Session Modification procedure.

Figure 6.x.2.1-1 depicts the establishment of redundant transmission during the PDU session establishment procedure in the non-roaming and roaming with local breakout cases.

Figure 6.x.2.1-1: Activate Redundant Transmission during PDU Session Establishment procedure

1. Steps 1-12 from clause 4.3.2.2.1(UE requested PDU Session Establishment) in TS 23.502 [3] are performed with the following differences:

In step 7b, the PCF indicates the SMF and the NG-RAN to activate redundant transmission for the QoS flow by the 5QI if the UE requested PDU Session Establishment procedure is initiated for a URLLC service.

The SMF then initiates an N4 Session Establishment procedure with the selected UPF and CN Tunnel Info 1& CN Tunnel Info 2 are allocated by the SMF or UPF.

In step 11, the N2 SM information includes the QFI(s), QoS Profile(s), CN Tunnel Info 1& CN Tunnel Info 2 corresponding to each N3 tunnel.

2. RRC Connection Reconfiguration takes place with the UE establishing the necessary NG-RAN resources related to the QoS Rules for the PDU Session request. The Secondary NG-RAN node Addition procedure is performed as defined in TS 37.340 [x].

3. Steps 14-20 from clause 4.3.2.2.1(UE requested PDU Session Establishment) in TS 23.502 [3] are performed with the following differences:

The AN Tunnel Info provided by the NG-RAN includes AN Tunnel Info 1 and AN Tunnel Info 2. each AN Tunnel Info includes a tunnel endpoint for each involved NG-RAN node, and the QFIs assigned to each tunnel endpoint.

The SMF provides AN Tunnel Info 1 and AN Tunnel Info 2 to the UPF as well as the corresponding forwarding rules. The forwarding rules indicates the UPF to replicate the downlink packets and send each replicate via one of DL N3 tunnels separately to NG-RAN, and eliminate the duplicate uplink packets received from the two UL N3 tunnels.

Editor’s Note:   How to associate a CN tunnel info and an AN tunnel Info is FFS.

6.x.2.2	Activate Redundant Transmission during PDU Session Modification procedure

Figure 6.x.2.1-2 depicts the establishment of redundant transmission during the UE or network requested  PDU session modification procedure in the non-roaming and roaming with local breakout cases.

Figure 6.x.2.2-1: Activate Redundant Transmission during PDU Session Modification procedure

1. Steps 1-12 from clause 4.3.3.2 (UE or network requested PDU Session Modification) in TS 23.502 [3] are performed with the following differences:

The PDU Session Modification procedure is triggered by the UE or the SMF to add a QoS flow for a URLLC service.

In step 1b or step 2, the PCF indicates the SMF to activate redundant transmission for the QoS flow by the 5QI.

The SMF initiates an N4 Session modification procedure with the UPF and CN Tunnel Info 1& CN Tunnel Info 2 are allocated by the SMF or UPF.

The N2 SM information includes the QFI(s), QoS Profile(s), CN Tunnel Info 1& CN Tunnel Info 2 corresponding to each N3 tunnel.

2. RRC Connection Reconfiguration takes place with the UE establishing the necessary NG-RAN resources related to the QoS Rules for the PDU Session request. The Secondary NG-RAN node Addition procedure can be performed as defined in TS 37.340 [x].

3. Steps 6-12 f rom clause 4.3.3.2 (UE or network requested PDU Session Modification) in TS 23.502 [3] are performed with the following differences:

The AN Tunnel Info provided by the NG-RAN includes AN Tunnel Info 1 and AN Tunnel Info 2. In case of Mode 1, each AN Tunnel Info includes a tunnel endpoint for each involved NG-RAN node, and the QFIs assigned to each tunnel endpoint.

The SMF provides AN Tunnel Info 1 and AN Tunnel Info 2 to the UPF as well as the corresponding forwarding rules. The forwarding rules indicates the UPF to replicate the downlink packets and send each replicate via one of DL N3 tunnels separately to NG-RAN, and eliminate the duplicate uplink packets received from the two UL N3 tunnels.

6.X.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

6.X.4	Solution Evaluation

Editor's note:	This clause provides an evaluation of this solution.

* * * * 2nd Change * * * *

6.Y	Solution for Key Issue #1: Supporting redundant data transmission via single UPF and single RAN node

6.Y.1	Description

This solution addresses the KI#1 of How to “Supporting high reliability by redundant transmission in user plane”.

In this solution, it is assumed that the reliability of NG-RAN node, UPF and CP NFs are high enough to fulfill the reliability requirement of URLLC services served by these NFs. The reliabilities of these NFs can be realized based on implementation (e.g. redundant mechanisms provided by NFV platform), which is out of scope of this solution. However, the reliability of single N3 tunnel is considered not high enough, e.g. due to the deployment environment of backhaul network, so the redundant packets will be transferred between UPF and RAN via two independent N3 tunnel over different transport layer path to enhance the reliability of service.

Editor’s Note: How to ensure the two N3 tunnels can be transferred via different transport layer paths are FFS.

Editor’s Note: The scenario that N3 UPF involved is FFS.

Editor’s Note: Whether and how to support redundant transmission in roaming scenario is FFS.

Editor’s Note: How to support redundant transmission during Handover procedure is FFS.

This solution supports the redundant transmission based on two N3 tunnels between a single NG-RAN mode and the UPF. The RAN node and UPF shall support the packet replication and elimination function.

Figure 6.y.1-1: Redundant transmission with two N3 tunnels between the UPF and a single NG-RAN node

A HRP (High Reliability Protocol) layer is implemented on the RAN and UPF to support packet replication and elimination function.

Editor's note: The format of HRP header and whether the HRP layer can be realized by enhancing existing protocol layers (e.g. GTP-U) are FFS.

6.y.2	Procedures

The Procedures for activate redandunt transimission during PDU session establishment/modification are same as corresponding procedures shown in clause 6.x.2 except for both of two AN tunnels are terminated on M-RAN nodes and S-RAN node addition is not needed.

6.X.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

6.X.4	Solution Evaluation

Editor's note:	This clause provides an evaluation of this solution.

* * * * End of Changes * * * *