3GPP TSG-SA2 Meeting #127	S2-183312

# Sanya, China, 16th - 20th April 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0236 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modifications for 6 bit QFI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There will only be 6 bits available for the QFI in 5G RAN. Therefore, it will not always be possible to use the signalling option of QFI=5QI. Also, some new 5QI’s have not been assigned numbers. For delay critical GBR Flows and GBR Flows, values above 64 should be selected, since dynamically assigned QFI’s are always used for GBR flows, and QFI=5QI can not be used anyway. For non-GBR flows, it would be possible to use the QFI=5QI signalling option if 5QI’s below 64 are used. However, all non-GBR 5QI’s correspond to QCI’s already standardized, and interworking between the systems would be simplified if the values are the same. Also, any operator that want to use the direct signalling option may define pre-configured 5QI with values below 64, with the same characteristics as the standardized 5QI. Therefore, we propose to keep the same values as in LTE when applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the 5QI can not be used as QFI if it is outside the range of the QFI. - Select values above 64 for all new delay critical GBR 5QI’s and GBR 5QI’s. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the 5QI may be signalled when the QFI is 6 bits. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1.3, 5.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************************************Start of 1st change*****************************************

#### 5.7.1.3	Control of QoS Flows

The following options are supported to control QoS Flows:

1)	For Non-GBR QoS Flows, and when standardized 5QIs or pre-configured 5QIs are used, and when the 5QI is within the range of the QFI, the 5QI value may be used as the QFI of the QoS Flow.

(a)	A default ARP shall be pre-configured in the AN; or

NOTE 1:	The above 1a option is intended to be used for non-3GPP ANs (e.g. Fixed AN) scenarios when there is no need for any N1 signalling including PDU Session Establishment, nor any N2 signalling.

(b)	The ARP and the QFI shall be sent to RAN over N2 at PDU Session Establishment or at PDU Session Modification and when NG-RAN is used every time the User Plane of the PDU Session is activated; and

2)	For all other cases (including GBR and Non-GBR QoS Flows), a dynamically assigned QFI shall be used. The 5QI value may be a standardized, pre-configured or dynamically assigned. The QoS profile and the QFI of a QoS Flow shall be provided to the (R)AN over N2 at PDU Session Establishment/Modification and when NG-RAN is used every time the User Plane of the PDU Session is activated.

NOTE 2:	The options 1b and 2 are intended to be used for 3GPP ANs.

NOTE 3:	Pre-configured 5QI values cannot be used when the UE is roaming.

**************************************Start of 2nd change*****************************************

### 5.7.4	Standardized 5QI to QoS characteristics mapping

The one-to-one mapping of standardized 5QI values to 5G QoS characteristics is specified in table 5.7.4-1.

Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR NOTE 1 | 20 | 100 ms | 10-2 | N/A | TBD | Conversational Voice |
| 2 |  | 40 | 150 ms | 10-3 | N/A | TBD | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms | 10-3 | N/A | TBD | Real Time Gaming, V2X messages Electricity distribution – medium voltage, Process automation - monitoring |
| 4 |  | 50 | 300 ms | 10-6 | N/A | TBD | Non-Conversational Video (Buffered Streaming) |
| 65 |  | 7 | 75 ms | 10-2 | N/A | TBD | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| 66 |  | 20 | 100 ms | 10-2 | N/A | TBD | Non-Mission-Critical user plane Push To Talk voice |
| 75 |  | 25 | 50 ms | 10-2 | N/A | TBD | V2X messages |
| 76 NOTE 4 |  | 18 | 10 ms | 10-4 | 255 B | TBD | Discrete Automation |
| 77 NOTE 4 |  | 19 | 10 ms | 10-4 | 1358 B NOTE 3 | TBD | Discrete Automation |
| 5 | Non-GBR NOTE 1 | 10 | 100 ms | 10-6 | N/A | N/A | IMS Signalling |
| 6 |  | 60 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 |  | 70 | 100 ms | 10-3 | N/A | N/A | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  | 80 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file |
| 9 |  | 90 |  |  | N/A | N/A | sharing, progressive video, etc.) |
| 69 |  | 5 | 60 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| 70 |  | 55 | 200 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as QCI 6/8/9) |
| 79 |  | 65 | 50 ms | 10-2 | N/A | N/A | V2X messages |
| 80 |  | 66 | 10 ms | 10-6 | N/A | N/A | Low Latency eMBB applications Augmented Reality |
| 81 |  | Delay Critical GBR |  | 11 |  |  | 5 ms |  | 10-5 | 160 B |  | TBD |  | Remote control (see TS 22.261 [2]) |  |
| 82 |  |  |  | 12 |  |  | 10 ms NOTE 5 |  | 10-5 | 320 B |  | TBD |  | Intelligent transport systems |  |
| 83 |  |  |  | 13 |  |  | 20 ms |  | 10-5 | 640 B |  | TBD |  |  |  |
| 84 |  |  |  | 18 |  |  | 10 ms |  | 10-4 | 255 B |  | TBD |  | Discrete Automation |  |
| 85 |  |  |  | 19 |  |  | 10 ms |  | 10-4 | 1358 B NOTE 3 |  | TBD |  | Discrete Automation |  |
| NOTE 1: a packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: it is required that default Maximum Data Burst Volume is supported by a PLMN supporting the related 5QIs. NOTE 3: This Maximum Burst Size value is intended to avoid IP fragmentation on an IPv6 based, IPSec protected, GTP tunnel to the 5G-AN node. NOTE 4: A delay of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 5: The jitter for this service is assumed to be 20 msec as per TS 22.261 [2]. |  |  |  |  |  |  |  |

NOTE:	For Standardized 5QI to QoS characteristics mapping, the table will be extended/updated to support service requirements for 5G, e.g. ultralow latency service.

**************************************End of changes*******************************************