SA WG2 Meeting #S2-127	S2-183590

23 - 27 April, 2018, Sanya, China

Title:	[DRAFT] Reply LS on QoS

Response to:	-

Release:	Release 15

Work Item:	5GS_Ph1

Source:	SA2

To:	RAN2, RAN3, CT4

Cc:                              CT1

Contact Person:

#### Name:	Aeneas Dodd-Noble

E-mail Address:	noblea at cisco dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

SA2 thanks RAN2 on their LS (R2-1801645) and would like to provide feedback in addition to the SA2 LS response in S2-182856.

This response provides further clarification on the question that RAN2 asked in their original LS:

QFI Size

For the SDAP sublayer, in order to keep the header contained within one byte, RAN2 has agreed to limit the number of QFI which can be signalled to 64. Therefore, RAN2 would like to understand if SA2 expects to use more than 64 reflective flows per PDU session per UE at a time.

SA2 has discussed this topic and come to the conclusion that limiting the QFI to 64 creates serious restrictions to the overall capability of the 5GS system and would like RAN2 to reconsider its decision, eg. consider how to efficiently encode larger range of QFI values. SA2 recommendation is that the size of QFI field be in the 12-14 bit range.

SA2 also requests CT4 and RAN3 to ensure that the size of QFI field on N3 and N9 interface is at least 2 bytes.

2. Actions:

To RAN2 group

ACTION: 	SA2 respectfully asks RAN2 to encode QFI value such that the max number of QFIs for PDU session is at least 1000 flows.

To CT4 and RAN3 group

ACTION: 	SA2 respectfully asks CT4 and RAN3 they should encode QFI on the N3 interface such that its length is at least 2 bytes.

3. Dates of Next TSG-SA2 Meetings:

| 3GPPSA2#127bis | 28 May - 1 Jun 2018 | Newport Beach | US |
| --- | --- | --- | --- |
| 3GPPSA2#128 | 2-7 July 2018 | Vilinius | LT |