SA WG2 Meeting #127	S2-183634

16 - 20 April, 2018, Sanya, China

Source:	Huawei, HiSilicon

Title:	Use Case: NWDA-assisting E2E QoS Assurance

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes a new use case in TR23.791 on the use of NWDAF to aid changes in the network or application to assure E2E QoS of 5G scenarios.

1 Discussion

Latest 3GPP technical specifications (TS 23.501 and TS23.503) define the NWDAF (Network Data Analytics Function). In Rel.15 the focus of NWDAF was to support PCF and NSSF operation with data analytics based on load level information from network slices.

Extracted from TS 23.503, Section 6.2.9:

Network Data Analytics Function (NWDAF)

NWDAF represents operator managed network analytics logical function. NWDAF provides slice specific network data analytics to the PCF and NSSF. NWDAF provides network data analytics (i.e., load level information) to PCF and NSSF on a network slice level and the NWDAF is not required to be aware of the current subscribers using the slice. NWDAF notifies / publishes slice specific network status analytic information to the PCF(s) and NSSF that are subscribed to it. PCF(s) and NSSF can collect directly slice specific network status analytic information from NWDAF. This information is not subscriber specific. PCF uses that data in its policy decisions. NSSF may use the load level information provided by NWDAF for slice selection.

NOTE 1:	NWDAF functionality beyond its support for Nnwdaf is out of scope of 3GPP.

NOTE 2:	NWDAF functionality for non-slice-specific analytics information is not supported in this release.

Nevertheless, with the establishment of the Study of enablers for Network Automation for 5G (FS_eNA), it becomes possible to investigate the usage of analytics (i.e., NWDAF) for supporting other NFs and the diverse services that the NFs will have to handle due to the introduction of different 5G scenario requirements as specified in TS 22.261.

Extracted from TR 23.791, Section 1:

1	Scope

The aim of this Technical Report is to study and specify how to collect data and how to feedback data analytics to the network functions.

One example of a 5G scenario defined in TS 22.261 is the URLLC. This type of scenario requires E2E QoS requirements (e.g., 5ms E2E latency for remote driving and 99.999% of reliability). The detection of situations that affect the E2E QoS provisioning is important in order to support the 5G user experience as stated in:

Extracted from TS 22.261 Section 6.7.1:

“Further, for QoS control in EPS only covers RAN and core network, but for 5G network E2E QoS (e.g., RAN, backhaul, core network, network to network interconnect) is needed to achieve the 5G user experience (e.g., ultra-low latency, ultra-high bandwidth).”

The E2E QoS in 5G scenarios is affected by parameters from AN, CN, and application, as illustrated in the figure below.

Furthermore, new initiatives in SA1 as well as 5GAA demonstrate the need for analytics and prediction specially related to E2E QoS in order to better support the service of verticals.

For instance the new study item in V2X have been approved in SA1 (FS_V2XIMP approved in S1-180451_ FeV2XIMP). The objects of this study item show that 5GS needs to support analytics for 5G scenarios, such as V2X.

Extracted from S1-180451 Section 4:

The new NESQO Work Item in 5GAA (Predictable QoS and End-to-end Network Slicing for Automotive Use Cases) show the importance of the requirements on E2E performance fulfillment and predictable QoS levels,

Extracted from 5GAA_A-180057

Despite the success of current mechanisms to provide QoS for Mobile networks, operators will have at least the following shortcomings for the detection of situations that affect E2E QoS and providing solutions for such situations:

- Information related to QoS performance is compartmentalized inside each one of the parts of the network (e.g., AN, CN, and application Server) limiting the capability of analysing them together. Few mechanisms exist to bring information from different parts of the mobile network to support decision making processes, such as PCC has the RAN Congestion Awareness Function (RCAF) which gives the RAN congestion info to support PCRF decision making (TS 23.203). Nevertheless these mechanisms are not able to provide analytics and prediction on E2E QoS as 5G scenarios require.

- The mechanisms used for load and status reporting are entity (i.e., Network Function) based and using aggregated values. For instance, in the GTP-C load and overload control mechanism (TS 29.274) nodes can report their load information which is based on aggregated values such as “used capacity of the node based on activated bearers in relationship to maximum number of bearers the node can handle” (TS 29.274, Section 12.2.5.1.2.2). The problem is that, only based on this type of aggregated information it is not possible to identify situations affecting the E2E performance of specific types of services (e.g., URLLC) and generate analytics that support changes in the system to improve the performance per type of service basis.

Therefore, in this document we propose the use case where NWDAF is used to collect and analyse information from different sources in 5GS, i.e., UE, AN, CN, and application Server so to provide notifications to 5GS about situations that affected E2E QoS profile.

3 Proposal

We propose to introduce the following changes to the TR 23.791.

* * * * Start of Change * * * *

5.1.X	Use Case X: NWDA-assisting E2E QoS Assurance

5.1.x.x	Description

Operators already use mechanisms for collection of information that enable CP and OAM to detect situations affecting the QoS of provided services. These mechanisms are effective although designed in a compartmentalized way (i.e., targeting AN, or CN) and focusing on per NF information data collection (e.g., TS 32.426 defines measurements related to EPC entities – such as MME, etc - which are aggregated values such as mean number of dedicated EPS bearers in active mode).  In 5G, there will be different scenarios, such as URLLC (as defined in TS 22.261) and V2X (as recently defined in the SA1 FS_V2XIMP and 5GAA NESQO), that require, respectively, E2E QoS (which includes at least 5GS, and application parameters) and prediction of QoS profile. By enhancing such mechanisms, operators will be able to improve how to handle the effects of network slice dynamicity in E2E QoS assurance per slice and/or per type of service (i.e, performance requirements for groups of UEs associated with a type of service, e.g. V2X).

NWDAF can be leveraged to collect information from the different sources, 5GS, and Application Server, and provide analytics about E2E QoS per slices and/or per types of services. By analyzing together mobile information (i.e., AN and CN) with application information (AF), NWDAF can provide feedback that enables operators to change the network to handle situations affecting E2E QoS. Examples of improvements that can be triggered by analytics/prediction of E2E QoS are: a) UPF placement in RSU to E2E latency of UEs with V2X support; b) try to improve its (RAN) network in areas with dense V2X traffic; c) inform the application when the UE is getting close to overloaded areas, so that both application and 5GS can be a hinted and prepared for a higher chance of oscillation on QoS levels. .

This use case is required to study:

- Which information from 5GS and Application Server can be used to enable generation of analytics related to E2E QoS per slice basis and/or per type of service basis;

- How can the feedback generated by NWDA hint the specific contributors of situations that affected E2E QoS profile to support the triggering of actions in 5GS and/or application with a higher chance of being effective

* * * End of all change * * *