3GPP TSG-SA2 Meeting #127	S2-184179

# Sanya, China, 16th Apr 2018 - 20th Apr 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0341 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove PCF as the consumer of Namf_EventExposure and Nsmf_EventExposure services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT. Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | We have agreed in the last meeting that Policy Control Request Triggers are handled in the Npcf_SMPolicyControl and Npcf_AMPolicyControl services, thus PCF is no longer the consumer of Namf_EventExposure and Nsmf_EventExposure services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify PCF is no longer the consumer of Namf_EventExposure and Nsmf_EventExposure services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear whether PCF can use the Namf_EventExposure and Nsmf_EventExposure services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1, 5.2.8.1, 5.2.8.3.2, 5.2.8.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************************************Start of 1st change*****************************************

#### 5.2.2.1	General

The following table shows the AMF Services and AMF Service Operations.

Table 5.2.2.1-1: List of AMF Services

| Service Name | Service Operations | Operation Semantic | Known Consumer(s) |
| --- | --- | --- | --- |
| Namf_Communication | UEContextTransfer | Request/ Response | Peer AMF |
|  | RegistrationCompleteNotify | Subscribe / Notify | Peer AMF |
|  | N1MessageNotify | Subscribe / Notify | SMF, SMSF, PCF, NEF, LMF |
|  | N1MessageSubscribe |  | SMF, SMSF, PCF, NEF |
|  | N1MessageUnSubscribe |  | SMF, SMSF, PCF, NEF |
|  | N1N2MessageTransfer | Request/ Response | SMF, SMSF, PCF, NEF, LMF |
|  | N2InfoSubscribe | Subscribe / Notify | SMF |
|  | N2InfoUnSubscribe |  | SMF |
|  | N2InfoNotify |  | SMF, LMF |
|  | EBIAssignment | Request/Response | SMF |
|  | AMFStatusChangeSubscribe | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
|  | AMFStatusChangeUnSubscribe | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
|  | AMFStatusChangeNotify | Subscribe / Notify | SMF, PCF, NEF, SMSF, UDM |
| Namf_EventExposure | Subscribe | Subscribe / Notify | NEF, SMF, UDM |
|  | Unsubscribe | Subscribe / Notify | NEF, SMF, UDM |
|  | Notify | Subscribe / Notify | NEF, SMF, UDM |
| Namf_MT | EnableUEReachability | Request/Response | SMSF |
| Namf_Location | ProvideLocation | Request/Response | GMLC |
|  | EventNotify | Subscribe / Notify | GMLC |

**************************************Start of 2nd change*****************************************

#### 5.2.8.1	General

The following table shows the SMF Services and SMF Service Operations.

Table 5.2.8.1-1: NF services provided by the SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nsmf_PDUSession | Create | Request/Response | V-SMF |
|  | Update | Request/Response | V-SMF, H-SMF |
|  | Release | Request/Response | V-SMF |
|  | Create SM context | Request/Response | AMF |
|  | Update SM context | Request/Response | AMF |
|  | Release SM context | Request/Response | AMF |
|  | SMContextStatusNotify | Subscribe/Notify | AMF |
|  | StatusNotify | Subscribe/Notify | V-SMF |
|  | Context | Request/Response | AMF |
| Nsmf_EventExposure | Subscribe | Subscribe/Notify | NEF, AMF |
|  | Unsubscribe |  | NEF, AMF |
|  | Notify |  | NEF, AMF |

**************************************Start of 3rd change*****************************************

##### 5.2.8.3.3	Nsmf_EventExposure_Subscribe service operation

Service operation name: Nsmf_EventExposure_Subscribe.

Description: This service operation is used by an NF to subscribe or modify a subscription for event notifications on a specified PDU Session or for all PDU Sessions of one UE, group of UE(s) or any UE.

Input, Required: NF ID, target of event as defined in clause 5.2.8.3.1, (set of) Event ID(s) defined in clause 5.2.8.3.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Input, Optional: Event Filter(s) associated with each Event ID; Event Filter(s) are defined in clause 5.2.8.3.1, Subscription Correlation ID (in case of modification of the event subscription).

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for modification). management of this subscription).

Output, Optional: None.

Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by SMF with this subscription.

##### 5.2.8.3.4	Nsmf_EventExposure_UnSubscribe service operation

Service operation name: Nsmf_EventExposure_UnSubscribe.

Description: This service operation is used by an NF to unsubscribe event notifications.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: None.

Output, Optional: None.

**************************************End of changes*******************************************